- Module 1

Introduction to the new Eurocodes

Lecture 2Introduction to Eurocode 1

- Contents:

1.	Scope and structure of Eurocode 1

2.	Definitions and notations

3.	Eurocode 1 Part 1.1

3.1.	Dead loads

3.2.	Imposed loads

4.	Eurocode 1 Part 1.3 Snow loads

5.	Eurocode 1 Part 1.4 Wind loads

6.	Load cases

6.1.	Design values of loads

6.2.	Combination values

7.	References

| - Scope and structure of Eurocode 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The Eurocodes require structures to be designed to resist the effect of ‘actions’. These are principally loads (direct actions) but also include indirect actions such as imposed deformations (for example foundation settlement) and temperature effects. |  |  |  |  |  |  |  |
| Eurocode 1 defines how actions on a structure should be determined. The principal exclusions relate to execution (temporary works), appraisal, special (e.g. nuclear), and cases which involve very large deformations. |  |  |  |  |  |  |  |
| The principal sources of actions on buildings are dead and imposed loads, wind loads and snow loads. These are covered in different parts of Eurocode 1 as follows: |  |  |  |  |  |  |  |
| - Eurocode 1 Part 1.1 - Densities, self-weight, imposed loads for buildings - Eurocode 1 Part 1.3 - Snow loads - Eurocode 1 Part 1.4 - Wind actions |  |  |  |  |  |  |  |
| - Definitions and notations |  |  |  |  |  |  |  |
| The principal terms and the equivalent symbols relating to actions are as follows: |  |  |  |  |  |  |  |
|  | A |  |  | loaded area |  |  |  |
|  | A0 |  |  | basic area |  |  |  |
|  | Qk |  |  | characteristic value of a variable concentrated load |  |  |  |
|  | gk |  |  | weight per unit area, or weight per unit length |  |  |  |
|  | n |  |  | number of storeys |  |  |  |
|  | qk |  |  | characteristic value of a uniformly distributed load, or line load |  |  |  |
|  | αA, αn |  |  | reduction factors |  |  |  |
|  |  |  |  | bulk weight density |  |  |  |
|  |  |  |  | dynamic magnification factor |  |  |  |
|  | ψ0 |  |  | factor for combination value of a variable action, see table A.1.1 of EN 1990 |  |  |  |
|  |  |  |  | angle of repose (degrees) |  |  |  |
| Eurocode 1 also makes extensive use of subscripts. These can be used to clarify the precise meaning of a symbol. Some common subscripts are as follows: |  |  |  |  |  |  |  |
|  |  | k |  | characteristic |  |  |  |
|  |  | d |  | design |  |  |  |
|  |  | sup |  | superior (upper bound) |  |  |  |
|  |  | inf |  | inferior (lower bound) |  |  |  |
| Specific notation is used for the parts of Eurocode 1 dealing with snow and wind loads as follows: |  |  |  |  |  |  |  |
| Eurocode 1-1-3: Snow loads |  |  |  |  |  |  |  |
|  |  | sk |  | characteristic snow load at ground level |  |  |  |
|  |  |  |  | snow load shape coefficient, |  |  |  |
|  |  | s |  | characteristic snow load on the roof |  |  |  |
| Eurocode 1-1-4: Wind loads |  |  |  |  |  |  |  |
|  |  | vref |  | reference wind velocity |  |  |  |
|  |  | qref |  | reference mean wind velocity pressure |  |  |  |
|  |  |  |  | the density of air |  |  |  |
|  |  | ce(z) |  | exposure coefficient, which takes into account the terrain’s roughness, topography and the height above ground |  |  |  |
|  |  | we |  | external wind pressure |  |  |  |
|  |  | wi |  | internal pressure |  |  |  |
| Three different types of action are considered in relation to time: |  |  |  |  |  |  |  |
| - Permanent actions |  |  |  |  | (G) | dead loads |  |
| - Variable actions |  |  |  |  | (Q) | e.g. imposed, wind, snow* loads |  |
| - Accidental actions |  |  |  |  | (A) | e.g. fire, impact |  |
| *in some locations snow may be regarded as an accidental action |  |  |  |  |  |  |  |
| Actions may also be classified in relation to: |  |  |  |  |  |  |  |
| - Spatial variation |  |  |  |  | fixed (dead loads); free (imposed, snow, wind) |  |  |
| - Nature |  |  |  |  | static; dynamic |  |  |
| Characteristic values, denoted by the subscript 'k', for different actions can be determined from appropriate parts of Eurocode 1. The subscript 'd' is used to denote design values. These are obtained by multiplying the characteristic values by a partial safety factor,  |  |  |  |  |  |  |  |
| For some cases in which more than one source of loads or actions has to be considered, the appropriate design loads are combined, using combination factors, , to model the probability of such conditions arising in practice. |  |  |  |  |  |  |  |
| Subscripts inf and sup are sometimes used to qualify partial safety factors. |  |  |  |  |  |  |  |
| The subscript inf is used to denote lower bound values, and should be used where the action has a favourable effect, for example in considering the downwards permanent loads in a wind uplift case. |  |  |  |  |  |  |  |
| The subscript sup is used to denote upper bound values, and should be used where the action has an unfavourable effect. This would be the normal condition. |  |  |  |  |  |  |  |
| - Eurocode 1 Part 1.1 |  |  |  |  |  |  |  |
| Eurocode 1 Part 1-1 provides detailed guidance on dead and imposed loads, and includes comprehensive data relating to material densities for both construction and stored materials. As with other Eurocodes it is organised in Chapters as follows: |  |  |  |  |  |  |  |
| Chapter 1 |  |  | General |  |  |  |  |
|  |  |  | This outlines the scope, defines specific terms, and provides a notation list |  |  |  |  |
| Chapter 2 |  |  | Classification of actions |  |  |  |  |
|  |  |  | Provides brief guidance on permanent/variable and fixed/free actions |  |  |  |  |
| Chapter 3 |  |  | Design situations |  |  |  |  |
|  |  |  | Provides broad comment on some relatively minor aspects of design |  |  |  |  |
| Chapter 4 |  |  | Densities of building materials and stored materials |  |  |  |  |
|  |  |  | Gives general information concerning the determination of material densities |  |  |  |  |
| Chapter 5 |  |  | Self weight of construction works |  |  |  |  |
|  |  |  | Provides detailed guidance for calculating self weight. |  |  |  |  |
| Chapter 6 |  |  | Imposed loads on buildings |  |  |  |  |
|  |  |  | Provides detailed guidance for determining values of imposed loading for different types of building, including reduction factors for elements supporting large floor areas. |  |  |  |  |
| Annex A |  |  | Tables for nominal density of construction materials, and nominal density and angles of repose stored material |  |  |  |  |
| Annex B |  |  | Vehicle barriers and parapets for car parks |  |  |  |  |
|  |  |  | Provides brief guidance for the consideration of impact load cases for vehicle barriers and parapets for car parks |  |  |  |  |
| - Dead loads |  |  |  |  |  |  |  |
| Dead loads are referred to as ‘self weight’ and are classified as permanent, fixed actions. They are calculated on the basis of material densities (see Table 1) and nominal dimensions. |  |  |  |  |  |  |  |

| - Densities of selected construction materials |  | Part 1-1 Tables A.1-A.12 |  |  |
| --- | --- | --- | --- | --- |
| Exemplary Materials | Density [kN/m3] |  |  |  |
| concrete (see ENV 206) |  |  |  |  |
| lightweight (varies with density class) | 9 - 20 |  |  |  |
| normal weight | 24 |  |  |  |
| heavyweight | > |  |  |  |
| reinforced and pre-stressed concrete; unhardened concrete | +1 |  |  |  |
| metals |  |  |  |  |
| aluminium | 27 |  |  |  |
| steel | 77 |  |  |  |
| wood |  |  |  |  |
| timber (depending on strength class C14 - C70; see prEN 338) | 2.9 – 9.0 |  |  |  |
| other materials |  |  |  |  |
| glass, in sheets | 25 |  |  |  |

| The self weight of the construction works includes the structural and non-structural elements including fixed services as well as the weight of earth and ballast. |  |  | Part 1-1 5.1(2) |  |
| --- | --- | --- | --- | --- |
| The sequence shown in the following details the procedures for the assessment of self weight of individual elements of the construction such as walls, floors etc. The procedure will be familiar to most practising design engineers, and is based on calculating unit weights per square metre of construction. |  |  |  |  |
| Determining dead loads: |  |  |  |  |
| - Itemise individual components contributing to the self weight of the element; - Determine materials density for each component (Tables A.1-A.12); - Calculate weight per square metre (= density·thickness) for each component; - Sum the unit weights for all components to determine total self weight (kN/m²). |  |  | Part 1-1 |  |
| - Imposed loads |  |  |  |  |
| Imposed loads on a building are those arising from occupancy and are principally modelled as uniformly distributed loads, line loads or concentrated loads or combinations of these loads. Characteristic values for these are tabulated in relation to building use. In special cases concentrated loads may be relevant, and these are also specified. |  |  | Part 1-1 6.1(2) |  |
| Imposed loads are generally classified as variable and free. |  |  |  |  |
| Loads due to moveable partitions and services which may be repositioned are treated as imposed loads. |  |  |  |  |
| The characteristic values of imposed loads are composed of long-term, medium-term, and short-term components. In practice there is often no need to distinguish between these except where materials are sensitive to time-dependent actions. For example, concrete is subject to creep, and load duration may therefore need to be considered in some aspects of the design of composite structures. |  |  |  |  |
| Building occupancy is defined in five categories (A-E) with some sub-divisions as shown in Table 2 and corresponding characteristic imposed floor loadings are specified. |  |  | Part 1-1 Table 6.1 Table 6.3 |  |
| Different categories for roofs with access, for garages and for vehicle traffic areas are treated separately. |  |  | Part 1-1 Tables 6.7- 6.9 |  |
| - Building occupancy categories |  |  | Part 1-1 Table 6.1, Table 6.3 |  |
| A | Areas for domestic and residential activities (including hospital wards, hotel bedrooms etc) |  |  |  |
| B | Office areas |  |  |  |
| C | Areas where people may congregate (subdivided into 5 sections depending on likely density of occupation and crowding) |  |  |  |
| D | Shopping areas (subdivided into 2 sections depending on the type of shopping area) |  |  |  |
| E | Storage areas (subdivided into 2 sections depending on general or industrial storage) |  |  |  |

| - Characteristic imposed floor loadings |  |  |  |  | Table 6.2, Table 6.4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Loaded areas |  | qk [kN/m2] | Qk [kN] |  |  |  |
| Category A - Floors |  | 1,5 to 2,0 | 2,0 to 3,0 |  |  |  |
| - Stairs |  | 2,0 to 4,0 | 2,0 to 4,0 |  |  |  |
| - Balconies |  | 2,5 to 4,0 | 2,0 to 3,0 |  |  |  |
| Category B |  | 2,0 to 3,0 | 1,5 to 4,5 |  |  |  |
| Category C - C1 |  | 2,0 to 3,0 | 3,0 to 4,0 |  |  |  |
| - C2 |  | 3,0 to 4,0 | 2,5 to 7,0 (4,0) |  |  |  |
| - C3 |  | 3,0 to 5,0 | 4,0 to 7,0 |  |  |  |
| - C4 |  | 4,5 to 5,0 | 3,5 to 7,0 |  |  |  |
| - C5 |  | 5,0 to 7,5 | 3,5 to 4,5 |  |  |  |
| Category D - D1 |  | 4,0 to 5,0 | 3,5 to 7,0 (4,0) |  |  |  |
| - D2 |  | 4,0 to 5,0 | 3,5 to 7,0 |  |  |  |
| Category E - E1 |  | 7,5 | 7,0 |  |  |  |
| Where a range is given in this table, the value may be set by the National annex. The recommended values, intended for separate application, are underlined. qk is intended for determination of general effects and Qk for local effects. The National annex may define different conditions. |  |  |  |  |  |  |
| Provision is made for reducing imposed loads on structural elements which are supporting large floor areas. The reductions are applied through a factor, , applied according to simple formulae related to the area supported by a beam, or the number of storeys supported by a column. |  |  |  |  |  | Part 1-1 6.2.1 (4) |
| - For beams: A = 5 / 7 0 + 10 / A ≤ 1.0 for all other categories - but  0.6 for categories C and D |  |  |  |  |  |  |
| - where A is the loaded area supported by the beam (m2) |  |  |  |  |  |  |
| - For columns: n = [2 + (n – 2) 0] / n |  |  |  |  |  |  |
| - where n is the number of storeys (> 2) supported. |  |  |  |  |  |  |
| In both cases, 0 is determined from EN 1990, and described in section 6.2 below. |  |  |  |  |  |  |
| Determining imposed load intensities: |  |  |  |  |  |  |
| - Define the category of building and specific use according to Table 2; - From Table 3 determine the appropriate value of imposed load intensity qk (kN/m²). Note that the alternative concentrated load (Qk) is intended only for use in assessing structural performance over a very small area; - Determine the area supported by individual beams or the number of storeys supported by individual columns; - Determine imposed load reduction factors as appropriate; - In calculating the characteristic imposed load on individual elements, multiply by the corresponding reduction factor if it is less than 1,0. |  |  |  |  |  |  |
| - Eurocode 1 Part 1.3 Snow loads |  |  |  |  |  |  |
| Eurocode 1 Part 1-3 provides detailed guidance on the calculation of roof loads due to snow, but excludes the following: |  |  |  |  |  |  |
| - Sites over 1500 m altitude; - Impact loads associated with snow falling from a higher roof; - Ice loading; - Sites where snow is present all year; - Lateral loads due to snow drifting; - Snow loads on bridges. |  |  |  |  |  |  |
| This part of the Eurocode 1 it is organised in Chapters as follows: |  |  |  |  |  |  |
| Chapter 1 | General |  |  |  |  |  |
|  | This outlines the scope, defines specific terms, and provides a notation list |  |  |  |  |  |
| Chapter 2 | Classification of actions |  |  |  |  |  |
|  | Provides brief guidance on snow as a variable, free action, and in some cases an accidental action |  |  |  |  |  |
| Chapter 3 | Design situations |  |  |  |  |  |
|  | Provides broad comment of a rather general nature |  |  |  |  |  |
| Chapter 4 | Snow load on the ground |  |  |  |  |  |
|  | Provides general comments for the determination of characteristic values, combination factors and exceptional snow loads |  |  |  |  |  |
| Chapter 5 | Snow load on the roofs |  |  |  |  |  |
|  | Provides detailed guidance for calculating snow load intensities in relation to the snow load on the ground (Chapter 4) and the shape of the roof. |  |  |  |  |  |
| Chapter 6 | Local effects |  |  |  |  |  |
|  | Provides forces to be applied for the local verifications of drifting at projections and obstructions. Furthermore snow loads for the edge of the roof and for snow fences are provided. |  |  |  |  |  |

| Annex A (normative) | Design situations and load arrangements to used for different locations |  |  |
| --- | --- | --- | --- |
| Annex B (normative) | Snow load shape coefficients for exceptional snow drifts |  |  |
| Annex C (informative) | European Ground Snow Load Maps |  |  |
| Annex D (informative) | Adjustment of the ground snow load according to return period |  |  |
| Annex E (informative) | Bulk weight density of snow |  |  |
| Snow loads, s, are calculated on the basis of a characteristic load, sk, corresponding to a uniform depth of snow accumulated under calm conditions on flat ground. This value is modified to account for the shape of the roof and the effect of wind on the distribution of snow. (Note the value of s may be reduced by exposure and thermal coefficients which account for more severe wind conditions, and reduced thermal insulation in the roof but these are generally taken as 1,0). The characteristic values are based on snow maps for individual member states, and relate sk directly to geographical location. An exemplary snow map of the Czech Republic is shown in Figure 1. The Annex C provides further detailed maps for different European sites. |  | Part 1-3 5.2(3), Annex C |  |
| - Exemplary snow map for the Czech Republic |  | Part 1-3 Fig.C11 |  |
| The influence of roof geometry is accounted by a snow load shape coefficient , related to the roof pitch. Conventional pitched roofs, mono-pitch, multi-pitch and cylindrical roofs are treated separately. |  | Part 1-3 5.3 |  |
| A typical chart giving values of snow load shape coefficient in relation to roof slope is shown in Figure 2 and Table 4. Note that for mono-pitch roofs two load cases should be considered, one in which the full snow load is applied over the full extent of the roof, and a second in which half the snow load intensity is applied over the most unfavourable half of the roof. The second load case will rarely be critical. |  | Fig. 5.3 Table 5.2 |  |
| - Snow load shape coefficients for multi-span roofs |  | Part 1-3 Fig. 7.3 |  |
| - Snow load shape coefficients |  | Part 1-3 Table 5.2 |  |
| Roofs with abrupt changes in height must account for the possibility of snow sliding from an upper level, and this is done by a further shape coefficient. |  | Part 1-3 5.3.6(1) |  |
| Rules are given to enable the effect of snow overhanging the edge of a roof to be accounted for. These apply to those parts of a roof cantilevered out beyond the walls, but the magnitude of the allowance is independent of the length of overhang. |  | Part 1-3 6.3 |  |
| The procedure for determining snow loads on roofs is shown in the following: |  |  |  |
| - Determine the characteristic value of snow on the ground according to the geographic location, including appropriate allowance for altitude (Annex C); - Determine snow load shape coefficients μi according to the type of roof and its pitch; - Calculate the characteristic snow load on the roof: s = sk μi. |  |  |  |
| - Eurocode 1 Part 1.4 Wind loads |  | Part 1-4 |  |
| Eurocode 1 Part 1-4 provides detailed guidance on the calculation of wind loads |  |  |  |
| This chapter of the Eurocode is organised in Chapters as follows: |  |  |  |
| Chapter 1 | General |  |  |
|  | This outlines the scope, defines specific terms, and provides a notation list |  |  |
| Chapter 2 | Design Situations |  |  |
|  | Provides broad comment of a rather general nature |  |  |
| Chapter 3 | Modelling of wind actions |  |  |
|  | Provides broad comment on the idealisation of the natural wind loading |  |  |
| Chapter 4 | Wind velocity and velocity pressure |  |  |
|  | Provides detailed guidance for calculating the characteristic parameters of the natural wind |  |  |
| Chapter 5 | Wind actions |  |  |
|  | Provides general information and definitions for wind pressures and wind forces on buildings |  |  |
| Chapter 6 | Structural factors cscd |  |  |
|  | The structural factor cscd should take into account the effect on wind actions from the non-simultaneous occurrence of peak wind pressures on the surface (cs) together with the effect of the vibrations of the structure due to turbulence (cd) | Part 1-4 6.1(1) |  |
| Chapter 7 | Pressure and force coefficients |  |  |
|  | Provides detailed information for the determination of pressure and force coefficients dependent on the building’s shape |  |  |
| Chapter 8 | Wind actions on bridges |  |  |
|  | Provides detailed guidance on the determination of wind effects on bridge structures |  |  |
| Annex A (informative) | Terrain effects |  |  |
| Annex B (informative) | Procedure 1 for determining the structural factor cscd |  |  |
| Annex C (informative) | Procedure 2 for determining the structural factor cscd |  |  |
| Annex D (informative) | cscd values for different types of structures |  |  |
| Annex E (informative) | Vortex shedding and aero-elastic instabilities |  |  |
| Annex F (informative) | Dynamic characteristics of structures |  |  |
| Two approaches are presented in Eurocode 1 Part 1-4. The simple method is applicable where structures are unlikely to be susceptible to dynamic excitation. For very slender or unusual structures, more detailed calculations may be required to justify this. This lecture covers only the simplified approach and applies to ‘permanent’ structures. |  |  |  |
| Wind loads are treated as quasi-static pressures acting normal to the surface of the building (except in special circumstances such as tangential friction forces due to wind blowing over large flat surfaces). The characteristic wind pressure is the reference mean wind velocity pressure, qref, derived from the corresponding velocity (vref,0). The reference wind velocity (vref,0) is determined from wind maps for each member state. |  | Part 1-4 4.2 |  |
| Exceptionally, the value of vref,0 may need to be modified to account for altitude or wind direction for some countries. |  |  |  |
| In general temporary structures may be designed for lower wind pressures. |  |  |  |
| The wind maps for the determination of the reference wind velocity are not included in the Eurocode 1 Part 1-4, they may be given in the national annexes. |  |  |  |
| With the knowledge of the reference wind velocity the wind pressure is then determined directly from qref = 0,5  vref2 where  is the density of air, taken as 1,25 kg/m3 unless otherwise specified. |  |  |  |
| The reference wind pressure is modified by an exposure coefficient ce(z), which takes into account the terrain roughness according to , topography and height above ground. For ‘flat’ terrain the exposure coefficient ce(z) can be determined from Figure 3 which relates height and terrain category. Terrain is generally considered ‘flat’ except for locations close to isolated hills and escarpments. |  |  |  |

| - Terrain categories and related parameters |  |  |  |  |  |  | Part 1-4 Table 4.1 and A.1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Terrain Category | kr | z0 [m] | zmin [m] | Illustration |  |  |  |
| 0 | Sea, coastal area exposed to the open sea | 0,16 | 0,003 | 1 |  |  |  |  |
| I | Rough open sea, lakes with at least 5 km fetch upwind and smooth flat country without obstacles | 0,17 | 0,01 | 1 |  |  |  |  |
| II | Farmland with boundary hedges, occasional small farm structures, houses or trees | 0,19 | 0,05 | 2 |  |  |  |  |
| III | Suburban or industrial areas and permanent forests | 0,22 | 0,3 | 5 |  |  |  |  |
| IV | Urban areas in which at least 15% of the surface is covered with buildings and their average height exceeds 15 m | 0,24 | 1 | 10 |  |  |  |  |
| Note: The parameters of this table are calibrated to obtain the best fit of available data. The values kr , z0 and zmin are used in 4.3 and 4.4. |  |  |  |  |  |  | Part 1-4 4.3 and 4.4 |  |
| - Exposure coefficient ce(z) as a function of height z above ground and terrain categories I to IV for ‘flat’ terrain (c0(z) = 1,0). |  |  |  |  |  |  | Part 1-4 Figure 4.2 |  |
| Note that tall buildings may be considered in sections with appropriate height ranges, and wind pressures calculated for each section according to the corresponding ‘reference height’, as shown in Figure 4. |  |  |  |  |  |  | Part 1-4 Figure 7.4 |  |
| - Reference height ze as a function of the breadth and actual building height. Wind pressure may be considered separately within the separate zones. |  |  |  |  |  |  | Part 1-4 Figure 7.4 |  |
| Wind pressure distribution |  |  |  |  |  |  |  |  |
| The net pressure on a surface is the algebraic sum of the internal and external pressures. This is indicated in Figure 5. |  |  |  |  |  |  | Part 1-4 Figure 5.1 |  |
| - Pressure on surfaces of buildings |  |  |  |  |  |  | Part 1-4 Fig. 5.1 |  |
| Wind pressure coefficients cpe and cpi are applied to determine the distribution of external and internal pressures respectively. |  |  |  |  |  |  |  |  |
| Values are tabulated for: - Vertical walls of rectangular buildings; - Flat roofs; - Mono-pitch roofs; - Duo-pitch roofs; - Hipped roofs; - Multi-span roofs; - Vaulted roofs and domes; A typical chart is shown in Figure 6.and Table 6. |  |  |  |  |  |  | Part 1-4 Table 7.1 Table 7.2 Table 7.3a Table 7.3b Table 7.4a Table 7.4b Table 7.5 Figure 7.12 |  |
| - External pressure coefficients for vertical walls of rectangular plan buildings. |  |  |  |  |  |  | Part 1-4 Figure 7.5 |  |
| - External pressure coefficients for vertical walls of rectangular plan buildings. |  |  |  |  |  |  | Part 1-4 Table 7.1 |  |
| cpe,10 is the external pressure coefficient for a load area of 10 m2 (intended for the design of the overall load bearing structure of buildings) cpe,1 is the external pressure coefficient for a load area of 1 m2 (intended for the design of small elements and fixings with an area per element of 1 m2 or less) |  |  |  |  |  |  |  |  |

| Internal and external pressures shall be considered to act at the same time. The worst combination of external and internal pressures shall be considered. For buildings without internal partitions internal pressure coefficients are related to the opening ratio, , which is defined as: |  |  |  |  |  |  | Part 1-4 7.2.9(1) Figure 7.13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Part 1-4 7.2.9(6) |
| For closed buildings with internal partitions, the worst case may be assumed: cpi = 0,8 or cpi = – 0,5. |  |  |  |  |  |  |  |
| Special consideration is given to canopy roofs, buildings with more than one skin, solid boundary walls, fences and signboards, and special structures. |  |  |  |  |  |  |  |
| The process for determining wind loads on a building is summarised in the following. |  |  |  |  |  |  |  |
| - Reference wind velocity vref (wind maps, National Annex). Exceptionally modified for altitude or direction; - Mean pressure qref = 0,5 ρ vref² (ρ = 1,25 kg/m³); - Reference height z; - External Pressure coefficients cpe (Tables 7.1-7.5) for walls and roofs of various shapes; - Internal Pressure coefficients cpi calculated in relation to μ, ratio and distribution of openings; - Repeat for alternative wind directions; - Terrain category 0-IV; - Exposure coefficient ce(ze); - External pressure we = qref ce(ze) cpe; - Internal pressure wi = qref ce(zi) cpi; - Net pressure on surface wnet = we +/– wi; |  |  |  |  |  |  | Part 1-4 |
| - Load cases |  |  |  |  |  |  |  |
| - Design values of loads |  |  |  |  |  |  |  |
| Design values are obtained from characteristic values multiplied by appropriate partial safety factors. These account for accidental overloading and inevitable inaccuracies in the analysis. They also implicitly incorporate the global factor of safety necessary in structural design. The magnitude of the partial safety factor depends on the limit state under consideration and the particular type of action. |  |  |  |  |  |  |  |
| The following ultimate limit states (ULS) shall be verified as relevant: |  |  |  |  |  |  |  |
| - EQU: Loss of static equilibrium of the structure or any part of it considered as a rigid body, where: - Minor variations in the value or the spatial distribution of actions from a single source are significant; - The strengths of construction materials or ground are generally not governing. - STR: Internal failure or excessive deformation of the structure or structural members, including footings, piles, basement walls, etc., where the strength of construction materials of the structure governs; - GEO: Failure or excessive deformation of the ground where the strengths of soil or rock are significant in providing resistance; - FAT: Fatigue failure of the structure or structural members. |  |  |  |  |  |  | EC0 6.4.1(1)P |
| The deformations to be taken into account in relation to serviceability requirements should be as detailed in the relevant Annex A (of Eurocode 0) according to the type of construction works, or agreed with the client or the National authority. |  |  |  |  |  |  | EC0 6.5.2(1)P |
| The basic values of partial safety factors to be used in design for STR - failure of structure or structural elements are summarised in Table 7 for both ultimate (Set B) and serviceability limit states. |  |  |  |  |  |  | EC0 Table A1.2(B) and Table A1.4 |
| - Basic values of partial safety factors |  |  |  |  |  |  |  |
|  |  | ULS (STR) |  | SLS |  |  |  |
|  |  | Q | G | Q | G |  |  |
|  | Unfavourable effect | 1,5 | 1,35 | 1,0* | 1,0* |  |  |
|  | Favourable effect | 0,0 | 1,0 | - | - |  |  |

| *except if differently specified in EN 1991 to EN 1999. |  |  |  |
| --- | --- | --- | --- |
| - Combination values |  |  |  |
| When a load combination includes more than one variable load (for example imposed loads and wind loads) the partial safety factors relating to the variable load components change, and each variable load, except the one which has the greatest effect, is multiplied by a combination factor, . If it is not clear which variable load has the greatest effect, all combinations should be considered. |  |  |  |
| The value of the combination factor to use depends on the circumstances, the load type, and the use of the building. For 'normal' circumstances (other than those concerning accidental or seismic situations), the appropriate factor is designated 0. The values of 0 are as follows: |  |  |  |
| Imposed loads for all buildings except storage buildings | 0,7 |  |  |
| Imposed loads for storage buildings | 1,0 |  |  |
| Snow loads | 0,5 – 0,7* |  |  |
| Wind loads | 0,6 |  |  |
| *Dependent on specific rules for the member states and the altitude of the location. |  |  |  |

- References

- Eurocode 1: EN 1991-1-1: Eurocode 1: Actions on structures – Part 1-1: General actions - Densities, self-weight, imposed loads for buildings

- Eurocode 1: EN 1991-1-3: Eurocode 1: Actions on structures – Part 1-3: General actions - Snow loads

- Eurocode 1: EN 1991-1-4: Eurocode 1: Actions on structures – Part 1-4: General actions - Wind actions