- Module 2

Structural analysis and design

Lecture 6Choice of structural analysis and consequences on design

- Contents:

1.	Introduction

2.	Allowance for imperfections

3.	Elastic or plastic global analysis?

3.1.	Elastic global analysis

3.2.	Plastic global analysis

3.2.1.	Requirements on steel properties

3.2.2.	Requirements on joint properties

3.2.3.	Requirements on cross-section shape

3.2.4.	Requirements on cross-section classification

4.	First order or second order elastic analysis?

4.1.	Classification as sway or non sway frame

4.2.	Elastic global analysis of non sway frames

4.3.	Elastic global analysis of sway frames

5.	Design checks

6.	References

| - Introduction |  |
| --- | --- |
| The design of a frame and its components consists in ensuring that both the ultimate and the serviceability limit states are satisfied. It is a two-step procedure involving a: |  |
| - Global frame analysis followed by - Individual design checks of the frame components. |  |
| Global frame analysis aims at the determination of internal forces and moments in all parts of the structure when the latter, known from a preliminary sizing, is subjected to a given load combination. As there are in general several load combinations to be considered in accordance with the standards, there are in principle as many global analyses to be conducted. |  |
| Instead of performing one global analysis per load combination, it is often preferred to conduct one analysis per individual type of load and then to make the appropriate combinations of the results drawn from the global analyses. Doing so consists in applying the very convenient so-called principle of superposition but that is however restricted to first order elastic global analysis. |  |
| Once the global analysis is completed, i.e. all relevant internal forces are determined in the whole structure, then the individual design checks of all the frame components, i.e. members and joints, are performed regarding: |  |
| - The ultimate limit states (ULS), i.e. to verify the components for what regards resistance (including stability), stiffness and deformation capacity (ductility); - The serviceability limit states (SLS), i.e. to verify whether the vertical and horizontal displacements and possibly the frequency of vibration are within recommended limits. |  |
| There are several options for performing the global analysis, going from the most simple to the most complex approach for what regards the assumptions made respectively on the behaviour of components (elastic or plastic analysis) and the geometric structural response (first-order or second-order analysis) of the frame. |  |
| Of course the design checks still to be carried out bear only on aspects which have not yet been covered within the global analysis. |  |
| - Allowance for imperfections |  |
| Before any global analysis of a structure is carried out, this structure shall be fully defined. In particular, in addition to the structure layout, the support conditions, load combinations, member and joint sizing, and the braced or unbraced character of the overall frame must be specified. |  |
| Due consideration shall be given to structural imperfections and geometric imperfections. |  |
| Structural imperfections consist in residual stresses. Usually they are not modelled as such because it is a rather complex process. Instead they are substituted by additional geometric member imperfections the magnitude of which is such that they induce similar effects and consequences on the structural behaviour. So-called equivalent geometric member imperfections result from the magnification of the initial geometric imperfections so as to account for the effects of residual stresses. |  |
| For a plane framed structure there are two main types of geometric imperfections: the frame imperfections and the (compression) member imperfections. |  |
| Frame imperfections consist in initial sway of all the storeys (Figure 1a), i.e. in initial out-of-plumb of the vertical members. This out-of-plumb is the same for all the columns of the frame and its magnitude is obtained as a reference initial slope on the vertical direction weighted with regard to both the number of bays and the number of storeys. Frame imperfections mostly affect the frame instability (global instability) through the so-called P- effects. | Part 1-1 5.3.2(3)-a |
| If modelling columns with an out-of-plumb is not difficult, it is however inconvenient. As an alternative, columns may be modelled as vertical members provided so-called equivalent horizontal loads applied at the level of all the storeys are substituted for the frame imperfections. Of course these equivalents horizontal loads must be superimposed to the actual actions composing the load combination in consideration. | Part 1-1 5.3.2(7) |
| The member imperfections consist in equivalent initial out-of-straightness of the compression members, the magnitude of which is supposed to cover the effects of both initial geometric imperfections and residual stresses (Figure 1b). Member imperfections mostly affect the member instability frame (local instability) through the so-called P- effects. | Part 1-1 5.3.2(3)-b |
| - Equivalent geometric imperfections |  |
| Modelling compression members with an out-of-straightness is also not convenient. As an alternative, these members may be modelled as straight members provided so-called equivalent uniformly distributed loads applied transversely to the longitudinal axis of these members are substituted for the member imperfections. Of course these equivalents transverse loads must be superimposed to the actual actions composing the load combination in consideration. | Part 1-1 5.3.2(7) |
| The rules enabling the determination of global and member imperfections or of their loads of substitution have been presented in Lecture 5. |  |
| In accordance with Eurocode 3, account for global frame imperfections is compulsory in any circumstance. | Part 1-1 5.3.1(1) |
| In contrast, account for local member imperfections is required for very slender compression members only. For global analysis purposes these imperfections may generally be disregarded in practice. Of course the effects of imperfections on local member instabilities are not lost; they are simply accounted for in further design checks by using appropriate design formulae of member resistance to buckling. This approach regarding member imperfections is adopted herein so that the P-effects remain the sole concern. | Part 1-1 5.3.4(1) |
| A global analysis which includes the second order effects, i.e. the P-effects, is susceptible of giving a more realistic representation of the structural behaviour. Moreover, the number of design checks to be made once the internal forces and moment are known is reduced because the effects of imperfections on global frame instability are covered by such a global analysis. However, out-of-plane and/or in-plane instability of members need still to be checked. |  |
| - Elastic or plastic global analysis? |  |
| In the perspective of global analysis, the designer may opt for either an elastic global analysis or, when appropriate, a plastic global analysis. |  |
| - Elastic global analysis |  |
| An elastic global analysis is permitted in all circumstances because the frame components (sections and joints) have not to fulfil any requirement dealing with ductile behaviour. | Part 1-1 5.4.1(2) |
| In the past, the designers mostly used elastic methods of global analysis. It is still the situation presently. |  |
| - Plastic global analysis |  |
| Performing a plastic global analysis accounts for yielding within cross-sections and possibly redistribution of internal forces when successive plastic hinges develop. There is thus a large demand for ductile behaviour. |  |
| The designer is never forced to perform, in specific circumstances, a plastic global analysis instead of an elastic global analysis. In contrast, should a plastic analysis be contemplated, it shall be subordinated to prior requirements regarding especially material properties, joint properties, shape and class of cross-sections. |  |
| - Requirements on steel properties |  |
| Plastic global analysis may only be used only where the structure has sufficient rotation capacity at the actual locations of the plastic hinges, whatever this is in the members or in the joints. |  |
| The minimum ductility required for steels is expressed in terms of limits for: |  |
| - The ratio fu / fy of the specified minimum ultimate tensile strength fu to the specified minimum yield strength fy; - The elongation at failure on a gauge length of 5,65 A0 (where A0 is the original cross-sectional area); - The ultimate strain εu, where εu corresponds to the ultimate strength fu. | Part 1-1 3.2.2(1) |
| The limiting values of the ratio fu / fy, the elongation at failure and the ultimate strain εu may be specified in the National Annex. The following values are recommended in EN 1993-1-1: | Part 1-1 3.2.2(1) |
| - fu / fy ≥ 1,1; - Elongation at failure not less than 15%; - εu ≥ 15 εy, where εy is the yield strain (εy = fy / E). |  |
| Steel conforming to one of the grades listed in Table 3.1 of EN 1993-1-1 should be accepted as satisfying these requirements. | Part 1-1 3.2.2(2) |
| - Requirements on joint properties |  |
| Where a plastic hinge occurs in a joint the joint should either have sufficient strength to ensure the hinge remains in the member or should be able to sustain the plastic resistance for a sufficient rotation, see EN 1993-1-8 and Lectures of Module 4. | Part 1-1 5.4.1(3) |
| - Requirements on cross-section shape |  |
| Where a plastic hinge occurs in a member, the member cross-sections shall be either doubly symmetric or single symmetric with a plane of symmetry in the same plane as the rotation of the plastic hinge and it shall satisfy the requirements specified below in 3.2.4. | Part 1-1 5.4.1(3) |
| - Requirements on cross-section classification |  |
| At plastic hinge locations, the cross-section of the member which contains the plastic hinge shall have a rotation capacity of not less than the required at the plastic hinge location. | Part 1-1 5.6(1) |
| In a uniform member sufficient rotation capacity may be assumed at a plastic hinge if both the following requirements are satisfied: a) The member has Class 1 cross-sections at the plastic hinge location; b) Where a transverse force that exceeds 10% of the shear resistance of the cross section is applied to the web at the plastic hinge location, web stiffeners should be provided within a distance along the member of h / 2 from the plastic hinge location, where h is the height of the cross section at this location. | Part 1-1 5.6(2) |
| Additional requirements are given in EN 1993-1-1 where the cross-section of the member varies along their length. They are normally not in force for usual tubular construction. | Part 1-1 5.6(3) |
| By better exploiting the capacities of steels, plastic analysis is prone to provide statically indeterminate structures with a higher carrying capacity than the one given by elastic analysis. As a result, the serviceability limit states are likely to become the most determinative limit states. That consideration, in addition to the above requirements which are as many restrictions, lead to recommend the use of elastic global analysis for most multi-storey frames. This recommendation is supposed to be adopted for what follows. |  |
| - First order or second order elastic analysis? |  |
| When a frame is subjected to a given load combination, it deflects so as to reach a deflected configuration where the internal forces and moments in any cross-section are in equilibrium with the external applied loads. Equilibrium is one of the fulfilments of any method of global analysis. Therefore, global analysis - the purpose of which is the determination of these internal forces and moments - should in principle be conducted with reference to the deflected configuration of the structure and appears then clearly as an iterative procedure. |  |
| Global analysis is said first order when it is carried out with reference to the initial geometry – i.e. the non deflected configuration - of the frame. It may only be contemplated when the effects of the deflections on the internal forces and moments – designated as second order effects - are sufficiently small so that they may be disregarded. There are situations where the conditions for such a simplification are met; due advantage is then understandably taken from the simplicity of first order elastic global analysis. |  |
| When the above second order effects are not negligible, due allowance for them shall be made. There are two main ways to do accordingly in the context of elastic global analysis: |  |
| - An actual second order elastic analysis is conducted: as said above, it is an iterative procedure that is therefore lengthy and time consuming; - Only the second order effects are approached based on approximate assessment methods and then reflected through a magnification of the results of a first order elastic analysis. |  |
| The selection of the appropriate way depends on the classification of the frame as ‘sway’ or ‘non sway’. |  |
| - Classification as sway or non sway frame |  |
| A frame is classified as non sway if its response to in-plane horizontal loads is sufficiently stiff for it to be acceptably accurate to neglect any additional internal forces or moments arising from horizontal displacements of its storeys (so-called P- effects). |  |
| The classification criterion for a frame being non sway is as follows (see Lecture 5): |  |
| for elastic analysis | Part 1-1 5.2.1(3) |
| where Fcr is the elastic critical load producing sway instability and FEd the sum of the design vertical loads applied to the frame. |  |
| For some types of frames, it is possible to assess the value cr while avoiding the calculation of Fcr (see section 2.1 of Lecture 5). | Part 1-1 5.2.1(4) |
| Any frame that does not fulfil above criterion shall be classified as sway and the effects of the horizontal displacement of its storeys on the internal forces and moments shall be taken into account. |  |
| It is worth while stressing that the classification of a frame is not an intrinsic property of the frame as it has to be determined for each load combination. |  |
| - Elastic global analysis of non sway frames |  |
| As a direct consequence of the definition of non sway frames, a first-order analysis suffices for frames classified as such. | Part 1-1 5.2.1(3) |
| The member resistance (instability) is conducted with buckling lengths relative to the non sway mode. Conservatively they may be taken as the system lengths. |  |
| - Elastic global analysis of sway frames |  |
| - Sway frames requires consideration of second order effects. Of course, a P- second-order analysis may be conducted whatever the circumstances. But there are cases where a less sophisticated approach may be contemplated so as to get an approximate but a satisfactorily accurate distribution of internal forces |  |
| - Therefore the range of sway frames is subdivided into two categories: |  |
| - Frames characterized by a small value of cr for which a due second order analysis must be conducted; - Frames characterized by moderate values of cr, for which it is acceptable to conduct a first-order analysis provided that the horizontal external loads, the equivalent frame imperfections horizontal loads and any other sway effects are amplified appropriately to account for the second-order P- effects that have been neglected in the global analysis. | Part 1-1 5.2.2(5) |
| Second order analysis is required when cr < 3. |  |
| The first simplified method of elastic global analysis is termed “Amplified Sway Moment Method” (in short ASMM). It consists in: 1) Performing a first order analysis under the gravity loads; 2) Performing a first order under the horizontal loads (including external loads, equivalent frame imperfections horizontal loads and any other sway effects); 3) Amplifying the results obtained in step 2) by the so-called amplification factor: - 4) Superimposing the results obtained in step 3) with those obtained in step 1); - 5) Checking the member resistance based on the so-called ‘non sway buckling lengths’ i.e. those associated to a non sway buckling mode. |  |
| In EN 1993-1-1, the use of ASMM is limited to frames where cr ≥ 3 with some additional restrictions specified in EN 1993–1–1 section 5.2.2(6) which are not properly limiting the use of the method as long as regular buildings are concerned. |  |
| Second order effects are implicitly included in the results got from the second order analysis. Though assessed separately in a quite different way in the ASMM, they are also included in the internal forces and moments. In both cases, they affect the loading side of the design checks; as a consequence the member resistance (instability) is checked with the non sway buckling lengths (see below). For sake of simplicity, the column length of the member may be conservatively taken as its buckling length. |  |
| A second simplified method of elastic global analysis is termed “Sway Mode Buckling Length Method” (in short SMBL). It consists in: - 1) Performing a first order analysis under all the loads; - 2) Using the results of step 1) for the design checks provided the check of the member resistance (instability) is conducted based on buckling lengths according to the global sway buckling mode of the whole structure. |  |
| According to the SMBL second order effects are implicitly accounted for in the resistance side of the design checks: indeed they contribute in giving a penalty on the member resistance through a magnified buckling length. It shall be noticed that SMBL does not explicitly consider the increase in moments at the ends of the beams and in the beam-column joints arising from the second-order effects. |  |
| In EN 1993-1-1, the use of SMBL is not subordinated to any limitation on the range of cr. |  |
| The way the P- effects are taken into consideration is more erratic within the SMBL than within the ASMM; therefore the latter method should be preferred. |  |
| - Design checks |  |
| The design checks to be carried out after analysis depend on the sophistication of the analysis tool used. For instance when second-order analysis is used there is usually no need to check in-plane stability of the frame or of its members. The choice of global analysis will thus depend not only on what is required by Eurocode 3, but also on personal choices, depending on specific situations, available software, etc. One must strike a balance between the amount of effort devoted to global analysis, and the amount of effort required for remaining ULS checks (see Figure 2). |  |
| The above-described design approaches consisting in global analysis and subsequent design checks are illustrated in Figure 3. |  |
| - Frame global analysis and design checks |  |
| - Balance of efforts between global analysis and ULS checks |  |

- References

- EN 1993-1-1: Eurocode 3: Design of steel structures - Part 1-1: General rules and rules for buildings. CEN, Brussels, May 2005.

- CIDECT: Training an Education for the Implementation of Eurocode 3, Lecture 4, Module 2, 2008.

- CIDECT: Training an Education for the Implementation of Eurocode 3, Lecture 5, Module 2, 2008.

- European Commission: Frame design including joint behaviour. Report EUR 18563 EN, Office of Official Publications of the European Communities, 1998.

The wording “sway/non sway”, introduced in the ENV version of Eurocode 3, is henceforth omitted. In the EN version though, the concept associated to this wording is still present.

Should a plastic global analysis be contemplated, then the upper bound of 10 should be replaced by 15 in accordance with EN 1993-1-1.