- Module 1

Introduction to the new Eurocodes

Lecture 1Introduction to the new Eurocodes

- Contents:

1.	Introduction to the design of structural steelwork in accordance with the new Eurocodes

2.	Terminology and Symbols

3.	Design approach adopted by the Eurocodes

4.	Partial safety factors

4.1.	General concept

4.2.	Variability of actions (loadings)

4.3.	Variability of material properties

4.3.1.	Strength

4.3.2.	Other material properties

4.4.	Variability of geometry

5.	Ultimate Limit States (ULS)

5.1.	Loss of equilibrium

5.2.	Resistance

5.3.	Fatigue

6.	Serviceability Limit States (SLS)

7.	References

| - Introduction to the design of structural steelwork in accordance with the new Eurocodes |  |
| --- | --- |
| The European Committee for Standardisation (CEN) is the body responsible for publication of Eurocodes, which are often referred to informally by the abbreviated form ‘ECs’. There are ten structural Eurocodes, designated as EC 0 to EC 9. For steel construction the most relevant ones are EC 0, EC 1, EC 3 and EC 4. |  |
| Eurocode 0 (EC 0) addresses especially the structural safety and the general design philosophy. Also, the combinations of actions are specified. The principles and rules in EC 0 are the basis for all other Eurocodes. |  |
| Eurocode 1 (EC 1) is concerned with actions. It mainly deals with loadings but also includes other influences such as temperature and imposed deformations, which affect structural behaviour. These are collectively referred to in the Eurocodes as ‘Actions’. |  |
| Eurocode 3 (EC 3) and Eurocode 4 (EC 4) are concerned with respectively steel structures and composite steel-concrete structures. While EC 1 is concerned with actions, EC 3 and EC 4 are concerned with resistance of such structures or its components and the required design checks following the principles of EC 0. |  |
| Other Eurocodes deal with the design of structures in other materials (EC 2 for concrete structures, EC 5 for timber structures, EC 6 for masonry structures and EC 9 for aluminium structures) or with more specialised design conditions such as earthquake resistance (EC 8) or geotechnical design (EC 7). The two latter Eurocodes apply irrespectively of the material. |  |
| The aim of the structural Eurocodes is to provide authoritative, safe and economical guidance for structural design and to make the latter consistent throughout the European Union. The adoption of common standards in all member states aims at facilitating cross-border interactions and at enabling designers and contractors from one state to compete fairly in another one. |  |
| - The Eurocodes have been developed by groups of specialists from across Europe. Initial drafts have been released as ‘Pre-standards’, identified by the prefix ENV, in association with ‘National Application Documents’ (NADs). These NADs include some values of design parameters which are specific to individual countries. The ENVs have been published with an invitation for the users to comment, and the responses received have been used in developing the full ‘Standards’, identified by the prefix EN. In each country, an EN shall be accompanied by a ‘National Annex’, the aim of which being: |  |
| - to give information on the parameters not yet fixed in this EN; - to make a choice between the alternative procedures offered in the EN when offered; - to quantify some parameters let free in the EN; - to specify some specific actions such as snow loads and wind loads; - to decide about the status of the possible annexes included in the EN; - to provide possibly the designer with additional non contradictory information. |  |
| The format of the official designation of the EN versions of the EC 1, EC 2, EC 3 and EC 4 standards is as follows: |  |
| EC 0: EN 1990: Eurocode 0: Basis of structural design. |  |
| EC 1: EN 1991: Eurocode 1: Actions on structures. |  |
| EC 3: EN 1993: Eurocode 3: Design of steel structures. |  |
| EC 4: EN 1994: Eurocode 4: Design of composite steel and concrete structures. |  |
| The Eurocodes are generally published in separate parts dealing with specific areas. These are referred to by means of a part number and a subtitle, such as for example: |  |
| For Eurocode 1: EC 1: EN 1991-1-1: Eurocode 1: Actions on structures – Part 1-1: General actions - Densities, self-weight and imposed loads for buildings. EC 1: EN 1991-1-2: Eurocode 1: Actions on structures – Part 1-2: General actions - Actions on structures exposed to fire. EC 1: EN 1991-1-3: Eurocode 1: Actions on structures – Part 1-3: General actions - Snow loads. EC 1: EN 1991-1-4: Eurocode 1: Actions on structures – Part 1-4: General actions - Wind actions. EC 1: EN 1991-1-5: Eurocode 1: Actions on structures – Part 1-5: General actions: Thermal actions. EC 1: EN 1991-1-6: Eurocode 1: Actions on structures – Part 1-6: General actions - Actions during execution. EC 1: EN 1991-1-7: Eurocode 1: Actions on structures – Part 1-7: General actions - Accidental actions. EC 1: EN 1991-2: Eurocode 1: Actions on structures – Part 2: Traffic loads on bridges. EC 1: EN 1991-3: Eurocode 1: Actions on structures – Part 3: Actions induced by cranes and machinery. EC 1: EN 1991-4: Eurocode 1: Actions on structures – Part 4: Silos and tanks. |  |
| For Eurocode 3: EC 3: EN 1993-1-1: Eurocode 3: Design of Steel Structures - Part 1-1: General rules and rules for buildings. EC 3: EN 1993-1-2: Eurocode 3: Design of Steel Structures - Part 1-2: Structural fire design. EC 3: EN 1993-1-3: Eurocode 3: Design of Steel Structures - Part 1-3: Supplementary rules for cold-formed members and sheeting. EC 3: EN 1993-1-4: Eurocode 3: Design of Steel Structures - Part 1-4: Supplementary rules for stainless steel. EC 3: EN 1993-1-5: Eurocode 3: Design of Steel Structures - Part 1-5: Plated structural elements. EC 3: EN 1993-1-6: Eurocode 3: Design of Steel Structures - Part 1-6: Strength and stability of shell structures. EC 3: EN 1993-1-7: Eurocode 3: Design of Steel Structures - Part 1-7: Strength and stability of planar plated structures subject to out of plane loading. EC 3: EN 1993-1-8: Eurocode 3: Design of Steel Structures - Part 1-8: Design of joints. EC 3: EN 1993-1-9: Eurocode 3: Design of Steel Structures - Part 1-9: Fatigue. EC 3: EN 1993-1-10: Eurocode 3: Design of Steel Structures - Part 1-10: Material toughness and through-thickness properties. EC 3: EN 1993-1-11: Eurocode 3: Design of Steel Structures - Part 1-11: Design of structures with tension components. EC 3: EN 1993-1-12: Eurocode 3: Design of Steel Structures - Part 1-12: Additional rules for the extension of EN 1993 up to steel grades S700. EC 3: EN 1993-2: Eurocode 3: Design of Steel Structures - Part 2: Steel bridges. EC 3: EN 1993-3-1: Eurocode 3: Design of Steel Structures - Part 3-1: Towers, masts and chimneys - Towers and masts. EC 3: EN 1993-3-2: Eurocode 3: Design of Steel Structures - Part 3-2: Towers, masts and chimneys – Chimneys. EC 3: EN 1993-4-1: Eurocode 3: Design of Steel Structures - Part 4-1: Silos. EC 3: EN 1993-4-2: Eurocode 3: Design of Steel Structures - Part 4.2: Tanks. EC 3: EN 1993-4-3: Eurocode 3: Design of Steel Structures - Part 4.3: Pipelines. EC 3: EN 1993-5: Eurocode 3: Design of Steel Structures - Part 5: Piling. EC 3: EN 1993-6: Eurocode 3: Design of Steel Structures - Part 6: Crane supporting structures. |  |
| For Eurocode 4: EC 4: EN 1994-1-1: Eurocode 4: Design of Composite Steel and Concrete Structures - Part 1-1: General rules and rules for buildings. EC 4: EN 1994-1-2: Eurocode 4: Design of Composite Steel and Concrete Structures - Part 1-2: General rules - Structural fire design. EC 4: EN 1994-2: Eurocode 4: Design of Composite Steel and Concrete Structures - Part 2: General rules and rules for bridges. |  |
| The main part of EC 3 deals with general rules and rules for buildings; it is designated as EC 3 Part 1-1. It was published as a ‘Pre-standard’ (ENV 1993-1-1) in 1992 for trial use. Based on the comments received during this trial use, the content has been revised and the full European Standard (EN 1993-1-1) was issued by CEN in 2005. Relatively little has been changed in the technical content of ENV 1993-1-1, but its organisation has been significantly modified and supplemented. |  |
| Presently, all the parts of Eurocode 3 have been approved by CEN and are available. Each country is now in the process of preparation of the corresponding National Annexes. The text of each part of EN without alteration (including its own annexes) together with its corresponding National Annex will henceforth be a registered national standard. |  |
| There will be a transition period during which the Eurocodes (with their appending National Annexes) will be available as an alternative to the national codes. At the end of this period, national codes should be withdrawn and the Eurocodes will then form the basis for all structural design throughout the European Union. |  |
| Part 1.1 of EC 3 contains general principles and detailed rules for usual buildings. It is this part of EC 3, together with Part 1-2 (Fire), Part 1-8 (Joints) and Part 1-9 (Fatigue), on which these notes focus. For those structures concerned only with conventional steel building frames under normal conditions, there is no need to refer to the other parts of EC 3. |  |
| The parts of Eurocode 1 that are most relevant to ordinary buildings are Parts 1-1, 1-3 and 1-4. These parts are mostly outlined in these notes, with some reference only to Part 1-2. Actions due to thermal effects or imposed deformations are not discussed in detail in these notes, and the term ‘loads’ is therefore often used instead of the more general term ‘actions’. |  |
| EC 4 – Eurocode for composite steel and concrete structures - makes extensive cross-references to both EC 3, the Eurocode for the design of steel structures, and EC 2, the Eurocode for the design of concrete structures. |  |
| These notes are mainly concerned with EC 1 and EC 3, and no further detailed reference is made to EC 4. |  |
| Other relevant reference standards are relative to steel products: |  |
| EN 10025: Hot rolled products of non-alloy steels: Technical delivery conditions. |  |
| EN 10113: Hot rolled products in weldable fine grain structural steels. |  |
| EN 10164: Steel products with improved deformation properties perpendicular to the surface of the product - Technical delivery conditions. |  |
| EN 10210: Hot finished structural hollow sections of non-alloy and fine grain steels. |  |
| EN 10219: Cold formed welded structural hollow sections of non-alloy and fine grain steels. |  |
| Of major importance for the steel construction is the standard EN 1090: Execution of steel structures - Technical requirements. |  |
| All individual parts of each Eurocode are structured into numbered chapters, sections, clauses, sub-clauses and paragraphs, and a consistent method of numbering is adopted, for example as follows: |  |
| Chapter 4 |  |
| Section 4.2 |  |
| Clause 4.2.1 |  |
| Sub-clause 4.2.1.1 |  |
| Paragraph 4.2.1.1(2) |  |
| Depending on the character of the individual clauses, distinction is made between ‘Principles’ and ‘Application Rules’. |  |
| Principles consist in general statements and definitions for which there is no alternative, as well as in requirements and analytical models for which no alternative is permitted unless specifically stated. The principles are identified by the letter ‘P’ following the paragraph number, for example 4.2.1.1(2)P. |  |
| Application Rules are generally recognised rules which comply with the ‘Principles’ and satisfy their requirements. However, they may be replaced by other technical rules as long as such alternatives fully comply with the ‘Principles’ of the relevant Eurocode(s). |  |
| - Terminology and Symbols |  |
| The Eurocodes generally use very precise terminology and associated symbols. Much of this is specific to particular Eurocodes or even sections within them, but there are a few general terms and some aspects of notation which are more widely applicable as follows: |  |
| F Action – This is a general term describing a set of forces (loads) applied to the structure (direct action) or a set of imposed deformations or accelerations caused for example, by temperature changes, moisture variation, uneven settlement or earthquake (indirect action). |  |
| E Effect of actions - This is a general term describing the effect of actions (or action effect) on structural members (e.g. internal force, moment, stress, strain) or on the whole structure (e.g. deflection, rotation, acceleration). |  |
| R Resistance - This is a general term to describe the ability of a structure or an element to resist the effects of actions; bending strength and compressive strength are example of resistance. |  |
|  Partial safety factors - These are factors which account for variability in parameters such as actions and material strength; they also provide a global margin of safety. |  |
| Also the Eurocodes make extensive use of subscripts. These can be used to clarify the precise meaning of a symbol. Some common subscripts used generally throughout the Eurocodes are as follows: |  |
| k Characteristic - This describes characteristic or typical values of variables such as actions or material strengths. |  |
| d Design - This describes the values of variables such as strength or action to be used in the design calculation process; design values are determined from characteristic values modified (i.e. increased or decreased) by appropriate partial safety factors. |  |
| Normal symbols may also be used as subscripts, and subscripts are often combined, such as for example: |  |
| Rd Design resistance. |  |
| Ed Design values of internal force or moment. |  |
| Subscripts can be arranged in sequence when necessary, separated by a comma. For example: |  |
| Npl,Rd Design plastic axial resistance. |  |
| - Design approach adopted by the Eurocodes |  |
| The Eurocode 0 states that a structure should be designed and executed in such a way that, during its intended life, it will, with appropriate degrees of reliability and in an economical way, sustain all actions and influences likely to occur during execution and use and remain fit for the use for which it is required. The design criteria are structural resistance, serviceability and durability. Not only normal but also exceptional actions (e.g. explosion, impact, …) are concerned. The designer is often afforded the opportunity to demonstrate this in a variety of ways, but the prescriptive methods detailed in the Eurocodes are based on limit state design principles. | . |
| The procedures of limit state design encourage the engineer to examine conditions which may be considered as failure - referred to as limit states - using appropriate calculation models. These conditions are classified into ultimate and serviceability limit states, designated as ULS and SLS, respectively. At any limit state the structural performance should be assessed, considering relevant aspects of structural behaviour under appropriate design loads. |  |
| Ultimate limit states concern safety, such as load-carrying resistance and equilibrium, when the structure reaches the point where it is substantially unsafe for its intended purpose. The designer shall check that the resistance of a structure (or elements of a structure) is adequate to sustain the maximum actions (loads or deformations) that will be imposed upon it with a reasonable margin of safety. For steelwork design, the principal aspects which must be checked are resistance (including yielding, buckling, and transformation into a mechanism) and stability against overturning. In some cases, it will also be necessary to consider other possible failure modes such as fracture due to material fatigue and brittle fracture. However, these may generally be ignored for simple building structures. |  |
| Serviceability limit states concern those states at which the structure, although standing up, starts to behave in an unsatisfactory fashion due to, say, excessive deformations or vibrations. Thus, the designer shall check that the structure fulfils its function satisfactorily when subject to its service, or working loads. |  |
| These aspects of behaviour may need to be checked under different conditions. Eurocode 3 defines three design situations, corresponding to: (a) normal use of the structure, (b) transient situations - for example during construction or repair - and (c) accidental situations. Different actions, i.e. various load combinations and other effects, such as temperature or differential settlement, may also need to be considered. |  |
| Despite the apparently large number of cases which should be considered, it is mostly sufficient to design on the basis of resistance and stability and then to check that the deflection limit is not be exceeded. Other limit states do clearly not apply or may be shown not to govern the design by means of quite simple calculations. |  |
| The procedure of limit state design can therefore be summarised as follows: |  |
| - Define relevant limit states at which the structural behaviour has to be checked; - For each limit state, determine appropriate actions to be considered; - Use appropriate structural models for design, and take due account of the inevitable variability of parameters such as material properties and geometrical data, verify that none of the relevant limit states is exceeded. The variability of parameters is formally accounted for by using partial safety factors. |  |
| - Partial safety factors |  |
| - General concept |  |
| Instead of the traditional single factor of safety used formerly in permissible stress design, limit state design provides a number of partial safety factors that relate the characteristic values, for example of loads and strengths, to the design values. In principle, the magnitude of a partial safety factor should be related to the degree of uncertainty or variability of a particular quantity (action or material property or resistance model) determined statistically. In practice, whilst this appears to be the case, the actual values of the partial safety factors used incorporate significant elements of the global safety factor and do not represent a rigorous probabilistic treatment of the uncertainties. |  |
| In essence, the characteristic actions Fk are multiplied by the partial safety factors on actions F to obtain the design loads Fd: |  |
| The effects of the application of the design loads to the structure, i.e. bending moment, shear force, deflection, etc. are termed the ‘design effects’ Ed. |  |
| The design resistance Rd is obtained by dividing the characteristic strengths Rk by the partial safety factors M, including appropriate modifications to take account of other considerations such as buckling problems: |  |
| When ultimate limit states are concerned, the design resistance (Rd) should be greater or equal to the ‘design effect’: |  |
| When serviceability is concerned, the design criterion writes: |  |
| where Cd is the limiting design value of the relevant serviceability criterion and Ed the design effect of actions specified in this serviceability criterion. |  |
| The principal sources of variability are associated with actions, material properties and physical geometry. These are formally accounted for by applying appropriate partial safety factors to the characteristic values. |  |
| - Variability of actions (loadings) |  |
| There is obviously considerable variability in the magnitude of applied actions. The values of the partial safety factors (F) reflect the degree of variability. For example, dead loads, which can be quantified with some confidence, attract a lower factor than imposed loads which are simply related to a broad category of building use. |  |
| Where different actions are considered in combination, lower values of partial safety factors may apply. This reflects the reduced probability of the simultaneous occurrence of maximum actions of different types. That is achieved through the combination factors ( |  |
| Different partial safety factors may also apply if a particular action has a beneficial effect. |  |
| The number of different cases which may need to be considered can become very high. However, in many cases, a small number of potentially critical combinations can be identified. |  |
| - Variability of material properties |  |
| - Strength |  |
| The variability of the structural material is reflected by variations in strength of the components of the structure. For structural steel, the most important property in this context is the yield strength; in a few circumstances, it is referred to the ultimate tensile strength – in short: ultimate strength - too. The characteristic yield (resp. ultimate) strength is normally defined as the value below which only a small proportion of all values would be expected to fall. For practical reasons, a nominal value, corresponding typically to the specified minimum yield (resp. ultimate) strength, is generally used as the characteristic value for structural design purposes. This is the case in Eurocode 3 which tabulates nominal values of yield strength and ultimate strength for different steel grades. |  |
| The design value of the steel strength is defined as the characteristic value of the strength divided by the appropriate partial safety factor. |  |
| - Other material properties |  |
| Other material properties, notably the modulus of elasticity, the shear modulus, the Poisson’s ratio, the coefficient of linear thermal expansion and the density, are much less variable than strength. Their design values are typically quoted as deterministic and no partial safety factor applies. |  |
| In addition to the quantified values used directly in structural design, some other material properties are normally specified to ensure the validity of the design procedures included within codified rules. For instance, if a plastic analysis is contemplated, Eurocode 3 stipulates minimum requirements for the ratio of ultimate to yield strength, elongation at failure and ultimate strain. |  |
| - Variability of geometry |  |
| Geometric data are generally represented by their nominal values. They are the values to be used for design purposes. Beside the fact that the variability is limited in many cases by allowable tolerances specified in product standards, it is also, for instance in cross-section dimensions, accounted for in partial safety factors applied elsewhere. Other imperfections such as lack of verticality, lack of straightness, lack of fit and unavoidable minor eccentricities present in practical connections should be allowed for. They may influence the global structural analysis, the analysis of the bracing system, or the design of individual structural elements and are generally accounted for in the design rules themselves. |  |

| - Ultimate Limit States (ULS) |  |
| --- | --- |
| The following conditions may need to be verified under appropriate design actions: |  |
| - Loss of static equilibrium of the structure or any part of it, considered as a rigid body; - Failure by excessive deformation, transformation of the structure of any part of it into a mechanism, rupture, loss of stability of the structure or any part of it, including supports and foundations: these aspects are currently designated by the generic term ‘resistance’; - Fatigue. |  |
| These are briefly introduced below. |  |
| - Loss of equilibrium |  |
| The requirement for stability can formally be stated as: |  |
| where Ed,destab and Ed,stab are the design effects of destabilising and stabilising actions respectively. This is the ultimate limit state of static equilibrium; it refers to the stability of the complete structure (including resistance to overturning) rather than the stability of individual components (related to buckling) which is considered in relation to strength. |  |
| - Resistance |  |
| The requirement for resistance can formally be stated as: |  |
| where Ed and Rd are the design values of internal action and resistance respectively. In this context, it may be necessary to check several aspects of an element’s resistance. These aspects include the cross-section resistance (as a check on local buckling and yielding) and the member resistance to various forms of buckling (such as overall buckling of a column, lateral-torsional buckling of beams and shear buckling of webs), as well as the fact that the structure is not likely to transform into a mechanism. That implies that the second-order effects involved either in the member stability or in the overall frame stability are suitably accounted for. |  |
| - Fatigue |  |
| In practice, fatigue is not relevant for the design of conventional building structures and need only to be formally considered under special circumstances. This would include cases where cyclic loading conditions are significant, for example in building structures supporting heavy machinery or cranes or areas subjected to high traffic loading. |  |
| - Serviceability Limit States (SLS) |  |
| The requirement for serviceability can formally be stated as: |  |
| The serviceability limit states which may require consideration include: |  |
| - Deformations or deflections which adversely affect the appearance or the effective use of the structure or cause damages to finishes or non structural elements; - Vibrations which cause discomfort to people, damage to the building or to its contents, or which limit the functional effectiveness of the structure. |  |
| Both deflections and vibrations are associated with the stiffness rather than strength of the structure. For steel structures, adequate stiffness is generally ensured by calculating deflections and ensuring that these are less than stipulated limits Cd. |  |
| Vibrations will then only need to be considered for special cases. |  |

- References

- EN 1990: Eurocode - Basis of structural design. CEN, Brussels, April 2002.

- EN 1991: Eurocode 1: Actions on structures - Part 1-1: Actions on structures - densities, self-weight and imposed loads. CEN, Brussels, April 2002.

- EN 1993-1-1: Eurocode 3: Design of steel structures - Part 1-1: General rules and rules for buildings. CEN, Brussels, May 2005.

- EN 1994-1-1: Eurocode 4: Design of composite steel and concrete structures - Part 1-1: General rules and rules for buildings. CEN, Brussels, December 2004.