- Module 7

Worked examples

Lecture 18Design of beam-to-column bolted joints

- Contents:

1.	Introduction

2.	1st bolted connection (rigid)

2.1.	Preliminary calculations

2.2.	Calculation of the stiffness and resistance properties

2.2.1.	Column web in compression

2.2.2.	Column web in tension

2.2.3.	End-plate in bending (column side)

2.2.4.	End-plate in bending (beam side)

2.2.5.	Beam flange in compression

2.2.6.	Bolts in tension

2.2.7.	Column web panel in shear

2.3.	Evaluation of the mechanical properties of the joint

2.3.1.	Resistance

2.3.2.	Stiffness

2.4.	Verification of the joint resistance and modelling

2.4.1.	Resistance

2.4.2.	Modelling

3.	2nd bolted connection (pinned)

3.1.	Geometrical description

3.2.	Design resistance of joint

3.2.1.	Bolts in shear

3.2.2.	Fin plate in bearing

3.2.3.	Fin plate in shear: gross cross-section

3.2.4.	Fin plate in shear: net cross-section

3.2.5.	Fin plate in shear: shear block

3.2.6.	Plate in bending

3.2.7.	Buckling of the fin plate

3.2.8.	Beam web in bearing

3.2.9.	Beam web in shear: gross cross-section

3.2.10.	Beam web in shear: net cross-section

3.2.11.	Beam web in shear: shear block

3.2.12.	RHS wall in shear: shear failure adjacent to welds

3.2.13.	Final resistance

3.3.	Additional requirements

3.3.1.	Requirements to ensure sufficient rotation capacity

3.3.2.	Requirements to avoid premature weld failure

3.3.3.	Requirements to permit a plastic redistribution of internal forces

3.3.4.	Additional inequality

References

| - Introduction |  |
| --- | --- |
| This lecture intends at illustrating the design of bolted connections between hollow section members (RHS) and open I-section shapes. This type of connection remains simple and practical on site, though being less common for hollow profiles than for open sections. |  |
| The different design checks presented in the following follow the principles of the so-called “component method”, according to Eurocode 3 Part 1-8. |  |
| In this lecture, the joints of the previous lecture structure are investigated. In particular, the connections between the lower storey columns (RHS 260x180x8 and RHS 260x180x12) and their adjacent beams are considered. |  |
| Two different joints are then studied: - The beam-to-column connection about the strong axis of the hollow section (rigid joint assumed in the analysis (see lecture 17), sway frame); - The beam-to-column connection about the weak axis of the hollow section (pinned joint assumed, non-sway frame). |  |
| Additional information on the loading conditions and geometrical definitions can be found in lecture 17. |  |
| - 1st bolted connection (rigid) |  |
| - Main data for joint and loading |  |
| The first joint investigated here is a double-sided beam-to-column joint on the lower storey of the investigated structure (see Figure 1). As the bending moments MEd,left and MEd,right are approximately counterbalancing each other, it will be assumed that they are equal in the calculations hereafter. |  |
| - Connection layout and dimensions |  |
| The connection layout and dimensions are given in Figure 2. The steel grade is S355 for all elements, and the bolts are M24 10.9 bolts, with the following characteristics: Thickness of the bolt head hh = 15 mm Thickness of the nut hn = 19 mm Thickness of washers hw = 4 mm |  |
| As can be seen, the connection is made through the welding of an extended end-plate on the hollow section face. The bolts are positioned “outside” the width of the hollow section, allowing the use of normal bolts (i.e. no blind bolts or equivalent systems are necessary). This arrangement prevents the face of the hollow profile from being bent, which is usually a weak component of the joint, regarding the relative small thickness of the tubular profile (another possibility would have been to locally thicken the hollow section). Such a configuration is slightly unusual but is useful in practice, on site. |  |
| For sake of illustration, only the joint depicted in Figure 1 and Figure 2 will be studied here, under load combination n°5. Detailed calculations according to the so-called “component method” (according to EN 1993-1-8 [2])are given hereafter. It is to be noticed that since this joint has been previously assumed to be rigid in the frame analysis (see Lecture 17), it is necessary to verify it a posteriori. |  |
| - Preliminary calculations |  |
| Column: |  |
| Beam: |  |
| End-plate (column side): |  |
| End-plate (beam side): |  |
| Bolts: |  |
| - Calculation of the stiffness and resistance properties |  |
| - Column web in compression |  |
| Resistance: | EN 1993-1-8 6.2.6.2 |
| (assumption almost satisfied) |  |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - Column web in tension |  |
| Resistance: |  |
|  | EN 1993-1-8 6.2.6.3 |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - End-plate in bending (column side) |  |
| Resistance: | EN 1993-1-8 6.2.6.5 |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - End-plate in bending (beam side) |  |
| Resistance: | EN 1993-1-8 6.2.6.5 |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - Beam flange in compression |  |
| Resistance: |  |
|  | EN 1993-1-8 6.2.6.7 |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - Bolts in tension |  |
| Resistance: |  |
|  | EN 1993-1-8 Table 3.4 |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - Column web panel in shear |  |
| Resistance: | EN 1993-1-8 6.2.6.1 |
| Stiffness: |  |
|  | EN 1993-1-8 Table 6.11 |
| - Evaluation of the mechanical properties of the joint |  |
| - Resistance | EN 1993-1-8 6.2.7 |
| Plastic design moment resistance: |  |
| - Stiffness | EN 1993-1-8 6.3.1 |
| - Verification of the joint resistance and modelling |  |
| - Resistance |  |
| For the load combination n°5 considered in Lecture 17, the maximum moment in the internal beam-to-column joint is equal to MEd = 299 kNm. |  |
| The joint as designed before is therefore sufficiently resistant as long as MEd < MRd. |  |
| - Modelling |  |
| In lecture 17, the beam-to-column joints have been modelled as rigid, and the analysis has been performed accordingly. According to Eurocode 3 Part 1.8, the joints may be considered as rigid, in an unbraced frame, as long as: |  |
|  | EN 1993-1-8 5.2.2.5 |
| where Ib and Lb designate respectively the second moment of area and the length of the adjacent beam. For the IPE 400 beams used in the example, one has Ib = 23130 mm4 and Lb = 8 m. |  |
| As a result: |  |
| As Sj,ini is greater than this value, the validity of the joint modelling selected in Lecture 17 is demonstrated. |  |
| - 2nd bolted connection (pinned) |  |
| - Geometrical description |  |
| As explained before, this second joint has been assumed as a pinned joint in the frame analysis, and must therefore be designed as so. Accordingly, a fin plate connexion between the IPE 330 beam and the RHS 260x180x8 column is chosen, as shown in Figure 3. |  |
| Since Eurocode 3 provides no specific rules for simple (pinned) joints, ECCS recommendations [3] are used in the following. It is to be noted that no major contradiction takes place between these recommendations and Eurocode 3 principles. |  |
| This simple joint is made by means of a 250x110x10 plate welded to the large face of the RHS column, bolted to the web of the beam with three M22 10.9 bolts. It is submitted to a design shear force VEd = 194 kN. |  |
| - Joint configuration and notations |  |
| The detailed characteristics of the different components of the joint are as follows: |  |
| Column RHS 260x180x8 (S355) Geometrical characteristics: Depth hc = 260 mm Width bc = 180 mm Thickness tc = 8,0 mm Radius r = 12 mm Area A = 6619 mm2 Weak axis inertia Iz = 6130 cm4 Material: Yield stress fyc = 355 N/mm2 Ultimate stress fuc = 510 N/mm2 Safety factor M0 = 1,00 |  |
| Beam IPE330 (S355) Geometrical characteristics: Depth hb = 330 mm Thickness of web twb = 7,5 mm Width bb = 160 mm Thickness of flange tfb = 11,5 mm Radius r = 18 mm Area A = 6261 mm2 Strong axis inertia Iy = 11 772 cm4 Elastic modulus Wel,y = 713 cm3 Plastic modulus Wpl,y = 804 cm4 Material: Yield stress fyb = 355 N/mm2 Ultimate stress fub = 510 N/mm2 Safety factor M0 = 1,00 |  |
| Fin plate 250x110x10 (S355) Geometrical characteristics: Vertical gap gv = 40 mm Horizontal gap gh = 10 mm Depth hp = 250 mm Width bp = 110 mm Thickness tp = 10,0 mm Direction of load transfer: Number of bolt rows n1 = 3 Top edge to 1st bolt row distance e11 = 50 mm Beam edge to 1st bolt row distance e1b = 90 mm Pitch between row 1 and 2 p11 = 75 mm Pitch between row 2 and 3 p12 = 75 mm Bottom edge to 3rd bolt row distance e1n = 50 mm Direction perpendicular to load transfer: Number of bolt rows n2 = 1 Left edge to 1st bolt row distance e21 = 50 mm Bolt to beam edge distance e2b = 50 mm Lever arm z = 60 mm Material: Yield stress fyp = 355 N/mm2 Ultimate stress fup = 510 N/mm2 Safety factor M0 = 1,00 |  |
| Bolts M22 10.9 Geometrical characteristics: Resistant area As = 303 mm2 Shank diameter d = 22 mm Holes diameter d0 = 24 mm Material: Yield stress fy = 900 N/mm2 Ultimate stress fu = 1000 N/mm2 Safety factor M2 = 1,25 |  |
| Welds Throat thickness aw = 5,0mm Length of the weld lw = 250 mm |  |
| In accordance with the component method, the different potential failure modes that need to investigated are the following: |  |
| - bolts in shear; - fin plate in bearing; - fin plate in shear: gross cross-section; - fin plate in shear: net cross-section; - fin plate in shear: shear block; - plate in bending; - buckling of the fin plate; - beam web in bearing; - beam web in shear: gross cross-section; - beam web in shear: net cross-section; - beam web in shear: shear block; - RHS wall in shear: shear failure adjacent to welds. |  |
| Besides this, additional checks are required, mainly to ensure sufficient rotational capacity and plastic redistribution of forces. |  |
| The next paragraphs give detailed information on every identified failure mode. |  |
| - Design resistance of joint |  |
| - Bolts in shear |  |
| The total shear resistance of the bolts in shear VRd,1 is as follows:. |  |
| - With: |  |
| - Fin plate in bearing |  |
| - With: |  |
| - Fin plate in shear: gross cross-section |  |
| - With: |  |
| - Fin plate in shear: net cross-section |  |
| - With: |  |
| - Fin plate in shear: shear block |  |
| - Plate in bending |  |
| - Buckling of the fin plate |  |
| - With: |  |
| - Beam web in bearing |  |
| - With: |  |
| - Beam web in shear: gross cross-section |  |
| - With: |  |
| - Beam web in shear: net cross-section |  |
| - With: |  |
| - Beam web in shear: shear block |  |
| - RHS wall in shear: shear failure adjacent to welds |  |
| Since no specific formula is made available from ECCS recommendations, the following is used, inspired by the one provided by the AISC code for CHS sections: |  |
| - Final resistance |  |
| The final resistance of the joint in shear is VRd = 235,9 kN, governed by beam web in bearing. Since VEd = 194 kN, the joint is seen to be resistant enough. |  |
| - Additional requirements |  |
| The design shear resistance of the joint can only be considered if all the following requirements are fulfilled. |  |
| - Requirements to ensure sufficient rotation capacity |  |
| This condition is fulfilled. |  |
| This assumption is assumed to be fulfilled. |  |
| - Requirements to avoid premature weld failure |  |
| This condition is fulfilled. |  |
| - Requirements to permit a plastic redistribution of internal forces |  |
| This condition is fulfilled. |  |
| For the beam web: |  |
| This condition is fulfilled. |  |
| - Additional inequality |  |
| The condition is fulfilled. |  |

- References

- EC 3: EN 1993-1-1: Eurocode 3: Design of Steel Structures Part 1.1: General rules and rules for buildings. CEN, Brussels, May 2005.

- Eurocode 3: EN 1993–1–8: Eurocode 3: Design of steel structures. Part 1–8: Design of joints.

- ECCS Publication No 126 : European Recommendations for the Design of Simple Joints in Steel Structures, 1st Edition, Brussels, 2009