- Module 7

Worked examples

Lecture 19Design of a welded joint in a lattice girder

- Contents:

1.	Description of designed frame and joint

2.	Loading

3.	Profile dimensions – Internal forces

4.	Applicability of design method

5.	Axial resistance of vertical brace

6.	Resistance of diagonal brace and chord members

7.	Design check of diagonal brace welds

References

| - Description of designed frame and joint |  |
| --- | --- |
| The example studied here consists in a portal frame with a truss beam rafter. The distance between the portal frames constituting the whole structure is assumed to be 12 m; the portal frames are supposed to be braced in-plane and out-of-plane. |  |
| The investigated truss is a so-called “N-shape” truss girder that spans over 42 m. The distance between the vertical braces is chosen as 3 m, and a slope of 4,8% of the upper chord is considered here (i.e. the height of the truss varies from 3 m at supports to 4 m at mid-span). |  |
| - Structure geometry and static system |  |
| Attention is devoted to the overlapping joint depicted on following Figure 2. It is to be noted that no eccentricity between the members’ centrelines is present. |  |
| - Overlapping “N” joint |  |

| - Loading |  |
| --- | --- |
| The applied loads acting on the truss structure are assumed to arise from purlins connected to the top chord every 3 m, at the intersection between chord, diagonal and vertical brace. Accordingly, the loading consists in a distribution of discrete loads. |  |
| For sake of simplicity, only a simplified loading situation is considered here. It consists in a set of vertical permanent loads QG = 300 N/m2 and variable loads QQ = 500 N/m2, self weight of the structure is assumed to be included in QG. |  |
| Accordingly, the Ultimate Limit State (ULS) loading consist on vertical concentrated forces FULS = 42 kN in span and FULS = 21 kN at the supports (see Figure 3) if the load factors G and Q are respectively taken as equal to 1,35 and 1,5. The Serviceability Limit State (SLS) combination leads to FSLS = 29 kN and 14,5 kN in span and at the supports, respectively. |  |
| - Distribution of loading at ULS |  |
| - Profile dimensions – Internal forces |  |
| The following cross-sections are selected: |  |
| - Chord: RHS 180x180x8; A = 54,35cm2, I = 2661 cm4 - Vertical brace: RHS 140x140x8; A = 41,55cm2, I = 1195 cm4 - Diagonal brace: RHS 140x140x8. |  |
| The analysis of the structure may be carried out by assuming that the connections between the various elements are pinned. From this analysis, the following maximum applied forces may be assumed to be applied at the ultimate limit state to the different elements: |  |
| - Chord: NEd = 784 kN (compression); - Vertical brace: NEd = 294 kN (compression); - Diagonal brace: NEd = 369 kN (tension). |  |
| The steel grade is assumed to be S355 for all members. Hence, with M1 = 1,0: - Chord: RHS 180x180x8 Npl,Rd = 1930 kN - Braces: RHS 140x140x8 Npl,Rd = 1475 kN |  |
| - Applicability of design method |  |
| Eurocode 3 Part 1.8 prescribes simplified rules for the evaluation of the design resistance of joints between RHS brace members and RHS chord brace members, provided that the joint meets certain geometrical conditions summarised in EN 1993-1-8 Table 7.8. | EN 1993-1-8 7.5.1(1) |
| These conditions are examined hereafter. |  |
| - Relative width ratio between vertical or diagonal member and chord: |  |
|  | EN 1993-1-8 Table 7.8 |
| - Then, since the slenderness ratios of both the vertical and diagonal brace ci / ti = 14,5 < 33 = 26,8, the sections are all Class 1 sections. | EN 1993-1-1 Table 5.2 |
| - Width-to-thickness ratio for diagonal in tension: |  |
|  | EN 1993-1-8 Table 7.8 |
| - Slenderness of section: |  |
|  | EN 1993-1-8 Table 7.8 |
|  | EN 1993-1-8 Table 7.8 |
| - Chord in Class 2: c0 / t0 = 18,5 < 38 = 30,9. |  |
| - Conditions on overlap: |  |
|  | EN 1993-1-8 Table 7.8 |
|  | EN 1993-1-8 Table 7.8 |
| Since all conditions are fulfilled, the simplified “reduced” design rules given in Table 7.10 may be applied. |  |
| In addition to these conditions, due allowance has to be made for possible secondary moments and/or eccentricities. | EN 1993-1-8 Table 5.3 |
| In a first step, secondary effects may need to be accounted for, for both the compression chord, the tension chord, brace members and joints. Reference is made to EN 1993-1-8 clause 5.1.5(3), which states that these effects may be neglected if (for the present case) the conditions of EN 1993-1-8 are be fulfilled. In the present example, they are satisfied | EN 1993-1-8 5.1.5(3) |
| In addition, the ratio of the system length to the depth of the member in the plane of the lattice girder must not be less than the appropriate value (may be assumed as 6). The condition is also fulfilled, so that no secondary effects have to be taken into account. | EN 1993-1-8 5.1.5(3) |
| Besides this, possible moments arising from eccentricities might need to be taken into account. Since the eccentricities are equal to zero in the present example, the condition is straightforwardly fulfilled. | EN 1993-1-8 5.1.5(5) |
| - Axial resistance of vertical brace |  |
| The design values of the internal axial forces in the different elements at the ultimate limit state should not exceed the design resistances of the members determined from EN 1993–1–1, in addition to the design check of the joint resistance. | EN 1993-1-8 7.2.1(1) |
| As one has 25% < ov = 40% < 50%, the following formulae applies for the verification of the joint resistance: |  |
|  | EN 1993-1-8 Table 7.10 |
| with: | EN 1993-1-8 Table 7.10 |
| and: | EN 1993-1-8 Table 7.10 |
| Therefore, the resistance of the joint is seen to be sufficient since the maximum applied axial force in a vertical brace is NEd = 294 kN. |  |
| Eurocode 3 Part 1.8 Table 7.10 also specifies that only the overlapping member has to be checked, i.e. the vertical brace in our case. The brace member efficiency (i.e. the design resistance of the joint divided by the design plastic resistance of the brace member) of the overlapped member should be taken as equal to that of the overlapping brace member. As the same cross section is selected for both the overlapping and the overlapped braces, no further consideration is needed.. |  |
| The verification of the vertical brace in compression according to Eurocode 3 Part 1.1 (buckling) must be made as follows. | EN 1993-1-1 6.3.1 |
| Since the cross-section is a square box section assumed to be hot finished (S355), the imperfection factor for buckling curves is y = z = 0,21. | EN 1993-1-1 Table 6.1 |
| Then, the following intermediate values are needed: | EN 1993-1-1 6.3.1.2 |
| The final verification is satisfactory, since with M1 = 1,0, one has: NEd = 294 kN < Nb,Rd = 1227 kN |  |
| - Resistance of diagonal brace and chord members |  |
| In the same way as previously, the worst chord member is checked against buckling, with y = 0,21. | EN 1993-1-1 6.3.1 |
| Therefore NEd = 784 kN < Nb,Rd = 1745 kN, satisfactory. |  |
| Then, the diagonal brace member has to be checked. No instability problem may occur since the member is acted by tension forces. Therefore, the member verification reduces to the following cross-section check: | EN 1993-1-1 6.2.3 |
| NEd = 369 kN < NRd = 1475 kN, satisfactory. |  |
| - Design check of diagonal brace welds |  |
| Fillet welds of 6 mm throat thickness are assumed on the contour of the assembled elements. As an example, the welds of the (tension) diagonal brace are checked. |  |
| The axial force in the diagonal brace is first decomposed into a horizontal and vertical components: NEd,hor. = NEd,vert. = 260 kN. |  |
| Horizontal check: if it is assumed that the only welds resisting the horizontal force are the welds parallel to the chord, their total carrying capacity is: |  |
|  | EN 1993-1-8 4.5.3.3(4.4) |
| which is satisfactory. |  |
| Vertical check: in the same way, it is assumed that the only welds resisting the vertical force are the welds perpendicular to the chord, their total carrying capacity is: |  |
|  | EN 1993-1-8 4.5.3.3(4.4) |
| which is satisfactory. |  |

- References

- Eurocode 3: EN 1993–1–1: Eurocode 3: Design of steel structures. Part 1–1: General rules and rules for buildings.

- Eurocode 3: EN 1993–1–8: Eurocode 3: Design of steel structures. Part 1–8: Design of joints.