- Module 3

Member design

Lecture 8Members in tension

- Contents:

1.	Introduction

2.	Tension member resistance

2.1.	Resistance of the gross cross-section

2.2.	Resistance of the net cross-section

2.3.	Member resistance

3.	Additional provisions

3.1.	Slip resistant connections at ULS

3.2.	Ductility requirement

4.	Determination of the net cross-sectional area

5.	Eccentric connections

6.	Design for block tearing

7.	Serviceability, corrosion and fatigue

8.	References

| - Introduction |  |
| --- | --- |
| The term ‘tension member’ is generally used to describe a structural component which is only subjected to an axial tensile force. Though it can be a tie or a cable or rope (under special loading conditions), it generally refers to tension pin-ended members found in trusses, lattice girders or bracing members. |  |
| Generally, tension members are designed using any type of sections: rolled sections (angles, I, H, channels), bars, flats and hollow sections. This lecture is concerned with the design of ties from such sections under static loads; cables and ropes are not covered because they have, in many circumstances, a geometric non-linear behaviour and merit, for that reason, a quite particular consideration. |  |
| The basic design of a tension member is essentially very simple: it consists in providing the member with sufficient cross-sectional area so as to resist the applied tensile force. Once, as a result of the global analysis, the magnitude of the tensile force to be resisted has been obtained and the strength of the material has been ascertained, the required cross-sectional area may be readily calculated by dividing the force by the design material strength. However, the end connection of tension members is especially a very important consideration which in many cases may govern the design of the member; therefore, it is a major criterion in the design and selection of a section. |  |
| Usually, a uniform distribution of tensile stresses over the cross-section of a tension member is assumed. Connection detailing may affect the validity of this assumption in two ways. First, if bolted connection (with non preloaded bolts) is used, the cross-sectional area is reduced by the bolt holes and the stresses around the holes are increased locally as shown in Figure 1 a-b. Second, some eccentricity in connections is often unavoidable and secondary moments are therefore induced. These problems may usually be accounted for by substituting an effective net cross-sectional area for the gross area and by performing then a classical check of the tension resistance. |  |
| In the tubular construction the connections of the structural elements are mostly full penetration welded joints; in that case, the cross-section is not weakened by bolt holes and the area of the cross-section is the gross area. However, for sake of generality, some consideration is also given to possible bolted joints in the following. |  |
| - Distribution of stress across a section with holes |  |
| - Tension member resistance |  |
| It is implicitly agreed that the ultimate limit state of a tension member is described by its deformation. |  |
| The design resistance of a tension member with a uniform cross-section is given as the design resistance of its cross-section. Which material strength and cross-sectional area to be taken into account for that purpose is commented on below. |  |
| - Resistance of the gross cross-section |  |
| For non bolted end connections, the cross-sectional area is the gross area over the whole length of the member. It is agreed that the onset of yielding is associated to an ultimate limit state. As the cross-section is uniform, yielding means indeed member capacity to exhibit a substantial strain which is equally distributed over the whole length of the member. This elongation which develops before the material strain-hardens represents a quite significant part of the initial length (about 10 to 15% depending on the steel material) and is considered as unacceptable. Then the ultimate limit state does not mean at all fracture or separation of the member but well excessive and unacceptable deformation. |  |
| The design tension resistance of the gross cross-section is therefore the design plastic resistance: |  |
|  | Part 1-1 6.2.3(2) |
| where |  |
| A is the gross cross-sectional area; |  |
| fy is the yield strength; |  |
| M0 is the partial safety factor for resistance of the gross cross-section. |  |
| In accordance with EN 1993-1-1, the recommended value of M0 is 1.00 irrespectively of the type of structures. However, it is up to the National Annex to specify it. |  |
| - Resistance of the net cross-section |  |
| In the major part of the member length of a tension member with end bolted connections, the resistant cross-sectional area consists in the gross area and the ultimate limit state in this area is reached when, as for non bolted connections, the yield strength is attained herein. The design resistance of the gross cross-section is still given by Eq. . |  |
| In contrast, at the ends of a tension member with end bolted connections, the cross-section is weakened because of the presence of bolt holes. The concept of net cross- sectional area at holes for fasteners is substituted for the gross cross-sectional area. Although the holes induce elastic stress concentrations (as seen in Figure 1), steel ductility makes in principle allowance for a plastic redistribution across the net cross-section. Nonetheless an additional resistance check is required in the net cross-sectional area. |  |
| The onset of the yield strength in the net cross-section may not be associated with a significant elongation of the member; indeed the length affected by yielding is small. Consider for example a tension member in which the connection accounts for 5% of the member length. If the strain in the connection region at the load which causes yielding in the gross cross-section is, say, 10 times the yield strain y, then the increase in length along the member length is: |  |
| In the connection: |  |
| In the remainder of the member: |  |
| Hence, the elongation within the connection zone is very much smaller than the elongation of the whole member and thus the ultimate limit state will occur by excessive elongation of the complete member, provided of course that the design plastic resistance of the gross cross-section Npl,Rd is smaller than the design ultimate resistance of the net cross-section Nu,Rd to rupture. |  |
| At the member ends there are unavoidable small eccentricities and possibly some remaining stress concentrations; therefore the sake for some additional safety through a penalty factor taken as 0.9. Therefore the design tension resistance of the net cross-section at holes for fasteners is the ‘reduced’ design ultimate resistance: |  |
|  | Part 1-1 6.2.3(2)b |
| - where |  |
| Anet is the net cross-sectional area; |  |
| fu is the ultimate tensile strength of steel; |  |
| M2 is the partial safety factor for resistance of the net cross-section, which is different from the one for resistance of the gross cross-section, because connection is concerned. |  |
| In accordance with EN 1993-1-1, the ‘recommended’ value of M2 is 1.25 irrespectively of the type of structures. However, it is up to the National Annexes to specify it. |  |
| The resistance given by Eq. (1) of the gross cross-section is said of ‘ductile’ nature because, once it is reached, the tension element is still able to experience a substantial elongation. In contrast, the resistance given by Eq. is of ‘brittle’ nature; indeed, it corresponds to rupture in the net area because of exhaustion of the deformation capacity of the material. |  |
| - Member resistance |  |
| In any generality the design tension resistance Nt,Rd of a member is thus taken as the smaller of the values calculated by Eqs. and , which correspond to the design resistance respectively of the gross and the net cross-section: |  |
| When the design resistance is governed by the plastic resistance of the gross cross-section, it is accompanied by a significant elongation and is therefore of ductile nature. It is no more the case when the ultimate resistance of the net section is determinative. This distinction may be of importance in some situations, as further discussed below. |  |
| - Additional provisions |  |
| - Slip resistant connections at ULS |  |
| When fatigue in bolted connections is of concern, it is generally required to use preloaded bolts. If, in addition, connections are designed to be slip resistant at the ultimate limit state (Category C connections in accordance with Part 1-8 of EC 3), the design resistance of the net cross-section Nnet,Rd is limited to yield; thus the design tension resistance Nt,Rd of the member in the net cross-section at holes for fasteners is taken as: | Part 1-1 6.2.3(4) |
| It should be noticed that, in the design process, this resistance is to be compared not directly with the design load existing in the gross section but with the one present in the net section. The latter is less because some proportion of the load has yet been transferred by friction. This proportion may only be assessed and it is of course conservative to disregard it. |  |
| - Ductility requirement |  |
| There are cases where ductility is required: for instance in seismic design, where energy dissipation must be favoured, and for robustness considerations, where substantial elongation without rupture is required. Then the governing limit state must correspond to a ductile phenomenon. In such cases, it is necessary to ensure that the resistance criterion is yielding in the gross cross-section and not fracture at the net cross-section: |  |
| With due account taken of Eqs. and , this will be satisfied if: |  |
| If the actual ratio Anet / A falls below the lower limit value and provided ductility is required, the detailing of the connection and/or the sizing of the cross-section need to be amended accordingly. |  |
| For members where ductile behaviour is not specifically required, no allowance for bolt holes such as Eq. is necessary. |  |
| - Determination of the net cross-sectional area |  |
| The net area of a cross-section should be taken as its gross area less appropriate deductions for all fastener holes and other openings. For a single fastener hole the deduction should be the gross cross-sectional area of the hole in the plane of its axis. For countersunk holes, appropriate allowance should be made for the countersunk portion. | Part 1-1 6.2.2.2(1) and (2) |
| When the fastener holes are not staggered, the total area to be deducted for fastener holes should be the maximum sum of the cross-sectional areas of the holes in any cross-section perpendicular to the member axis (see plane 1 in Figure 2). The maximum sum determines the position of the critical fracture line. | Part 1-1 6.2.2.2(3) |
| When the fastener holes are staggered, the total area to be deducted for fastener holes should be the greater of: a) the deduction for non-staggered holes; b) with: s staggered pitch, the spacing of the centres of two consecutive holes in the chain measured parallel to the member axis; p spacing of the centres of the same two holes measured perpendicular to the member axis; t plate thickness; d diameter of fastener hole; n number of holes extending in any diagonal or zigzag line progressively across the member or part of the member (see plane 2 in Figure 2); Σ sum of all the gauge spaces in the chain of holes. | Part 1-1 6.2.2.2(4) |
| - Staggered holes |  |
| In members with holes in more than one plane, the spacing p should be measured along the mid-thickness line of the material (see Figure 3). | Part 1-1 6.2.2.2(5) |
| - Spacing of holes in more than one plane |  |
| - Eccentric connections |  |
| It is not always possible to introduce a tensile force in a member without this force being applied eccentrically with regard to the centroid of the cross-section. Such small eccentricity forces the tension member to bend and is the cause of parasitic bending stresses in the section. |  |
| In principle the primary bending moment, i.e. the moment distribution produced by the sole parasitic end moments, is reduced by the tensile force. In practice, such a second order analysis is usually not performed and the gross cross-section is supposed to be axially loaded. |  |
| In the end sections, it is usual to disregard both the small eccentricities in connections and the bending stresses they produce when the tensile force is produced by a static or quasi-static loading. It is indeed observed that such minor eccentricities do not affect significantly the ultimate resistance of the member. |  |
| In contrast, the effects of eccentricities should be taken into account in the case of cyclic loading because of the possibility for fatigue fracture. |  |
| Should the eccentricity in connections not be regarded as negligible, the designer is invited to check the sections in combined bending and tension. That question is examined furthermore in Lecture 11 which deals with combined axial force and bending. |  |
| When angles are connected by one leg only, the eccentricity is not negligible but a check as centrically loaded members is nonetheless allowed provided that a so-called modified net cross-sectional area be used. The latter consists in giving a penalty on the contribution of the non connected legs to the cross-sectional area. |  |
| In a more general way, consideration should be given to shear lag effects in the connection. These develop when only some parts of the section are directly connected by bolting or welding while the critical net cross-section includes elements which are not connected. For that purpose, the concept of effective net area, lower than the net area, is introduced. |  |
| The effective net area is given as: |  |
| Recommended values of the shear lag factor for bolted and welded connections, drawn from one of the CIDECT Design Guide (see [4]), are as follows: |  |
| - For bolted connections, the shear lag factor  applies the net section. It comes into operation when a member is connected by some but not all of its cross-sectional elements, if the critical net section includes elements which are not connected. The critical net section in this instance may include net area segments which are perpendicular to the load or inclined to it, but not those which are parallel to it. (This is not a check against block shear tear out.) The factor  can be taken as: - 0.90 when shapes like I-sections (or tees cut from them) are connected only by their flanges with at least three transverse rows of fasteners (flange width at least two thirds the depth); - 0.85 for all other structural shapes, e.g. RHS, connected with three or more transverse rows of fasteners; - 0.75 for all members, e.g. RHS, connected with two transverse rows of fasteners. - For welded connections, shear lag factor also applies when a member is not welded all around its cross-section. The shear lag factor applies the gross section. It is a function of the weld lengths Lw and the distance bi between them. The distance between such welds would be bi. The factor  can be taken as: - 1.0 when the weld lengths Lw along the RHS corners are larger than 2 bi; - 0.87 when the weld lengths Lw along the RHS corners are 1.5 bi  5 Lw  2 bi; - 0.75 when the weld lengths Lw along the RHS corners are bi  5 Lw < 1.5 bi. The minimum length of welds Lw is the distance between them. Of course – that is not specific to tubular construction - the failure modes of the gusset plate must be checked: bearing pressure at the bolt holes and yielding across an effective dispersion width of the plate, which can be calculated using the Whitmore effective width concept. This check applies to both tension and compression load cases. If the member is in compression, buckling of the gusset plate must also be prevented. |  |
| - Design for block tearing | Part 1-8 3.10.2 |
| When bolted connections, due attention should be paid to block tearing failure. This mode of failure consists of failure in shear at the row of bolts along the shear face of the hole group accompanied by tensile rupture along the line of bolt holes on the tension face of the bolt group. Some examples of block tearing failure are given in Figure 4. |  |
| It is up to the designer to consider the most probable contour lines for such a failure mode and to check the design block tearing resistance in each case. |  |
| Block tearing resistance is a topic covered in EN 1993-1-8 which is dedicated to the design of joints. |  |
| - Block tearing failure |  |
| - Serviceability, corrosion and fatigue |  |
| Tension members transfer load very efficiently. Therefore they have relatively small cross-sectional areas. This makes ties susceptible to: |  |
| - Excessive elongation under axial load, which may permit large drift of a structure if the tie forms part of a bracing system; - Transverse deflection under self weight; - Undesirable vibration or flutter when exposed to wind or simply resulting from exploitation of the structure. |  |
| Light sections are also easily damaged in transport. |  |
| For these reasons, it is good practice to limit the slenderness of tension members. The slenderness is an important concept for compression members; here it should be understood as L/i where L is the member length and i the smaller of both radii of gyration of the section. Most of the standards do not impose or even recommend such limit values. However the Canadian standard recommends for instance 300 for tension members in buildings and a more severe value of 200 for bridges because the effects of vibrations may be especially detrimental (especially with regard to fatigue). |  |
| A further consideration is brought to the consequences of loss of material due to corrosion, which are greater for tension members since they are relatively highly stressed. |  |

- References

- EN 1993-1-1: Eurocode 3: Design of Steel Structures Part 1.1: General rules and rules for buildings. CEN, Brussels, May 2005.

- EN 1993-1-8: Eurocode 3: Design of Steel Structures – Part 1-8: Design of Joints. CEN, Brussels, May 2005.

- Timoshenko S.P. and Gere J.M.: Theory of Elastic Stability, McGraw -Hill, New York, 1961.

- CIDECT: Design Guide for Rectangular hollow section (RHS) joints under predominantly static loading. Verlag TÜV Rheinland, 1992.

- Trahair N.S. and Bradford M.A.: The Behaviour and Design of Steel Structures, E&F Spon, 1994.

- Zhao X-L., Wilkinson T. and Hancock G.: “Cold-Formed Tubular Members and Connections”, Elsevier, 2005.

- Canadian Standard association: Limit state design of steel structures. CSA Standard CAN-CSA-S16-01, 2001.