- Module 7

Worked examples

Lecture 21Multiplanar joint

- Contents:

1.	Description of the studied truss and joint

1.1.	Geometry

1.2.	Loading

1.2.1.	Basic loading

1.2.2.	Load combination cases

1.2.3.	Partial safety factors on resistance

2.	Internal forces

3.	Multiplanar CHS KK-Joints

3.1.	General

3.2.	Multiplanar joints

3.3.	Chord face failure - K joints

3.4.	Punching shear failure

4.	References

| - Description of the studied truss and joint |  |
| --- | --- |
| - Geometry |  |
| The structure of a multiplanar truss is shown on Figure 1; it is composed of circular hollow sections. |  |
| - Multiplanar truss geometry |  |
| The truss spans about 36 m; and its total height is 2,08 m. The cross-section of the truss builds an equilateral triangle with length of 2,4 m, i.e.  = 38,7° |  |
| The structure is build with two different circular hollow sections: the upper and lower chords are made of CHS 219,1x11 sections, and the braces are CHS 114,3x4,5 sections. |  |
| The horizontal distance between the nodes of the braces are 3 m in the edges and 6 m in the regular parts of the truss. |  |
| The members are standard sections, made of S235 steel according to EN 10025. |  |
| - Loading |  |
| - Basic loading |  |
| The values for the characteristic permanent actions are given in Table 1. |  |
| - Vertical loading |  |
| Self weight and permanent actions; Variable actions (imposed loads). Load; 15,0 kN; 10 kN. |  |
| The loading cases shown schematically in Figure 2 have been considered in appropriate combinations. | EN 1991-1-1 6.2.2(1) |
| - Loading cases |  |
| - Load combination cases |  |
| Load combinations for the ultimate and the serviceability limit states are derived according to EN 1990 (cf. Table 2 and Figure 2). |  |
| - Load combination cases at ULS |  |
| Load combination; Ultimate limit state. Case 1; 1,35 G + 1,5 Q. |  |
| - Partial safety factors on resistance |  |
| For this worked example, the values of the partial safety factors on resistance are assumed to be M0 = M1= M5 = 1,0 for the resistance of cross-sections and members. |  |
| - Internal forces |  |
| The internal forces are calculated with the FEM-program InfoCad Version 6.55, InfoCad GmbH Aachen. |  |
| See detail Figure 4 - Internal forces from Load Case 1 - Detailed internal forces on the node to be verified, Load Case 1 |  |
| - Multiplanar CHS KK-Joints |  |
| - General |  |
| Provided that the geometry of the joints remains within the range of validity given in EN 1993-1-8 Table 7.1, the design resistances of welded joints between circular hollow section members may be determined using sections 7.4.2 and 7.4.3 of EN 1993-1-8. | EN 1993-1-8 7.4.1 |
| In addition, the sections must be shown to be at least Class 2. |  |
| Chord: |  |
| Brace: |  |
| (no eccentricity) |  |
|  Satisfactory. |  |
| - Multiplanar joints |  |
| In each relevant plane of a multiplanar joint, the design criteria given in EN 1993-1-8 7.4.2 should be satisfied using the reduced design resistances. The design resistances for each relevant plane is determined by applying the appropriate reduction factor  given in Table 7.7 of EN 1993-1-8 to the resistance of the corresponding uniplanar joint by using the appropriate chord force for kp. In this example, the multiplanar KK joints is made of two identical K-joints. i.e. only one K-joint to be checked, see below. | EN 1993-1-8 7.4.3 |
| KK-Joint:  = 0,9 |  |
| and |  |
|  Satisfactory. |  |
| - Chord face failure - K joints |  |
| In a brace member connections subject only to axial forces, the design internal axial force Ni,Ed should not exceed the design axial resistance of the welded joint Ni,Rd obtained from EN 1993-1-8 Table 7.2. | EN 1993-1-8 7.4.2 |
| and |  |
| Reduced design resistances: |  |
|  Satisfactory. |  |
| - Punching shear failure |  |
|  Satisfactory. |  |

- References

- Eurocode 1: EN 1991–1–1: Eurocode 1: Actions on structures – Part 1–1: General actions - Densities, self-weight, imposed loads for buildings.

- Eurocode 3: ENV 1993–1–8: Eurocode 3: Design of steel structures. Part 1–8: Design of joints.