- Module 3

Member design

Lecture 7Local buckling and section classification

- Contents:

1.	Introduction

2.	Classification of cross-sections

3.	Behaviour of plate elements in compression

4.	Effective width approach to design of Class 4 sections

5.	References

| - Introduction |  |
| --- | --- |
| Rectangular hollow sections (in short RHS) may be considered as an assembly of individual plate elements, designated as internal elements; two such elements which experience mainly shear are called webs while the two others are called flanges – see Figure 1. As the plate elements in structural sections are relatively thin compared with their width, they may buckle locally when loaded in compression (as a result from axial load and/or bending). The proneness to buckle of any plate element within the cross-section may limit the axial load carrying capacity or the bending resistance of the section, by preventing the attainment of the yield strength. Avoidance of premature failure arising from the effects of local buckling may be achieved by limiting the width-to-thickness ratio of the individual elements composing the cross-section. |  |
| In the construction with hollow steel sections, there is one type of section which is composed of one single element: the circular (or elliptic) hollow section (in short CHS) (Figure 2); there, the (largest) diameter-to-thickness ratio is substituted for the usual width-to-thickness ratio. In the following, when it is referred to the term ‘width’, it should possibly be understood as ‘diameter’ when circular or elliptic sections are concerned. |  |
| The width-to-thickness ratio is the basis of the so-called cross-section classification. |  |
| h b t - Rectangular hollow section |  |
| d t - Circular hollow section |  |
| - Classification of cross-sections |  |
| Cross-section classification aims at identifying the extent to which the resistance and possibly the rotation capacity of cross-sections are limited by possible local buckling of the compression plate elements composing that section. Compression plate elements include every plate element that is either totally or partially subjected to compression under the load combination being considered. |  |
| The various compression plate elements within a cross-section often belong to different classes. The class of the whole cross-section is then defined as the highest (least favourable) class of its compression plate elements. The class in which a compression plate element falls depends upon the width-to-thickness ratio of this element and on the compressive stress distribution (i.e. uniform or linear) to which it is subjected. The class of a compression plate element – and thus of a cross-section – depends on the loading; it is thus not an intrinsic property of that element. |  |
| EN 1993-1-1 defines four classes of compression plate elements and therefore of cross-sections. The classes are usually defined in terms of performance requirements for resistance to bending moments: |  |
| Class 1 (also designated ‘plastic’) cross-sections are those which can form a plastic hinge with the rotational capacity required for plastic analysis (formation of a plastic hinge mechanism) without reduction of the resistance. |  |
| Class 2 (also designated ‘compact’) cross-sections are those which can develop their full plastic moment of resistance, but exhibit limited rotational capacity because of local buckling. |  |
| Class 3 (also designated ‘semi-compact’) cross-sections are those in which, assuming an elastic distribution of stresses, the stress in the extreme compression fibre can reach the yield strength but local buckling is liable to prevent development of the plastic moment of resistance of the section. |  |
| Class 4 (also designated ‘slender’) cross-sections are those in which local buckling will occur before the attainment of the yield strength in one or more parts of the cross-section so that the moment of resistance is smaller than the elastic moment of resistance of the section. |  |
| As soon as now, two consequences may be pointed out: |  |
| - Class 2 cross-sections are not appropriate for global structural plastic analysis as far as no sufficient rotation capacity is available to form a plastic mechanism; only a global structural elastic analysis is allowed. - When Class 4 sections, explicit allowance for the effects of local buckling is necessary when determining the resistance of cross-sections and/or of members. |  |
| The different classes of sections in terms of behaviour, moment capacity and required rotational capacity are detailed in Table 1. |  |
| For axially loaded members, only two cross-sectional classes are distinguished. Class 1, 2 or 3 cross-sections may transfer the plastic load (squash load). Class 4 cross-sections may only transfer a limited axial force because of elastic local buckling. |  |

| - Classification of cross-sections, moment resistance and rotation capacity |  |
| --- | --- |

| - Behaviour of plate elements in compression |  |
| --- | --- |
| A thin flat rectangular plate subjected to a linear distribution of in-plane direct stresses (comprising a part of compressive stresses) along two opposite edges has an elastic critical buckling stress cr given by: |  |
|  | Part 1-5 A1 |
| where: |  |
| k is the plate buckling factor which accounts for edge support conditions, stress distribution and aspect ratio of the plate (see Figure 3); b is the plate width; t is the plate thickness;  Poisson’s coefficient; E Young’s modulus; |  |
| The elastic critical buckling stress c is thus inversely proportional to the square of the plate width-to-thickness ratio b/t. This ratio b/t is currently termed ‘plate slenderness’; it plays for local plate buckling the same role as the column slenderness Lfl / i for column buckling. |  |
| A hollow section consists in four plate elements which are supported along their longitudinal edges (see Figure 3 a) and are normally long compared with their width. The buckled shape for such a plate element subjected to uniform compression is illustrated in (Figure 3 a). The width of the walls has to be distinguished from the clear width b or h (see Figure 1) because of radius effects. It is here designated as c and, for a rectangular hollow section, is to be taken equal to (b-3t) or (h-3t) depending on the wall being considered. The relationship between the aspect ratio  = a/c and the buckling factor k for a long simply supported element of this type is shown in Figure 3 b, from which it is clear that the value of the buckling factor is about 4 as soon as the plate aspect ratio  exceeds unity. |  |
| a c t a)Four simply supported edges b) Plate buckling factor aspect ratio  buckling factor k - Behaviour of a plate element in uniform compression |  |
| For a plate element to be classified as Class 3 or better, the elastic critical buckling stress cr exceeds the yield strength fy. From Eq. , with  = 0,3, this will be so if: |  |
| This expression is general; indeed the respective effects of the stress gradient, of the boundary conditions and of the aspect ratio are all encompassed within the buckling factor k. Table 2 gives expressions for the buckling factor in the case of large aspect ratios ( >> 1) and linearly varying direct stress distributions. |  |
| - Buckling factors for linear direct stress distributions |  |
| 1 = maximum compressive stress. 1; 1 >  > 0; 0; 0 >  > -1; -1 >  > -2. Case I Internal element; 4,0; 7,81; 7,81 – 6,29  + 9,78  2; 5,98 (1 – 2). |  |
| The elastic-plastic behaviour of a ‘perfect’ plate element subject to uniform compression may be usefully represented by a normalised load-slenderness diagram where the ‘reduction factor’ , and the ‘relative plate slenderness’ are defined as: |  |
| where is the average ultimate stress at collapse. |  |
| Substituting Eq. for cr into Eq. , and introducing the yield strength coefficient (where fy shall be expressed in N / mm²), the relative plate slenderness may be expressed as: |  |
| where c is the appropriate width for the compression plate element, determined as explained earlier. |  |
| Figure 4 shows the relationship between  and for a perfectly flat plate. For a relative plate slenderness less than one, the reduction factor is one: the plate is able to reach its plastic resistance. For larger values of the relative plate slenderness , the reduction factor decreases as the relative plate slenderness increases, the ultimate stress being limited to the elastic critical buckling stress cr. |  |
| For such an ideal plate, the limit value of the relative plate slenderness from which the resistance of the plate is affected by plate buckling effects is . |  |
| However a plate element within a section is never a perfect plate; it is neither strictly flat nor free from residual stresses. These unavoidable imperfections result in adverse effects on the stability of the plate element. |  |
| Steel has an elastic-perfectly plastic behaviour; it may exhibit strain hardening, provided that the strains become sufficiently large. |  |
| Also, when a simply supported plate element buckles, tensile membrane forces develop in the direction perpendicular to the direction of the direct stresses; that results in a capacity for that plate of sustaining loads well beyond the level causing elastic buckling (post-buckling behaviour). |  |
| To account for all these behavioural aspects, the limit value of the relative plate slenderness needs to be reduced in order to delay the onset of local buckling until the requisite strain distribution through the section – yield strength at the extreme fibre or fully plastic stress distribution – has been attained. |  |
|  Class 3 3 Class 2 Class 1 1 0,5 0,6 0,9 1,0 Euler buckling stress |  |
| - Dimensionless representation of the elastic-plastic buckling stress |  |
| The following limit values of the relative plate slenderness have been adopted for the classification of the compression plate elements: |  |
| Class 1 |  |
| Class 2 |  |
| Class 3 for elements under stress gradient; for plate elements subjected to compression throughout. |  |
| By substituting the appropriate values of k into Eq. and noting the limit value of to be used for each class, limiting ratios of the plate width-to-thickness ratios can be calculated for compression plate elements of rectangular hollow sections. Such values applicable to rectangular hollow sections are given in Table 3, where c is the generic symbol for the width of the walls. |  |
| Limiting values of the d/t ratio for circular hollow sections, where d is the outer diameter, may be determined similarly. However, the elastic critical plate buckling stress is no more given by Eq. since the section in its whole has to be considered as a cylindrical shell. Moreover, buckling of such shells does not exhibit post-bucking strength reserve and looks more like column buckling; that paramount difference in behaviour has to be accounted for in the limit values of the relative plate slenderness. That results in the limiting d/t values given in the second part of Table 3. |  |
| - Maximum width-to-thickness ratios for compression plate elements |  |
| Rectangular hollow sections. Compression plate element subject to:; Class 1; Class 2; Class 3. Uniform compression; c / t ≤ 33 ; c / t ≤ 38 ; c / t ≤ 42 . Pure bending; c / t ≤ 72 ; c / t ≤ 83 ; c / t ≤ 124 . Circular hollow sections. Compression plate element subject to:; Class 1; Class 2; Class 3. Compression and/or bending; d / t ≤ 50 ²; d / t ≤ 70 ²; d / t ≤ 90 ² For d / t > 90 ², see EN 1993-1-6. |  |
| - Effective width approach to design of Class 4 sections |  |
| A cross-section with Class 4 elements may be replaced by an ‘effective’ cross-section, taken as the gross section minus ‘holes’ inserted where plate buckling may occur. The effective section is then designed quite similarly to a Class 3 section, i.e. using the elastic cross-sectional resistance limited by yielding in the extreme fibres. |  |
| In tubular construction, the proportions between the dimensions of the cross-section and the member length are such that shear lag has not to be considered. Therefore the wording ‘effective’ used in the following reflects the sole effects of plate buckling. |  |
| The effective cross-sectional properties of members should be based on the effective areas of the compression elements and on the area of the tension elements and their locations within the effective cross-section. |  |
| The effective area Aeff of the section should be determined assuming that the cross-section is subject only to stresses due to axial compression NEd. Similarly, the effective section modulus Weff should be determined assuming that the cross-section is subject to bending stresses due to MEd only. For biaxial bending, the effective section modulus should be determined for both principal axes (i.e. separately). |  |
| As an alternative, a single effective section may be determined for the resulting state of stress from NEd and MEd acting simultaneously. |  |
| Figure 5 shows an example of effective cross-section for a rectangular hollow section (RHS) subject to compression and bending. Notice that the centroidal axis of the effective cross-section may shift relatively to that of the gross cross-section. For a RHS in compression, there is no move of the centroidal axis because of the double symmetry of the cross-section. For the same section in bending, there is a shift because only the compression zone is affected by local plate buckling; that will duly be taken into account when calculating the section properties of the effective section. For a member subjected to combined axial force and bending, the shift of the centroidal axis will give rise to an additional bending moment which should be accounted for in the design of the member. |  |
| - Effective section of a RHS subjected to combined axial force and bending |  |
| Effective widths of compression elements may be calculated by means of a reduction factor  which depends on the relative plate slenderness – which in turn depends on the stress distribution and element boundaries through application of the buckling factor k – and duly accounts for imperfections. This reduction factor writes as follows: |  |
| where  is the ratio of the extreme stresses over the width of the compression plate element, i.e. , 1 being the maximum compressive stress. |  |
| For a plate element fully in compression, the width bc of the compression zone over the plate width is equal to the latter, thus bc = c. For a plate element partially in compression, it amounts . |  |
| The effective width of the compression zone is obtained as: |  |
| The width of the tension zone over the plate width is fully effective. |  |
| The effective width is subdivided into two parts which are respectively attached to the extreme fibres of the compression zone (see Table 4 for internal elements and Table 5 for outstands). |  |
| Proceeding similarly for all the compression plate elements composing the cross-section enables the determination of the so-called effective section relative to the loading in consideration. The elastic cross-section resistance is then calculated on the basis of the sectional and inertia properties of the effective section. Should the effective section no more be symmetrical, due account should be taken of the shift e of the centroid especially when the section is subjected to combined bending and axial force. Indeed the sole axial force increases the bending moment by an amount Ne. |  |
| It may be noticed that the maximum values of the relative plate slenderness which provide a full efficiency (i.e.  = 1) in accordance with Eq. are for uniform compression, and for pure bending. |  |
| - Effective with: Internal elements |  |
| - Effective width: Outstands |  |
| Notes: |  |
| - In both Table 4 and Table 5, beff and represent respectively the symbols bc,eff and c used earlier in the present lecture. |  |
| - There are different values used above for the classification of compression plate elements in Class 3 (respectively 0,74 for uniform compression and 0,90 for pure bending), as said on page 8. |  |

- References

- EN 1993-1-1: Eurocode 3: Design of steel structures – Part 1.1: General rules and rules for buildings. CEN, Brussels, May 2005.

- EN 1993-1-5: Eurocode 3: Design of steel structures – Part 1.5: Plated structural elements. CEN, Brussels, October 2006.

- EN 1993-1-6: Eurocode 3: Design of steel structures – Part 1.6: General rules – Strength and stability of shell structures. CEN, Brussels, February 2004.

- ESDEP: Lectures 7.2 and 7.3. The Steel Construction Institute, Silwood Park, Berkshire.

- Trahair N.S. and Bradford M.A.: The Behaviour and Design of Steel Structures, Chapter 4- Local buckling of thin plate elements, E & FN Spon, Revised Second Edition 1994.

- Zhao X-L., Wilkinson T. and Hancock G.: Cold-Formed Tubular Members and Connections, Elsevier, 2005.