- Module 5

Fatigue design

Lecture 15Introduction to fatigue design according to Eurocode 3

- Contents:

1.	Introduction

2.	Basic concepts and principles

2.1.	Fatigue strength and fatigue life

2.2.	Fatigue damage accumulation

2.3.	Principles of fatigue resistance verification

2.4.	Partial safety factors

3.	Design values of stress ranges

3.1.	Design value of nominal stress range

3.2.	Design value of modified nominal stress range

3.3.	Design value of stress range for hot spot stress

4.	Fatigue strength

4.1.	Nominal stress range and modified nominal stress range

4.2.	Hot spot stress ranges

4.3.	Fatigue strength modifications

4.3.1.	Non-welded or stress relieved welded details in compression

4.3.2.	Size effect

5.	Fatigue verification

5.1.	Limit of application of the EN 1993-1-9 rules

5.2.	Verification format based on the fatigue strength

5.3.	Verification based on the damage accumulation

6.	References

| - Introduction |  |
| --- | --- |
| A structure subjected to fluctuating loadings may, in the course of its life, fail after a certain repetition of such loads whilst its ‘static’ resistance has never been exhausted. This phenomenon is designated as ‘fatigue’ and it results from pre-existent micro- or macroscopic cracks. |  |
| Fatigue is thus a mechanism whereby cracks grow in a structure. This grow only occurs under fluctuating stresses and final failure generally occurs in region of tensile stresses when the crack has reached a size such that the reduced cross-section becomes insufficient to carry the peak load without rupture. |  |
| Failure due to fatigue cracks is sudden and may be either ductile or brittle. |  |
| Many structures, such as building frames, do normally not experience sufficient fluctuating stress to give rise to fatigue problems. Others do, such as slender structural elements subjected to wind loads, road and almost all railway bridges, cranes and off-shore structures, where the live loading is a significant proportion of the total load and the way the loads are applied is more dynamic than static. |  |
| In a plain material the growth of a fatigue crack presents three stages: - Initiation stage: - The crack initiates from grain scale microscopic defects at the close vicinity of the surface; it develops over some grains according to a direction at 45° on the local tensile stress and then takes an orientation perpendicular to the latter. The crack may propagate through or between the material grains. - Slow propagation rate: - Once a crack is initiated (i.e. when it is 0,1 to 0,5 mm deep), it grows approximately in accordance with Paris’ rule; this law has been established based on fracture mechanics and a lot of experimental results: - where: - is the growth rate of the crack size a for one cycle; - C is a material constant determined experimentally and comprised, assuming m = 3, between 1,210-13 and 510-13 for structural steels; - K is a function of the stress fluctuation (= K (max) – K (min)). - This crack propagation phase may lead to visible cracks. - Sudden failure: - It occurs when the size of the crack reaches a critical value ac or, alternatively, when K(max) reaches a critical value K1c designated as ‘material toughness’. |  |
| In welded structures, fatigue cracks start in the majority of cases to grow from welds rather than from other details. There are several reasons for that: |  |
| - Welding processes leave metallurgical discontinuities which are as many potential crack sources. In contrast with plain wrought material, the initiation period, which is normally needed to start a crack, is either very short or even non-existent in welded details. Cracks therefore spend most of their life propagating, i.e. getting longer; - Structural welds have usually a rough profile. Sharp changes of direction generally occur at the toe of butt welds and at the toes and root of fillet welds (Figure 1) with the consequence of local stress concentrations (Figure 2); - Unless the structural details are stress relieved, welds are always affected by high residual stresses due to rolling process, flame cutting, welding process and differential shrinkage of thermal stresses during cooling down. |  |
| - Small discontinuities in presence of high local stress levels react as though they are in a more highly stressed member and grow faster. |  |
| Weld toe Crack Weld toe Cracks Weld root Butt weld (full penetration)Fillet weld |  |
| - Cracks in welded details |  |
| - Local stress concentration at weld toe |  |
| Fracture mechanics shows that, given same stress fluctuation and degree of stress concentration, the growth rate of a crack is proportional to the square root of its size a. Therefore a fatigue crack spends most of its life as a very small crack which is hardly detectable. It causes a significant reduction of the cross-section area only in the last stages of life; in-service inspections of structures are therefore problematic. A typical crack growth curve is illustrated in Figure 3. |  |
| In the design practice, regions subjected to compressive stresses are checked for fatigue similarly to regions subjected to tensile stresses. Indeed, though the risk of fatigue failure is lower in compression, the presence of high residual stresses in the vicinity of and in the welded zones is likely to result in opening the crack. |  |
| Though fatigue may occur in plain material, constructional details, and especially welded details, are more questionable. Therefore it is referred mostly to welded details in the following because welding is the most common connection technique used in tubular construction. |  |
| - Crack growth curve |  |
| Fatigue cracks which occur during service life do not necessarily mean the end of the service life; they should be carefully repaired so as to avoid introducing more severe notch conditions. |  |
| The assessment methods presented in EN 1993-1-9 use fatigue resistance in terms of fatigue strength curves for two types of details: |  |
| - Standards details applicable to ‘nominal stresses’; - Reference weld configurations applicable to ‘geometric stresses’. |  |
| - Both types of stresses from fatigue actions are later commented on. |  |
| Present lecture mainly concentrates on the determination of the fatigue resistance of CHS and RHS welded joints in accordance with Part 1-9 of Eurocode 3, designated EN 1993-1-9. |  |
| - Basic concepts and principles |  |
| Fluctuating stresses producing fatigue may be either direct stresses  or shear stresses . In some cases it is preferable to refer to principal direct stresses. In the following the symbol  is used to designate the direct stress range. It shall be kept in mind that, except otherwise stated, what is said about  can be repeated for shear stress range . |  |
| - Fatigue strength and fatigue life |  |
| First, let us consider, for sake of simplicity, a welded detail subjected to a stress range  which fluctuates with ‘constant amplitude’. The stress range  is the difference between the two extreme stresses of a particular stress cycle (Figure 4). |  |
| Should the stress range  be given, then the ‘fatigue life’, also designated ‘endurance’, is defined as the number of cycles NR required to cause failure under a constant stress range . That is the most usual way to approach fatigue design. |  |
| The ‘fatigue strength’ of this welded detail is defined as the constant stress range R which causes failure after a given number of cycles N. |  |
| The fatigue life of welded joints depends mainly upon the joint type, the type of loading and the structural detailing of the joint. Fabrication and all kinds of imperfections (material, misalignments) influence the fatigue life as well. The loading (axial force and bending moment) in the joints depends on the type of structure and the fatigue actions. |  |
|  max min mean Number of cycles N Stress  |  |
| - Constant amplitude stress range |  |
| However, in general, a structure prone to fatigue is subjected to a ‘loading spectrum’ rather than to constant amplitude loading. Otherwise speaking, the structure experiences variable stress ranges. For fatigue verification purposes, such a spectrum recorded during a relatively short period of time is first extrapolated to the expected service life of the structure. Second it is substituted by a histogram of stress ranges (Figure 5). This histogram is actually an accumulation of ‘stress blocks’, each of which having a constant amplitude (that varies from block to block) and a certain number of cycles. |  |
| - Example of spectrum and corresponding histogram of stress ranges |  |
| For the verification of the fatigue resistance under variable stress range loading, the concept of fatigue damage accumulation is useful. |  |
| - Fatigue damage accumulation |  |
| The direct stress range  = max – min) is the governing parameter for fatigue design. Due to the presence of high residual stresses, the magnitude of which is not known accurately, the stress ratio R = min / max is generally not taken into account in modern fatigue design except, possibly, when the structure is fully stress relieved. |  |
| For constant amplitude stress ranges, it is agreed that there is no fatigue damage when the stress ranges are below a certain limit designated as ‘constant amplitude fatigue limit’. |  |
| For variable amplitude stress ranges, no fatigue damage is supposed to develop when the stress ranges are below another limit designated as “cut-off fatigue limit”, lower than the ‘constant amplitude fatigue limit’. |  |
| When the stress ranges of a constant amplitude loaded structure or the maximum stress range for a variable amplitude loaded structure is above the constant amplitude fatigue limit, the fatigue damage accumulation D can be assessed based on the Palmgren-Miner linear rule introduced below. |  |
| - In non aggressive environment the allowable ‘fatigue damage’ for structures is generally taken as 1,0 provided the effect of fatigue cracks and the possibility for in-service inspections are duly accounted for by appropriate partial safety factors. That means that D = 1,0 corresponds to fatigue failure. |  |
| If the number Ni of cycles is the fatigue life for a constant stress range i, Palmgren assumed that each cycle with i produces a same partial damage di obtained as: |  |
| - Consequently, ni cycles with i (with ni ≤ Ni) produce altogether a damage: |  |
| In case of variable stress range loading composed of k stress blocks of ni cycles of constant amplitude i, the accumulation of damage D is: |  |
| - Based on fatigue tests, Miner has observed that fatigue failure occurs when the damage D reaches a value close to unity. In accordance with all the standards dealing with fatigue, putting D = 1 means that the fatigue life is exhausted. It is thus important to get a value lower than unity when designing constructional details for fatigue based on the cumulated damage concept: |  |
| - Principles of fatigue resistance verification |  |
| Several methods have been developed to check the fatigue resistance of welded joints of structural hollow sections. Some of them are of a more fundamental nature (fracture mechanics), some others are much more appropriate to practice purposes. Present lecture concentrates on the latter; more especially they are: |  |
| - The ‘classification method”: this method is based on structural details for different types of joints which are classified into various categories with about the same fatigue life. Each detail category corresponds to a constant amplitude ‘nominal stress range’ under which a joint will fail after 2106 cycles. The classification method has been adopted by many national standards; it is the first method included in EN 1993-1-9; - The ‘hot spot stress method’: this method which is also called ‘geometric stress method’ relates the fatigue life of a joint to the so-called hot spot stress at the joint. It takes the uneven stress distribution around the perimeter of the joint directly into account. The hot spot stress range includes the influences of geometry and type of load but exclude the effects due to fabrication such as the configuration of the weld (flat, convex, concave) and the local condition of the weld toe (radius, undercut, …). The hot spot stress is the maximum geometrical stress occurring in the joint where the cracks are usually initiated; in the case of welded joints, this generally occurs at the weld toe. This method has been recommended by the Institute of Welding for design of welded tubular joints under fatigue loading; it is the second method included in EN 1993-1-9. |  |
| - Partial safety factors |  |
| In accordance with the semi-probabilistic approach for structural safety, partial safety factors for fatigue loading Ff and for fatigue strength Mf need to be accounted for in fatigue checks. |  |
| Fatigue needs to be checked for so-called ‘fatigue loading’. This loading is close to but normally less than the loads corresponding to service conditions. Fatigue loading is described in EN 1993-1-9; that question is no further discussed herein. The relevant partial safety factor Ff is mostly taken as 1,0 in accordance with the recommendation of EN 1993-1-9. |  |
| - The value of the partial safety factor Mf for fatigue strength depends, on the one hand, on the type of fatigue assessment – ‘damage tolerant method’ or ‘safe life method’ – and, on the other hand, on the degree of consequence of the fatigue failure. | Part 1-9 Chapter 3 |
| - In the ‘damage tolerant method’, it is expected that the constructional detail will perform satisfactorily with an acceptable reliability for its specified design life provided inspection and maintenance regime for detecting and correcting possible fatigue damage is implemented throughout the design life. In contrast, with the ‘safe life method’, such a satisfactorily behaviour is reliably expected during the whole design life without the need of regular in-service inspection for fatigue damage. |  |
| - Regarding the consequence of fatigue failure, one may distinguish between ‘low consequence’ (for instance the failure of non essential part of the structure or of a redundant component) and ‘high consequence’ (for instance the failure of a statically determinate structural element of a bridge). In EN 1993-1-9, values of Mf are recommended (see Table 1); they have to be fixed in the National Annexes. |  |
| - Recommended values for partial safety factor Mf for fatigue strength |  |
| - Assessment method; - Consequence of failure. - Low consequence; - High consequence. - Damage tolerant; - 1,00; - 1,15. - Safe life; - 1,15; - 1,35. |  |
| - Design values of stress ranges |  |
| - Design value of nominal stress range |  |
| Fatigue assessment is carried out based on ‘nominal stress ranges’ for standard constructional details from tubular construction identified in EN 1993-1-9. These details are listed in Table 2 and Table 3. |  |
| Nominal stresses are calculated at the site of potential fatigue crack initiation. Effects producing stress concentrations at constructional details listed in Table 2 and Table 3 are accounted for in the resistance side of the fatigue assessment relation through the fatigue strength that is allotted to these details. |  |
| For the fatigue assessment, the design value of nominal stress ranges is designated: |  |
|  | Part 1-9 6.2 |
| where  (FfQk) is the stress range caused by the fatigue loads specified in EN 1991 and i are damage equivalent factors depending on the spectra as specified in the relevant parts of EN 1993. In Eq. , the values of i need to be determined in accordance with a number of cycles NC = 2106 (therefore the index 2 in E,2). |  |
| Nominal stresses caused by fatigue loads shall be obtained from a linear elastic analysis for members and connections with a modelling accounting for all action effects including distortional effects. |  |

| - Constructional details – Hollow sections (t  12,5 mm), taken from [3] |  |
| --- | --- |
| - Constructional details – Lattice girder node joints, taken from [3] |  |
| - Design value of modified nominal stress range |  |
| Fatigue assessment is carried out based on ‘modified nominal stress ranges’ when abrupt changes of sections occur close to the site of potential fatigue crack initiation. Effects producing stress concentrations at constructional details other than those included in Table 2 and Table 3 shall be accounted for by using a stress concentration factor so as to get a ’modified nominal stress’. These stress concentration effects are thus accounted for in the loading side of the fatigue assessment relation. |  |
| In any generality, the design value of ‘modified nominal stress ranges’ is designated: |  |
|  | Part 1-9 6.3 |
| where kf is the stress concentration factor which accounts for the local stress magnification in relation to detail geometry not yet covered by the reference R-N curves; kf-values may be taken from handbook or from appropriate finite element calculations. |  |
| As yet said above, FfE,2 corresponds to Nc = 2106 cycles. |  |
| For the particular case of lattice girders made of hollow sections simplified truss models with pinned connections may be used. Provided that the stresses due to external loading applied to members between joints are taken into account, the effects from secondary moments due to the stiffness of the connection can be allowed for by the use of k1 factors (see Table 4 for circular hollow sections and Table 5 for rectangular hollow sections). |  |
| - k1 factors for CHS under in-plane loading, taken from [3] |  |
| - k1 factors for RHS under in-plane loading, taken from [3] |  |
| For welded joints of hollow sections, the design value of stress ranges FfE,2 is calculated as: |  |
| where is the design value of nominal stress range calculated with a simplified truss model with pinned connections and k1 is the magnification factor drawn from Table 4 or Table 5. |  |
| It is reminded that (welded joints of hollow sections) corresponds to Nc = 2106 cycles. |  |
| - Design value of stress range for hot spot stress |  |
| - Fatigue assessment of welded joints of hollow sections in lattice girders is carried out based on ‘hot spot stress ranges’ where high stress gradients occur close to a weld toe in joints covered by Table 6. The detail categories given in this table are relative to cracks initiating from: |  |
| - Toes of butt welds; - Toes of fillet welded attachments; - Toes of fillet welds in cruciform joints. |  |
| The design values of the ‘hot spot stress ranges’ FfE,2 is calculated as: |  |
|  | Part 1-9 6.5 |
| where kf and have been introduced earlier. Because the values of kf are not given in EN 1993-1-9, they are to be searched for in the scientific literature; much information, such as analytical expressions and charts and tables, may be found in CIDECT Design Guide 8 [4]. |  |

| - Detail categories for use of the hot spot stress method (Table B1 taken from [3]) |  |  |  |
| --- | --- | --- | --- |
| - Fatigue strength |  |  |  |
| - From Paris’law, the fatigue life N expressed in terms of number of cycles to failure is obtained as: |  |  |  |
| which writes also: |  |  |  |
| and for shear stresses: |  |  |  |
| Predictions of fatigue life are therefore particularly sensitive to accuracy of stress range evaluations. |  |  |  |
| - Nominal stress range and modified nominal stress range |  |  |  |
| For constant amplitude nominal stresses the fatigue strength is represented by a series of such log R – log N curves, which are called S-N curves and correspond to typical detail categories; S means stress range and N number of cycles. |  |  |  |
| Each detail category is designated by a number which represents (in N/mm²) the reference value C or C for the fatigue strength corresponding to a fatigue life of at 2106 cycles under constant amplitude nominal stresses: |  |  |  |
| In EN 1993-1-9 the S-N curves relative to fatigue under direct stresses have a conventional tri-linear shape illustrated in Figure 6: | Part 1-9 Figure 7-1 |  |  |
| - Segment 1: It corresponds to above expression . It is fully determined by i) its slope m = 3, and ii) one characteristic point which is the reference fatigue strength C of the detail category. It extends till N = ND = 5106 cycles to which, in accordance with the fatigue tests, correspond a so-called ‘fatigue limit for constant amplitude stress range D at the number of cycles ND. If the stress range is lower than D, then there is no damage due to fatigue – i.e. cracks do not propagate - and fatigue check is not requested. | Part 1-9 Figure 7-1 |  |  |
| - As a result of Paris’ law, it is easily found that: |  |  |  |
| - Segment 2: When the loading is a spectrum, the latter is substituted by a histogram, i.e. a succession of sets of ni cycles of constant amplitude i. When each part of the histogram has a i lower than D, above conclusion applies and there is no damage. When, in contrast, the histogram is composed of parts some stress ranges of which are lower than D and others higher than D, the latter cannot be anymore disregarded. Indeed, in accordance with fracture mechanics, the size of the crack plays a role in the crack propagation and those parts with  > D cause cracks to get a size such that those with  < D will contribute the crack propagation. Therefore the segment 2 starts from point (ND, D) and extends with a smaller slope than segment 1 (taken as m = 5) till NL = 108 cycles to which corresponds a ‘cut-off limit’ L for stress ranges at the number of cycles NL. |  |  |  |
| As a result of Paris’ law, it is easily found that: |  |  |  |
| - Segment 3: It is an horizontal line; all the  < L are sufficiently small for not contributing any crack propagation, i.e. stress ranges  < L can be ignored in fatigue check. |  |  |  |
| - It is while stressing that the part of the S-N curves below the fatigue limit D is fictitious and does not correspond directly to a physical behaviour. It has been adopted in order to simplify the determination of the cumulated damage, especially for enabling the use of Eq. , relative to the damage di produced by  < D as for those  >D. |  |  |  |
| - It should be stressed that, for lattice girder node joints – in contrast with what is written above – the statistical evaluation has shown that a single slope m = 5 is more appropriate in the whole range 0 < N  108 cycles. That is clearly stated in the first column of the Table 3. The shape of the S-N curves for direct stresses to be used for these types of constructional details looks like the typical S-N curves for shear stresses (see Figure 8). |  |  |  |
| Number of cycles Direct stress range 1 m = 3 m = 5 Cut-off limit Constant amplitude fatigue limit C D L 2106 5106 108 Detail category |  |  |  |
| - Typical shape of the S-N curves for direct stresses |  |  |  |
| EN 1993-1-9 gives fifteen equidistant fatigue strength curves (S-N curves) applicable for direct stress ranges (see Figure 7). Each of these curves is identified by the ‘detail category’, i.e. the C value associated to an endurance of 2106 cycles; this value is given in the first column of Table 2 and Table 3 for the standard constructional details. |  |  |  |
| - Fatigue strength curves for direct stresses |  |  |  |
| In EN 1993-1-9, all the S-N curves relative to fatigue under shear stresses have a conventional bi-linear shape (Figure 8): | Part 1-9 Figure 7.2 |  |  |
| - Segment 1: it corresponds to above expression . It is fully determined by i) its slope m = 5, and ii) one characteristic point which is the reference fatigue strength C of the detail category. It extends till N = NL = 108 cycles to which, in accordance with the fatigue tests, correspond a ‘cut-off limit” for constant amplitude stress ranges L at the number of cycles NL. - As a result of Paris’ law, it is easily found that: | Part 1-9 Figure 7.2 |  |  |
| - Segment 2: it is a horizontal line; all the  < L are sufficiently small for not contributing any crack propagation. |  |  |  |
| EN 1993-1-9 gives two fatigue strength curves (S-N curves) applicable for shear stress ranges (see Figure 9). Each of these curves is identified by the ‘detail category’, i.e. the C value. |  |  |  |
| The National Annexes may give fatigue strength categories values C and C for details not covered in Table 2 and Table 3. |  |  |  |

| Number of cycles Shear stress range  1 m = 5 Cut-off limit Detail category C L 2106 108 |  |
| --- | --- |
| - Typical shape of the S-N curves for shear stresses |  |
| - Fatigue strength curves for shear stresses |  |
| - Hot spot stress ranges |  |
| The fatigue strength categories C for geometric stress ranges are given in the first column of Table 6. |  |
| The National Annexes may give fatigue strength categories values C and C for details not covered in Table 6. |  |
| - Fatigue strength modifications |  |
| - Non-welded or stress relieved welded details in compression |  |
| For what regards fatigue, compression is less critical than tension. Yet it has been stated that, because of residual stresses, what is meant in compression in welded details might actually be in tension; therefore practice is to check fatigue resistance as well in compression than in tension. However there are situations where the residual stresses are low or mostly inexistent so that above procedure could result in an undue penalty. |  |
| Such a situation is met when non-welded or stress relieved welded details in compression. Then an ‘effective stress range’ may be used which is obtained by adding the tensile portion of the stress range and 60% only of the magnitude of the compressive portion: | Part 1-9 7.2.1 |
| - Effective stress range |  |
| - Size effect |  |
| It has been found that fatigue strength is not independent of some size effect due to thickness and other dimensional effects. That should be taken into account by reducing the fatigue strength from C to C,red according to: | Part 1-9 7.7.2 |
| or applying the recommendations given in Table 2 and Table 3 for some of the constructional details. |  |
| - Fatigue verification |  |
| The general method for fatigue assessment given in EN 1993-1-9 follows the principles of design verification by comparing action effects and fatigue strength. As an alternative, the fatigue assessment may be based on the concept of damage accumulation. |  |
| For that purpose fatigue actions must be determined in accordance with EN 1993-1-9. Fatigue actions are specific actions which are different from actions for ultimate limit states (ULS) and serviceability limit state (SLS) verification. |  |
| - Limit of application of the EN 1993-1-9 rules |  |
| Nominal, modified nominal or geometric stress ranges due to frequent loads defined in accordance with EN 1990 should not exceed: | Part 1-9 8(1) |
| for direct stress ranges |  |
| or: |  |
| for shear stress ranges |  |
| - Verification format based on the fatigue strength |  |
| Under fatigue loading, it shall be verified that: |  |
|  | Part 1-9 8(2) |
| or: |  |
| For some details, stress ranges might be required to be based on principal stresses. |  |
| - Verification based on the damage accumulation |  |
| For fatigue assessment in practice using the design spectrum, the general concepts of partial and total damages as well as the endurance values (in cycles) must incorporate the partial safety factors. |  |
| Therefore the applied stress ranges i should be multiplied by partial safety factor Ff and the fatigue strength values C divided by partial safety factor Mf in order to obtain the fatigue resistance limit NR,i for each stress block in the spectrum. |  |
| The damage Dd during the design life of the structure is calculated from: |  |
|  | Part 1-9 Annex A |
| where: nEi is the number of cycles associated with the stress range Ff i for stress block i in the factored spectrum; |  |
| NRi is the endurance (in cycles) obtained from factored C / Mf –NR curve for a stress range Ff i. |  |
| Based on the damage accumulation, it should be verified that: |  |
|  | Part 1-9 A.6(1) |
| Alternatively the design stress range spectrum (histogram) may be transformed into an equivalent constant amplitude design stress range which produces the damage Dd for a total number of cycles N = ni. |  |
| Based on the equivalent stress range, the following criterion should be met: |  |
| The value of m is either m = 3 or m = 5 as explained more in detail in section 4.1. |  |

- References

- EN 1990: Eurocode: Basis of structural design. CEN, Brussels, April 2002.

- EN 1991: Eurocode 1: Actions on structures – Part 1-1: Actions on structures - densities, self-weight and imposed loads. CEN, Brussels, April 2002.

- EN 1993-1-9: Eurocode 3: Design of Steel Structures – Part 1-9: Fatigue. CEN, Brussels, May 2005.

- CIDECT: Design Guide for circular and rectangular hollow section welded joints under fatigue loading, n°8, TÜV-Verlag, Köln, 2000.