- Module 4

Design of joints

Lecture 14Bolted connections

- Contents:

1.	Introduction to the design of bolted joints

2.	Flange-plate connections

2.1.	Flange plate connections to rectangular hollow sections

2.1.1.	Bolted on two sides of the RHS

2.1.2.	Bolted on four sides of the RHS

2.2.	Flange plate to circular hollow sections

3.	Gusset-plate connection

3.1.	RHS to gusset-plate connections

3.2.	Net area, effective net area and reduced effective net area

3.3.	Consideration of the shear-lag

3.3.1.	The shear-lag criterion

3.3.2.	The Whitmore criterion

4.	Semi-rigid connections

4.1.	Bolted hollow section beam and column connections

4.1.1.	CHS beam-to-column connections

4.1.2.	RHS beam-to-column-connections

4.2.	Bolted I-beam-to-hollow section column connections

4.2.1.	I-beam-to-CHS column connections

4.2.2.	I-beam-to-RHS column connections

5.	Bolted through diaphragm connections

6.	Blind bolting connections

6.1.	Flowdrill

6.2.	End plate connections with blind bolts

7.	Nailed connections

8.	References

| - Introduction to the design of bolted joints |  |
| --- | --- |
| The calculation methods used for many types of bolted connections between, or to hollow sections are not basically different from those used for any other type of connection in conventional steel construction. |  |
| Bolted connections are especially desirable for site joints between prefabricated sub-assemblies. |  |
| The following figures (Figure 1 to Figure 3) show different examples of bolted connections. |  |
| - Bolted truss support connections (CIDECT Design Guide 1, 1991) |  |
| - Bolted purlin connections (CIDECT Design Guide 1, 1991) |  |
| - Bolted end connections (CIDECT Design Guide 1, 1991) |  |
| - Flange-plate connections |  |
| - Flange plate connections to rectangular hollow sections |  |
| Not only bolting along all four edges of the plate are effective, also the option of bolting along two edges. |  |
| - Bolted on two sides of the RHS |  |
| By selecting specific connection parameters, the tensile resistance of the member by bolting along only two sides of the tube can be reached. |  |
| - Bolts on two sides (Wardenier 2002) |  |
| - Design steps for RHS flange-plate connections with bolts along two sides (CIDECT Design Guide 3, 1992) |  |
| This design method should be restricted to the range of flange-plate thickness over 12 to 26 mm. |  |
| - Bolted on four sides of the RHS |  |
| - Bolts on four sides (Wardenier 2002) |  |
| The flange plate thickness should be less than the bolt diameter for connections with four bolts. |  |
| The flange plate thickness should be less than the bolt diameter plus 3 mm for connections having eight bolts. |  |
| For flange-plate connections bolted along all four sides design guide lines are given in chapter 11.1 of CIDECT Design Guide 9. |  |
| - Flange plate to circular hollow sections |  |
| Economical structural tension connections can be obtained if prying force is permitted at the ultimate limit state, with the connection proportioned on the basis of a yielding failure mechanism of the flange plates. |  |
| Primary structural connections are preferably to be designed on the basis of the yield resistance of the circular hollow section. |  |
| - Bolted CHS flange plate connection |  |
| The flange plate thickness tp can be determined from: |  |
| where tp = flange plate thickness, N = tensile member force, fyp = yield strength of plate and |  |
| In this simplified method it is required that e1 =e2. |  |
| The number of bolts n can be determined from: |  |
| Nb is the ultimate tensile resistance of a bolt, with no allowance for prying forces. |  |
| - Gusset-plate connection |  |
| The design can be based on the various possible failure modes, e.g. for a tension member: |  |
| - Yielding of the cross section; - Rupture of the effective net area or; - Rupture of the effective net area reduced for shear lag. |  |
| - RHS to gusset-plate connections |  |
| RHS bracing members can be field bolted to gusset plates which have been shop welded to RHS chord members. Thus producing bolted shear connections as shown in Figure 7. |  |
| - Bolted RHS gusset-plate connections (Wardenier 2002) |  |
| An important limitation to the use of RHS gusset-plate connections is the need to have closely matching member widths. |  |
| Equal width members are connected directly. The gussets often need to be spread slightly be jacking after welding is complete in order to allow field assembly. |  |
| - Net area, effective net area and reduced effective net area |  |
| To describe various failure modes for a tension member with holes or openings the concept of gross area, net area, effective net area and reduced effective net area can be used. |  |
| The three basic checks are: |  |
| (yielding on gross area, M0 = 1,0) |  |
| (rupture on effective net area, M2 = 1,25) |  |
| (net cross-section reduced for shear lag) |  |
| The effective net area Anet is the sum of individual net areas Anet along a potential critical section of the member. |  |
| - Consideration of the shear-lag |  |
| - The shear-lag criterion |  |
| For bolted connections the effective net area, reduced for shear lag, A*net is the effective net cross-section Anet multiplied by a shear lag factor. |  |
| This factor comes into operation when a member is connected by some, but not all of its cross-sectional elements, if the critical net section includes elements which are not connected. |  |
| The critical net section in this instance may include net area segments An which are perpendicular to the load or inclined to it, but not those which are parallel to it. |  |
| The shear lag factor to be applied to Anet can be taken as: |  |
| - 0,90 when shapes like I-sections (or tees cut from them) are connected only by their flanges with at least three transverse rows of fasteners (flange width at least 2/3 the depth), - 0,85 for all other structural shapes (e.g. RHS) connected with 3 or more transverse rows of fasteners, - 0,75 for all members (e.g. RHS) connected with 2 rows of fasteners. |  |
| - The Whitmore criterion |  |
| Another failure mode of the gusset plate which must be checked is yielding across an effective dispersion width of the plate, which can be calculated using the Whitmore effective width concept. |  |
| - Withmore criterion for gusset-plate yielding (CIDECT Design Guide 3) |  |
| For this failure mode (for two gusset plates) Ni* is: |  |
| where M = 1,0; p = sum of the bolt pitches in a bolted connection or the length of the weld in a welded connection. |  |
| The use of Ni*indicates that this check applies to both tension and compression load cases. |  |

| - Semi-rigid connections |  |
| --- | --- |
| - Bolted hollow section beam and column connections |  |
| Bolted beam to column connections can be designed in various ways, mainly depending on the type of load that has to be transmitted. |  |
| Bolted connections between hollow section members can be made using: |  |
| - Flange plates; - Gusset plates; - Angles; - Cleats; - Cut-outs of open sections. |  |
| Most bolted connections are designed as: |  |
| - Shear connections; - Tension loaded splices; - Stiffened moment connections. |  |
| - CHS beam-to-column connections |  |
| The only common type of bolted moment connections between CHS members is shown in Figure 9(a): |  |
| - Bolted knee assemblies of CHS or RHS members for portal frames (CIDECT Design Guide 9, 2004) |  |
| It is recommended to use a plate thickness such that the connection can be assumed to be rigid. The bolts should be designed for the moment capacity of the connected hollow section. |  |
| - RHS beam-to-column-connections |  |
| Further to the connection types in Figure 9(b) and (c), the following figure shows a bolted flange plate connection between RHS column and an I-section beam. |  |
| - Bolted flange plate connection between RHS members (CIDECT Design Guide 9) |  |
| - Bolted I-beam-to-hollow section column connections |  |
| Bolted connections between I-beams and hollow section columns can be distinguished in: |  |
| - Connections with continuous beams; - Through plates connections; - Connections with continuous columns. |  |
| The strength and stiffness of bolted connections with a continuous beam is directly dependent on the thickness of the cap plates, the reinforcement of the beam and the bolts. It is recommended to design these connections as rigid moment connections with relatively thick cap plates. |  |
| The trough-plate connections allow a direct load transfer from beam to beam or to a column whereas the shear is transferred by shear tabs or angles welded to the column web. |  |
| Figure 11 shows bolted continuous beam-to-column connections: |  |
| - Bolted continuous beam-to-column connections (CIDECT Design Guide 9) |  |
| Figure 12 shows bolted through-plate moment connections: |  |
| - Bolted through-plate moment connections (CIDECT Design Guide 9) |  |
| Figure 13 shows a bolted-beam-to-column connection with a continuous column: |  |
| - Bolted-beam-to-column connection with a continuous column (CIDECT Design Guide 9) |  |
| - I-beam-to-CHS column connections |  |
| Most of the bolted moment connections between I-beams and CHS columns are stiffened with plates and can be designed as rigid connections. |  |
| - I-beam-to-RHS column connections |  |
| Besides the bolted I-beam-to-RHS column connections with extended plates nowadays it is also possible to connect directly to the face of the RHS column. In this case single sided (also called blind) bolting is used. |  |
| - Bolted through diaphragm connections |  |
| The through diaphragms of these beam-to-column connections are extended sufficiently far to accommodate bolted beam splices used as field connections. |  |
| Figure 14 shows bolted through diaphragm connections: |  |
| - Bolted through diaphragm connections(CIDECT Design Guide 9) |  |
| These connections are similar in ultimate behaviour to the bolted flange plate connections recommended in FEMA 350 (2000). |  |
| Good inelastic behaviour is achieved with balanced yielding in the following three preferred mechanisms: |  |
| - Flexural yielding and local buckling of the beam adjacent to the beam splice; - Yielding of the stub-beam; - Yielding of the beam splice. |  |
| - Blind bolting connections |  |
| In recent years, systems have been developed for direct connections for one side only. |  |
| A number of patented blind bolting systems is available, e.g. Huck “Ultra Twist Blind Bolt”, Lindapter “HolloFast” and “HolloBolt”. |  |
| - Lindapter “HolloFast” (Wardenier 2002) |  |
| The systems are based on the principle that after bringing them in from one side the bolts are torqued and a “bolt head” forms on the inside of the connection. |  |
| The design rules for blind bolting systems are based on typical failure modes, i.e.: |  |
| - Punching shear of the fastener through the column face; - Yielding of the column face; - Bolt failure in shear, tension or a combination of both. |  |
| - Flowdrill |  |
| The flowdrill system is a special patented method for extruded holes. |  |
| - Flow drill connection for joining double angle web cleats or flexible end plates to RHS (Wardenier 2002) |  |
| - Flowdrill process (Wardenier 2002) |  |
| Flowdrilling is a thermal drilling process to make a hole through the wall of hollow section by bringing a tungsten carbide bit into contact with the hollow section wall and generating sufficient heat by friction to soften the steel. |  |
| The bit moves through the wall, the metal flows to form an internal bush. |  |
| In the subsequent step, the bush is threaded using a roll tap. |  |
| Wall thicknesses up to 12.5 mm can be recommended by using the flowdrill method. |  |
| Threaded roll tapped holes with M16, M20 and M24 ISO course thread profiles can be made. |  |
| The full tension capacity of grade 8.8 bolts can be carried, provides that the RHS thickness is equal to or grater than the minimum thickness shown in the following table and fy between 275-355 N / mm². |  |
| - Minimum RHS thickness(CIDECT Design Guide 7, 1998) |  |
| Bolt size; Minimum RHS thickness [mm]. M16 grade 8.8; 6.4. M20 grade 8.8; 8.0. M24 grade 8.8; 9.6. |  |
| The shear and bearing capacities of the hole can be calculated in the normal manner. |  |
| In most applications in which the bolts are loaded in tension, the deformation or yielding of the RHS face will determine the overall connection capacity and not the capacity of each individual bolt. |  |
| - End plate connections with blind bolts |  |
| Beams with end plates are directly connected to column faces by tension bolts. Closed column sections necessitate the use of blind bolts. |  |
| The column walls have to be reinforced to prevent local distortion of the column walls and to fulfil the requirements for full strength connections. |  |
| The column walls may be partially thickened over the areas where the end plates are attached. |  |
| - Nailed connections |  |
| CHS can be nailed together to form reliable structural connections. |  |
| They are verified for splice connection between two co-axial tubes. |  |
| In such a connection, one tube can fit snugly inside the other. |  |
| - Nailed CHS connection (from Packer) |  |
| The observed failure mode were: |  |
| - Nail shear failure; - Tube bearing failure; - Net section fracture of the tube. |  |
| Simple design formulae, derived from bolted and riveted connections, have been verified for both these load cases. |  |

- References

- Dutta, D., Wardenier, J. Yeomans, N., Sakae, K. Bucak, Ö., Packer, J.A. 1998. DG No 7, Design guide for fabrication, assembly and erection of hollow section structures. CIDECT Construction with hollow steel sections. TÜV Verlag Köln.

- EN 10210-1. 2006. Hot finished structural hollow sections on non-alloy and fine grain steels – Part 1: Technical delivery conditions: German version.

- EN 10210-2. 2006. Hot finished structural hollow sections on non-alloy and fine grain steels – Part 2: Tolerances dimensions and sectional properties: German version.

- EN 10219-1. 2006. Cold formed structural hollow sections of non-alloy and fine grain steels Part 1: Technical delivery conditions: German version.

- EN 10219-2. 2006. Cold formed structural hollow sections of non-alloy and fine grain steels Part 2: Tolerances, dimensions and sectional properties: German version.

- EN 1993-1-8. 2005. Eurocode 3: Design of steel structures – Part 1.8: Design of joints: German version.

- Kurobane, Y., Packer, J.A., Wardenier, J., Yeomans, N., 2004. DG No 9, Design guide for structural hollow section column connections. CIDECT Construction with hollow steel sections. TÜV Verlag Köln.

- Packer, J.A., Wardenier, J., Kurobane, Y., Dutta, D., Yeomans, N. 1991. DG No 1, Design Guide for circular hollow sections (CHS) joints under predominantly static loading. CIDECT Construction with hollow steel sections. TÜV Verlag Köln.

- Packer, J.A., Wardenier, J., Kurobane, Y., Dutta, D., Yeomans, N. 1992. DG No 3, Design Guide for rectangular hollow section (RHS) joints under predominantly static loading. CIDECT Construction with hollow steel sections. TÜV Verlag Köln.

- Wardenier, J. 2002. Hollow Sections in Structural Applications. Bouwen met Staal, Zoetermeer, The Netherlands.

- FEMA. 2000. Recommended seismic design criteria for new steel moment frame buildings. Prepared by SAC Joint Venture. FEMA-350. Federal Emergency Management Agency. Washington DC. USA.