- Module 3

Member design

Lecture 10Members in compression

- Contents:

1.	Introduction

2.	Stub columns

3.	Slender columns

4.	Relative slenderness

5.	Buckling resistance

6.	Design steps for compression members

7.	References

| - Introduction |  |
| --- | --- |
| The term ‘compression member’ is generally used to describe a structural component which is only subjected to an axial compression force. It can be a column (under special loading conditions) but it generally refers to compressed pin-ended struts found in trusses, lattice girders or bracing members. If the member is subjected to significant bending moments in addition to the axial load, it is called ‘beam-column’. |  |
| This lecture deals with compression members and, therefore, concerns very few real columns because eccentricities of the axial loads and transverse forces are generally not negligible. Nevertheless, the compression member represents an elementary case which leads to the understanding of the compression effects in the study of the beam-column. |  |
| Column buckling is the instability phenomenon which is specific to compression members. The more slender the compression member, the more prone it is to column buckling. |  |
| In any generality, an initially straight compression member may buckle in several ways: |  |
| - Column flexural buckling: the compression member leaves its initially straight position and buckles in one plane by bending about one of the principal axes of the cross-section; - Column torsional buckling: the axis of the compression member remains straight but each section along the member twists about its centroid; - Column torsional-flexural bucking which consists in an interactive buckling associating both above phenomena so that the member exhibits bending and torque deformations. |  |
| Each of above three types of column buckling is characterised by an elastic critical column buckling load designated Ncr for flexural buckling, Ncr,T for torsional buckling and Ncr,TF for torsional-flexural buckling. Depending on both the type and geometry of the cross section, either torsional or torsional-flexural buckling load can be less than flexural buckling load. |  |
| Rolled sections are mostly prone to flexural buckling. Torsional-flexural buckling may be governing the behaviour of open thin-walled sections; that is the result of non coincident centroid and shear centre together with a small torsional rigidity. In contrast, closed cross-sections have an incomparable higher torsional rigidity than open cross-sections: therefore hollow sections fail normally by flexural buckling. |  |
| Generally compression members are designed using mostly rolled sections (preferably I, H but sometimes also angles and channels) and hollow sections. Due to the distribution of material within the cross-section, hollow sections are especially appropriate everywhere compression is present. |  |
| The lecture briefly describes the behaviour of both stocky and slender columns and introduces the flexural buckling curves which are used for the determination of the resistance of compression members. |  |
| - Stub columns |  |
| ‘Stocky columns’ are columns which have a sufficiently low slenderness such that they are unaffected by overall buckling of the member; they are also called ‘stub columns’. In such cases the compression capacity of the member is governed by the compression resistance of the cross-section, as a function of the section classification. |  |
| In compression Class 1, 2, 3 cross-sections are all unaffected by local buckling. Hence the design compression resistance Nc,Rd of a stocky member is taken as the design resistance of its cross-section; the latter is taken as the design plastic resistance Npl,Rd since strain hardening effects are usually disregarded: |  |
|  | Part 1-1 6.2.4(2) |
| In contrast, for Class 4 cross-sections, local buckling in one or more plate elements of the cross-section prevents the attainment of the squash load and thus the design compression resistance of a stocky member is limited to the local buckling resistance of the cross-section: |  |
|  | Part 1-1 6.2.4(2) |
| where Aeff is the effective area of the cross-section relative to axial compression as determined in accordance with clause 6.2.2.5 of EN 1993-1-1 (see also Lecture 7). |  |
| - Slender columns |  |
| Columns which are not stocky are called ‘slender columns’. Depending on their slenderness, ideal columns - i.e. columns without residual stresses, without lack of straightness and perfectly axially loaded - exhibit two different types of behaviour: |  |
| - Columns with high slenderness present a quasi elastic buckling behaviour; - Columns with medium slenderness are very sensitive to the effects of imperfections and there is a significant interaction between yielding and instability. |  |
| As yet stated above, hollow sections are concerned with flexural column buckling only. Therefore only this type of column buckling is commented on in the following. For sake of simplicity the adjective ‘flexural’ will therefore be largely omitted. |  |
| If Lcr is the elastic critical buckling length, the Euler elastic critical load for flexural buckling Ncr, in short the Euler load, is given as: |  |
| It is possible to define the Euler elastic critical buckling stress cr, in short the Euler stress, by dividing the Euler load by the cross-sectional area A: |  |
| By introducing the radius of gyration of the cross section, , and the column slenderness, for the relevant buckling direction, Eq. becomes: |  |
| Plotting the curve cr as a function of  on a graph (Figure 1), with the horizontal line  = fy representing yielding, it is interesting to note the idealised zones where failure occurs respectively by buckling, or by yielding. |  |
| Failure by yielding Failure by yielding Failure by buckling Stress P |  |
| - Behaviour of an ideal compression member |  |
| The intersection point P of these two curves represents the maximum theoretical value of column slenderness for a member being compressed to the yield strength. The abscissa of that point is the column slenderness  at which the Euler stress cr is thus equal to the yield strength fy of the steel; it is given by: |  |
|  | Part 1-1 6.3.1.3 (1) |
| where: |  |
|  | Part 1-1 6.3.1.3 (1) |
| Therefore, the limiting slenderness 1 looks as a kind of material property; it is equal to 93,9 for steel grade S235 / 86,8 for steel grade S275 / 76,4 for steel grade S355 / 70,2 for steel grade S420 and 67,1 for steel grade S460. |  |
| Figure 1 may be redrawn in a dimensionless form, see Figure 2, by dividing the Euler elastic critical buckling stress cr by the yield strength fy and the slenderness  by the limiting slenderness 1. This representation is helpful because a similar plot may then be applied to struts of different slenderness and material strengths. |  |
| P |  |
| - Dimensionless representation of the behaviour of an ideal compression member |  |
| A ‘real’ slender column behaves differently than the ‘ideal’ column considered above. Indeed it fails by inelastic buckling before reaching the Euler load as a consequence of its various imperfections: initial out-of-straightness of the member axis, residual stresses, eccentricity of the applied axial load and material strain-hardening. All these imperfections influence the buckling resistance and, therefore, the ultimate strength of the column. Experimental investigations conducted on real columns give results as shown in Figure 3; the resistance curve of a real column may be understood as a kind of lower bound of the test results. |  |
| The lower bound curve is derived from a statistical analysis of the available test results and represents a safe limit for the permissible loading. Such a curve is known as ‘flexural buckling curve’. |  |
| Compared to the resistance curve of the ‘ideal’ column, the behaviour of the ‘real’ column shows more significant differences in the range of medium slenderness than in the range of large slenderness. |  |
| In the region of medium slenderness, i.e. in the vicinity of the limiting slenderness 1 - the effects of structural imperfections are the largest; the severe interaction between yielding and instability results in a large relative reduction in strength. That is the slenderness range of most columns used in practice. |  |
| In the region of large slenderness, the proneness to elastic instability is so large that there is a little effect of the imperfections. |  |
| Medium slenderness Large slenderness Stress fy Point of contraflexure |  |
| - Real column test results and buckling curves |  |
| Though the distinction between medium and large column slenderness is not so much important, it is generally agreed that the range of large slenderness starts is beyond the point of contra flexure of the resistance curve shown in Figure 3. The ultimate load of columns with large slenderness is not too far from the Euler load Ncr and is thus mostly independent of the yield stress. |  |
| Columns of medium slenderness are those the behaviour of which deviates the most from Euler’s theory. When buckling occurs, some fibres have already reached the yield strength of the material and the ultimate load is not only a function of the slenderness; the larger the imperfections, the larger the difference between the actual behaviour and the ideal one. Out-of-straightness of the member axis and residual stresses are the imperfections which influence the most significantly the ultimate column strength. The distribution and the peak values of the residual stresses within a cross-section depend on several parameters such as both the geometry and massiveness of the section. Residual stresses can be distributed in various ways across the section as shown in Figure 4. Combined with axial stresses they cause early yielding in the cross section. As the yielded fibres have no more axial rigidity, the area still able to resist the axial load is therefore reduced. |  |
| - Residual stress pattern |  |
| Let us assume an initial out-of-straightness of the member axis which is similar to the first buckling mode with a magnitude e0 at mid-span. The axial load applied on the member produces a bending moment giving a maximum bending stress B (see Figure 5a), which superimposes to the residual stress R. The resultant stress distribution is shown in Figure 5b. If B + R is greater than the yield strength fy, a partial plastic redistribution across the section occurs (see Figure 5c). The fibres which have yielded have henceforth a zero axial stiffness; therefore the flexural rigidity decreases with the result of more proneness to buckling. |  |
| - Partially yielded compression member |  |
| - Relative slenderness |  |
| EC 3 defines the dimensionless column slenderness , also termed ‘relative’ column slenderness, as: |  |
|  | Part 1-1 6.3.1.2.(1) |
| Using Eqs. and and substituting in Eq. gives: |  |
| where A = 1 for Class 1, 2, 3 cross-sections (resisting section is the gross section) and A = Aeff / A for Class 4 (resisting section is the effective section). |  |
| - Buckling resistance |  |
| The flexural buckling curves - also designated as the European column buckling curves - are based on the results of more than 1100 tests on various types of members (I, H, T, , , ), with different values of column slenderness  (between 55 and 160). Based on a probabilistic approach using the experimental strength and associated with a theoretical analysis, design buckling curves have been established which describe characteristic column strength as a function of the column slenderness. A half sine-wave geometric imperfection of magnitude equal to 1 / 1000 of the system length of the column and the effects of residual stresses relative to each kind of cross-section are accounted for by these design curves. |  |
| In the region of stocky members the favourable effects of strain hardening makes more than counterbalancing the unfavourable effects of imperfections. Though the experimental strengths are found larger than the section plastic resistance the latter is taken conventionally as the ultimate resistance of stocky members. Stocky members are within the range 0 < ≤ 0,2 or, what is equivalent, NEd / Ncr ≤ 0,04. In this range, the buckling effects may be ignored and only the cross-sectional checks apply. | Part 1-1 6.3.1.2(4) |
| Due to mainly different peak value and distribution of the residual stresses, the behaviour of compression members may vary significantly. In view of not giving an excessive penalty, through the use of a single buckling curve, to those columns being the least affected by the residual stresses, five basic curves have been adopted. |  |
| These buckling curves are respectively designated a0, a, b, c and d; they are shown in Figure 6. Each of them gives, for a specific set of cross-sections, the value of the reduction factor  for flexural column buckling as a function of the relative slenderness of the member. In practice, the value of the reduction factor  may either be read from charts or, better, calculated from the analytical expression of the curves. | Part 1-1 6.3.1.2(3) |
| EC 3 expresses these flexural buckling curves by the mathematical expression: |  |
|  | Part 1-1 6.3.1.2(1) |
| where: |  |
|  | Part 1-1 6.3.1.2(1) |
| Each flexural buckling curve is characterised by a value of the ‘imperfection factor’ ; The larger the  value, the larger the effects of the imperfections and the lower the corresponding flexural buckling curve. The values of  are given in Table 1. | Part 1-1 6.3.1.2(2) |
| - Flexural column buckling curves |  |
| - Values of the imperfection factor |  |
| Buckling curve; a0; a; b; c; d. Imperfection factor ; 0,13; 0,21; 0,34; 0,49; 0,76. | Part 1-1 Table 6.1 |
| How to select the flexural buckling curve appropriate to a given cross section depends on several criteria: fabrication process (rolled or welded for I and H sections, cold-formed or hot-finished for hollow sections), shape of the cross-section, massiveness (measured by the depth-to-width ratio) and flange thickness, the direction in which buckling is considered (bending about major y-y axis or minor z-z axis) and the steel grade. |  |
| This selection process is illustrated in Table 2 for hot-rolled sections and hollow sections. | Table 6.2 (excerpt) |
| It is to be noticed that a more favourable column buckling curve is generally allocated to steel grade S460 because it has been observed that residual stresses are not proportional to the material yield strength and have therefore relatively lower detrimental effects on the ultimate resistance of the member. |  |
| - Selection of appropriate column buckling curve |  |
| The design ultimate strength of the compression member, also termed ‘design buckling resistance’, is then given as: |  |
| where  is the partial safety factor for instability phenomena. In accordance with EN 1993-1-1, the ‘recommended’ values of  are 1,0 for buildings and 1,1 for bridges; however it is up to the National Annexes to specify them. |  |
| When determining the cross-section area A to be used in Eq. , it is agreed that holes for fasteners at the column ends need not to be accounted for. |  |
| It is reminded that only the cross-sectional checks apply when ; as a consequence, when the values of partial safety factors  and  are chosen different, a sudden drop in the design resistance shall be expected when bridging the limit . |  |

| - Design steps for compression members |  |
| --- | --- |
| The verification procedure should then proceed as follows: |  |
| - Determine the buckling lengths Lcr,y and Lcr,z relative to flexural buckling about respectively the major axis y and the minor axis z; |  |
| - Select a steel grade, wherefrom the characteristic yield strength fy; |  |
| - Select a hollow section on the base of a preliminary design or of design experience; |  |
| - Calculate the section properties or draw them from a catalogue of sections: cross-sectional area of the effective section, AA (with (A = 1 for Class 1 or 2 or 3 cross-section and A = Aeff / A for Class 4 cross-section), and second moments of area Iy and Iz of the gross cross section about respectively major axis y-y and minor axis z-z; |  |
| - Calculate the characteristic section resistance: |  |
| - Calculate the elastic critical flexural column buckling loads about both axes: |  |
| - Calculate the relative slenderness values for buckling about both axes: |  |
| - and keep the largest value (Notice that is allowed only because the buckling curve for hollow sections is the same for both directions of buckling as shown in Table 2) - ; |  |
| - Select the appropriate flexural column buckling curve (applicable to both directions of buckling when hollow sections) with regard to both the forming process (hot-finished or cold-formed) and steel grade, and deduce the value of the corresponding imperfection factor; |  |
| - Calculate the reduction factor associated to this buckling curve by using the expressions Eqs. and . |  |
| - Calculate the design buckling resistance Nb,Rd of the compression member in accordance with Eq. . |  |
| Check whether NEd ≤ Nb,Rd in which case the design of the compression member is acceptable; if not, there is a need for adopting a larger cross-section and/or a higher steel grade and for restarting the verification process. |  |

- References

- EC 3: EN 1993-1-1: Eurocode 3: Design of Steel Structures Part 1.1: General rules and rules for buildings. CEN, Brussels, May 2005.

- EC 3: EN 1993-1-8: Eurocode 3: Design of Steel Structures – Part 1-8: Design of Joints. CEN, Brussels, May 2005.

- Structural Stability Research Council: Guide to Stability Design Criteria for Metal Structure, 5th edition. Edited by Galambos T.V., John Wiley & Sons, New York, 1998.

- [4]	Trahair N.S. and Bradford, M.A., The Behaviour and Design of Steel Structures, E&F Spon, 1994.

- [5]	Zhao X-L., Wilkinson T. and Hancock G.: “Cold-Formed Tubular Members and Connections”, Elsevier, 2005.