- Module 5

Design of joints

Lecture 12Generalities on joint design

- Contents:

1.	Introduction

1.1.	General

1.2.	The merits of the consistent approach for structural joints

1.3.	A parallel between member sections and joints

2.	Definitions of joint configuration, joint and connection

3.	Source of joint deformability

3.1.	Beam-to-column joints

3.1.1.	Major axis joints

3.1.2.	Minor axis joints

3.1.3.	Joints with beams on both major and minor column axes

3.2.	Beam splices and column splices

3.3.	Beam-to-beam joints

3.4.	Column bases

4.	Analysis, classification and modelling

4.1.	Global analysis

4.1.1.	General

4.1.2.	Elastic global analysis

4.1.3.	Rigid-plastic global analysis

4.1.4.	Elastic-plastic global analysis

4.1.5.	Global analysis of lattice girders

4.2.	Classification of joints

4.2.1.	General

4.2.2.	Classification by stiffness

4.2.3.	Classification boundaries

4.2.4.	Classification by strength

4.2.5.	Ductility classes

4.3.	Modelling of Joints

4.3.1.	General

4.3.2.	Modelling and sources of joint deformability

4.3.3.	Simplified modelling according to EN 1993-1-8, 5.3 (5)

4.3.4.	Concentration of the joint deformability

5.	Basis of design

5.1.	Assumptions

5.2.	General requirements

5.3.	The applied forces and moments

5.4.	Resistance of joints

5.5.	Design assumptions

5.6.	Joints loaded in shear subject to impact, vibration and/or load reversal

5.7.	Eccentricity at intersections

6.	Connections made with bolts, rivets or pins

6.1.	Categories of bolted connections

6.1.1.	Shear connections

6.1.2.	Tension connections

7.	Welded connections

7.1.	General

7.2.	Welding consumables

7.3.	Geometry and dimensions

7.3.1.	Type of weld

7.4.	Design resistance of a fillet weld

7.4.1.	Length of welds

7.4.2.	Effective throat thickness

7.4.3.	Design Resistance

7.4.4.	Design resistance of fillet welds all round

7.5.	Design resistance of butt welds

8.	Design Resistance of basic components

8.1.	Column web panel in shear

8.2.	Column web in transverse compression

8.3.	Rotational stiffness

8.3.1.	Basic model

8.4.	Rotation capacity

8.4.1.	General

8.4.2.	Bolted joints

8.4.3.	Welded Joints

9.	Hollow section joints

9.1.	General

9.1.1.	Scope

9.1.2.	Field of application

9.2.	Design of hollow section connections

9.2.1.	General

9.2.2.	Failure modes for hollow section connections

10.	References

| - Introduction |  |  |  |  |
| --- | --- | --- | --- | --- |
| - General |  |  |  |  |
| The rotational behaviour of actual joints is well recognised as being often intermediate between the two extreme situations, i.e. rigid or pinned. |  |  |  |  |
| Later in this lecture, the difference between joints and connections will be introduced. For the time being, examples of joints between one beam and one column only will be used. |  |  |  |  |
| Consider now the bending moments and the related rotations at a joint (Figure 1): |  |  |  |  |
| (a) Rigid joint | (b) Pinned joint |  | (c) Semi-rigid joint |  |
| - Classification of joints according to stiffness |  |  |  |  |
| When all the different parts in the joint are sufficiently stiff (i.e. ideally infinitely stiff), the joint is rigid, and there is no difference between the respective rotations at the ends of the members connected at this joint (Figure 1a). The joint experiences a single global rigid-body rotation which is the nodal rotation in the commonly used analysis methods for framed structures. |  |  |  |  |
| Should the joint be without any stiffness, then the beam will behave just as simply supported whatever the behaviour of the other connected member(s) (Figure 1b). This is a pinned joint. |  |  |  |  |
| For intermediate cases (non zero and non infinite stiffness), the transmitted moment will result in there being a difference  between the absolute rotations of the two connected members (Figure 1c). The joint is semi-rigid in these cases. |  |  |  |  |
| The simplest means for representing the concept is a rotational (spiral) spring between the ends of the two connected members. The rotational stiffness S of this spring is the parameter that links the transmitted moment Mj to the relative rotation  which is the difference between the absolute rotations of the two connected members. |  |  |  |  |
| When this rotational stiffness S is zero, or when it is relatively small, the joint falls back into the pinned joint class. In contrast, when the rotational stiffness S is infinite, or when it is relatively high, the joint falls into the rigid joint class. In all the intermediate cases, the joint belongs to the semi-rigid joint class. |  |  |  |  |
| For semi-rigid joints the loads will result in both a bending moment Mj and a relative rotation  between the connected members. The moment and the relative rotation are related through a constitutive law which depends on the joint properties. This is illustrated in Figure 2, where, for the sake of simplicity, the global analysis is assumed to be performed with linear elastic assumptions. |  |  |  |  |
| At the global analysis stage, the effect of having semi-rigid joints instead of rigid or pinned joints is to modify not only the displacements, but also the distribution and magnitude of the internal forces throughout the structure. |  |  |  |  |
| As an example, the bending moment diagrams in a fixed-base simple portal frame subjected to a uniformly distributed load are given in Figure 3 for two situations, where the beam-to-column joints are respectively either pinned or semi-rigid. The same kind of consideration holds for deflections. |  |  |  |  |
| (a) Rigid joint ( = 0) | (b) Pinned joint (Mj = 0) |  | (c) Semi-rigid joint (Mj and   0) |  |
| - Modelling of joints (case of elastic global analysis) |  |  |  |  |
| (a) Pinned joints |  | (b) Semi-rigid joints |  |  |
| - Elastic distribution of bending moments in a simple portal frame |  |  |  |  |
| - The merits of the consistent approach for structural joints |  |  |  |  |
| Both the Eurocode 3 requirements and the desire to model the behaviour of the structure in a more realistic way leads to the consideration of the semi-rigid behaviour when necessary. |  |  |  |  |
| Many designers would stop at that basic interpretation of Eurocode 3 and hence would be reluctant to confront the implied additional computational effort involved. Obviously a crude way to deal with this new burden will be for them to design joints that will actually continue to be classified as being either pinned or fully rigid. However such properties will have to be proven at the end of the design process and, in addition, such joints will certainly be found to be uneconomical in a number of situations. |  |  |  |  |
| It should be noted that the concept of rigid and pinned joints still exists in Eurocode 3. It is accepted that a joint which is almost rigid, or almost pinned, may still be considered as being truly rigid or truly pinned in the design process. How to judge whether a joint can be considered as rigid, semi-rigid or pinned depends on the comparison between the joint stiffness and the beam stiffness, which depends on the second moment of area and length of the beam. |  |  |  |  |
| The designer is strongly encouraged to go beyond this "all or nothing" attitude. Actually it is important to consider the benefits to be gained from the semi-rigid behaviour of joints. Those benefits can be brought in two ways: |  |  |  |  |
| - The designer decides to continue with the practice of assuming -sometimes erroneously- that joints are either pinned or fully rigid. However, Eurocode 3 requires that proper consideration be given to the influence that the actual behaviour of the joints has on the global behaviour of the structure, i.e. on the precision with which the distribution of forces and moments and the displacements have been determined. This may not prove to be easy when the joints are designed at a late stage in the design process since some iterations between global analysis and design checking may be required. Nevertheless, the following situations can be foreseen: |  |  |  |  |
| - So that a joint can be assumed to be rigid, it is common practice to introduce web stiffeners in the column. Eurocode 3 now provides the means to check whether such stiffeners are really necessary for the joint to be both rigid and have sufficient resistance. There are practical cases where they are not needed, thus permitting the adoption of a more economical joint design; |  |  |  |  |
| - When joints assumed to be pinned are later found to have fairly significant stiffness (i.e. to be semi-rigid), the designer may be in a position to reduce beam sizes. This is simply because the moments carried by the joints reduce the span moments in the beams. |  |  |  |  |
| - The designer decides to give consideration, at the preliminary design stage, not only to the properties of the members but also to those of the joints. It may be shown that this new approach is not at all incompatible with the sometimes customary separation of the design tasks between those who have the responsibility for conceiving the structure and carrying out the global analysis and those who have the responsibility for designing the joints. Indeed, both tasks are very often performed by different people, or indeed, by different companies, depending on national or local industrial habits. Adopting this novel early consideration of joints in the design process requires a good understanding of the balance between, on the one hand, the costs and the complexity of joints and, on the other hand, the optimisation of the structural behaviour and performance through the more accurate consideration of joint behaviour for the design as a whole. Two examples are given to illustrate this: |  |  |  |  |
| - It was mentioned previously that it is possible in some situations to eliminate column web stiffeners and therefore to reduce costs. Despite the reduction in its stiffness and, possibly, in its strength, the joint can still be considered to be rigid and be found to have sufficient strength. This is shown to be possible for industrial portal frames with rafter-to-column haunch joints, in particular, but other cases can be envisaged; |  |  |  |  |
| - In a more general way, it is worthwhile to consider the effect of adjusting the joint stiffness so as to strike the best balance between the cost of the joints and the costs of the beams and the columns. For instance, for braced frames, the use of semi-rigid joints, which are probably more costly than the pinned joints, leads to reducing the beam sizes. For unbraced frames, the use of less costly semi-rigid joints, instead of the rigid joints, leads to increased beam sizes and possibly column sizes. |  |  |  |  |
| - Of course the task may seem a difficult one, and this is why the present lectures are aimed at providing the reader with useful information. The whole philosophy could be termed as "Because you must do it, take advantage of it". |  |  |  |  |
| - Thus Eurocode 3 now presents the designer with a choice between a traditionalist attitude, where however something may often be gained, and an innovative attitude, where the most economical result may best be sought. |  |  |  |  |
| - It is important to stress the high level of similarity that exists between the member classification and the joint classification. This topic is addressed in the next section. |  |  |  |  |
| - A parallel between member sections and joints |  |  |  |  |
| - Member cross-section behaviour may be considered through an M –  curve for a simply supported beam loaded at mid span (M = bending-moment at mid-span;  sum of rotations at the span ends). Joint behaviour will be considered through a similar relationship, but with M = Mj being the bending moment transmitted by the joint and  being the relative rotation between the connected member and the rest of the joint. Those relationships have similar shapes as illustrated in Figure 4. |  |  |  |  |
| - To flexural stiffness EI / L and the design resistance Mb.Rd of the member correspond the initial stiffness Sj,ini and the design resistance Mj.Rd of the joint. |  |  |  |  |
| - According to Eurocode 3 member cross-sections are divided into four classes according to their varying ability to resist local instability, when partially or totally subject to compression, and the consequences this may have on the possibility for plastic redistribution. Therefore their resistance ranges from the full plastic resistance (Class 1 and 2) to the elastic resistance (Class 3) or a reduced elastic resistance (Class 4) as explained in the Lecture on "Local buckling and section classification". |  |  |  |  |
| - M –  characteristics for member cross-section and joint |  |  |  |  |
| - The allocation of a cross-section to a specific class is governed by the assumptions on: |  |  |  |  |
| - The behaviour to be idealised for global analysis (i.e. Class 1 will allow the formation of a plastic hinge and permit the redistribution of internal forces in the frame as loads are increased up to or beyond the design loads); |  |  |  |  |
| - The behaviour to be taken into account for local design checks (i.e. Class 4 will imply that the resistance of the cross-section is based on the properties of a relevant effective cross-section rather than of the gross cross-section). |  |  |  |  |
| - In Eurocode 3, the classification of a cross-section is based on the width-to-thickness ratio of the component walls of the section. Ductility is directly related to the amount of rotation during which the design bending resistance will be sustained. For joints, the rotation capacity concept is equivalent to the ductility concept for sections. |  |  |  |  |
| - In a manner similar to that for member cross-sections, joints are classified in terms of ductility or rotation capacity. This classification is a measure of their ability to resist premature local instability and, even more likely, premature brittle failure (especially due to bolt failure) with due consequences on the type of global analysis allowed. |  |  |  |  |
| - The practical interest of such a classification for joints is to check whether an elastic-plastic global analysis may be conducted up to the formation of a plastic collapse mechanism in the structure, which implies such hinges in at least some of the joints. |  |  |  |  |
| - Ductility or rotation capacity in joints |  |  |  |  |
| - As will be shown, this classification of joints by ductility, while not explicitly stated in Eurocode 3, may be defined from the geometric and mechanical properties of the joint components (bolts, welds, plate thickness, etc.). |  |  |  |  |
| - Joints may therefore be classified according to both their stiffness and their ductility. Moreover, joints may be classified according to their strength. |  |  |  |  |
| - In terms of their strength, joints are classified as full-strength or partial-strength according to their resistance compared to the resistance of the connected members. For elastic design, the use of partial-strength joints is well understood. When plastic design is used, the main use of this classification is to foresee the possible need to allow a plastic hinge to form in the joint during the global analysis. In order to permit a further increase of loads beyond that corresponding to the formation of the hinge, a partial-strength joint may be required to act as a hinge from the moment when its plastic bending resistance is reached. In that case, the joint must also have sufficient ductility. |  |  |  |  |

| - Definitions of joint configuration, joint and connection |  |
| --- | --- |
| Building frames consist of beams and columns, usually made of H or I shapes that are assembled together by means of connections. These connections are between two beams (Figure 6; Point B) , two columns (Figure 6; Point C), a beam and a column (Figure 6; Point A) or a column and the foundation (Figure 6; Point D). |  |
| - Different types of connections in a building frame |  |
| A connection is defined as the set of the physical components which mechanically fasten the connected elements. One considers the connection to be concentrated at the location where the fastening action occurs, for instance at the beam end/column interface in a major axis beam-to-column joint. When the connection as well as the corresponding zone of interaction between the connected members are considered together, the wording joint is then used (Figure 7a). |  |
| - Joints and connections |  |
| Depending on the number of in-plane elements connected together, single-sided and double-sided joint configurations are defined (Figure 8). In a double-sided configuration (Figure 8b), two joints – left and right – have to be considered (Figure 7b.). |  |
| The definitions illustrated in Figure 7 and Figure 8 are valid for other joint configurations and connection types. |  |
| - In-plane joint configurations |  |
| As explained previously, the joints which are traditionally considered as rigid or pinned and are designed accordingly, possess, in reality, their own degree of finite flexibility or finite stiffness resulting from the deformability of all the constitutive components. The next section is aimed at describing the main joint deformability sources. |  |
| - Source of joint deformability |  |
| The rotational behaviour of the joints may affect the local and/or global structural response of the frames. In this section, the sources of rotational deformability are identified for beam-to-column joints, splices and column bases. |  |
| It is worthwhile mentioning that the rotational stiffness, the joint resistance and the rotation capacity are likely to be affected by the shear force and/or the axial force acting in the joint. |  |
| These shear and axial forces may obviously have contributions to the shear and axial deformability within the connections. However it is known that these contributions do not affect significantly the frame response; therefore, the shear and axial responses of the connection, in terms of rotational deformability, are neglected. |  |

| - Beam-to-column joints |  |
| --- | --- |
| - Major axis joints |  |
| In a major axis beam-to-column joint, different sources of deformability can be identified. For the particular case of a single-sided joint (Figure 9.a and Figure 10.a), these are: |  |
| - The deformation of the connection. This includes the deformation of the connection elements : column flange, bolts, end-plate or angles,... and the load-introduction deformation of the column web resulting from the transverse shortening and elongation of the column web under the compressive and tensile forces Fb acting on the column web. The couple of Fb forces are statically equivalent to the moment Mb at the beam end. These deformations result in a relative rotation c between the beam and column axes; this rotation, which is equal to b - c (see Figure 9.a) is concentrated mainly along edge AB and provides a flexural deformability curve Mb – c; |  |
| - The shear deformation of the column web panel associated with the shear force Vwp acting in this panel. This leads to a relative rotation  between the beam and column axes; this rotation makes it possible to establish a shear deformability curve Vwp – ; |  |
| - The deformability curve of a connection may obviously be influenced by the axial and shear forces possibly acting in the connected beam; |  |
| - Similar definitions apply to double-sided joint configurations (Figure 9b and Figure 10b). For such configurations, two connections and a sheared web panel, forming two joints, must be considered. |  |
| In short, the main sources of deformability which must be contemplated in a beam-to-column major axis joint are: |  |
| - Single-sided joint configuration: |  |
| - The connection deformability Mb – c characteristic; |  |
| - The column web panel shear deformability Vwp –  characteristic. |  |
| - Double-sided joint configuration: |  |
| - The left hand side connection deformability Mb1 – c1 characteristic; |  |
| - The right hand side connection deformability Mb2 – c2 characteristic; |  |
| - The column web panel shear deformability Vwp –  characteristic. |  |
| (a) Single-sided joint configuration (b) Double-sided joint configuration |  |
| - Sources of joint deformability |  |
| The deformability of the connection (connection elements + load-introduction) is only due to the couple of forces transferred by the flanges of the beam (equivalent to the beam end moment Mb). The shear deformability of the column web panel results from the combined action of these equal but opposite forces and of the shear forces in the column at the level of the beam flanges. Equilibrium equations of the web panel provide the shear force Vwp (see Figure 10 for the sign convention): |  |
| Another formula to which it is sometimes referred, i.e.: |  |
| is only a rough and conservative approximation of Eq. . |  |
| In both formulae, z is the lever arm of the resultant tensile and compressive forces in the connection(s). |  |
| - Loading of the web panel and the connections |  |
| - Minor axis joints |  |
| A similar distinction between the web panel and the connection shall also be made for a minor axis joint (Figure 11). The column web exhibits a so-called out-of-plane deformability while the connection deforms in bending as it does in a major axis joint. However no load-introduction deformability is involved. |  |
| In a double-sided joint configuration, the out-of-plane deformation of the column web depends on the bending moments experienced by the right and left connections (see Figure 12): |  |
| For a single-sided joint configuration (Figure 11), the value of Mb equals that of Mb. |  |
| - Deformability of a minor axis joint |  |
| - Loading of a double-sided minor axis joint |  |
| - Joints with beams on both major and minor column axes |  |
| A 3-D joint (Figure 13) is characterised by the presence of beams connected to both the column flange(s) and web. In such joints, a shear deformation and an out-of-plane deformation of the column web develop coincidently. |  |
| The loading of the web panel appears therefore as the superposition of the shear loading given by Eqs or and the out-of-plane loading given by Eq. . |  |
| The joint configuration of Figure 13 involves two beams only; configurations with three or four beams can also be met. |  |
| - Example of a 3-D joint |  |
| - Beam splices and column splices |  |
| The sources of deformability in a beam splice (Figure 14) or in a column splice (Figure 15) are less than in a beam-to-column joint; indeed they are concerned with connections only. The deformability is depicted by the sole Mb –  curve. |  |
| - Deformation of a beam splice |  |
| The single Mb –  curve corresponds to the deformability of the whole joint, i.e. the two constituent connections (left and right connections in a beam splice, upper and lower connections in a column splice). |  |
| In a column splice where the compressive force is predominant, the axial force affects the mechanical properties of the joint, i.e. its rotational stiffness, its strength and its rotation capacity, in a significant way. The influence of the axial deformability of splices on the global frame response is however limited and is therefore neglected. |  |
| - Deformation of a column splice |  |
| - Beam-to-beam joints |  |
| The deformability of a beam-to-beam joint (Figure 16) is quite similar to the one of a minor axis beam-to-column joint; the loadings and the sources of deformability are similar to those expressed earlier for minor axis joints. |  |
| - Deformation of a beam-to-beam joint |  |
| - Column bases |  |
| In a column base, two connection deformabilities need to be distinguished (Figure 17). |  |
| - The deformability of the connection between the column and the concrete foundation (column-to-concrete connection); |  |
| - The deformability of the connection between the concrete foundation and the soil (concrete-to-soil connection). |  |
| For the column-to-concrete connection, the bending behaviour is represented by a Mc –  curve, the shape of which is influenced by the ratio of the bending moment to the axial load at the bottom of the column. |  |
| For the connection between the concrete foundation and the soil, two basic deformability curves are identified: |  |
| - a Nc – u curve which corresponds to the soil settlement due to the axial compressive force in the column; in contrast with the other types of joint, this deformability curve may have a significant effect on the frame behaviour; |  |
| - a Mc –  curve characterising the rotation of the concrete block in the soil. |  |
| As for all the other joints described above, the deformability of the column base due to the shear force in the column may be neglected. |  |
| The column-to-concrete connection and concrete-to-soil connection Mc –  characteristics are combined in order to derive the rotational stiffness at the bottom of the column and conduct the frame analysis and design accordingly. |  |
| Similar deformability sources exist in column bases subjected to biaxial bending and axial force. The connection Mc –  characteristics are then defined respectively for both the major and the minor axes. |  |
| - The connections in a column base |  |

| - Analysis, classification and modelling |  |  |  |
| --- | --- | --- | --- |
| - Global analysis |  |  |  |
| - General |  |  |  |
| The effects of the behaviour of the joints on the distribution of internal forces and moments within a structure, and on the overall deformations of the structure, should generally be taken into account. Where these effects are sufficiently small they can be neglected. |  |  |  |
| To identify whether the effects of joint behaviour on the analysis need be taken into account, a distinction may be made between three simplified joint models as follows: |  |  |  |
| - Simple, in which the joint may be assumed not to transmit bending moments; - Continuous, in which the behaviour of the joint may be assumed to have no effect on the analysis; - Semi-continuous, in which the behaviour of the joint needs to be taken into account in the analysis. |  |  |  |
| The appropriate type of joint model should be determined from Table 1, depending on the classification of the joint and on the chosen method of analysis. The design moment-rotation characteristic of a joint used in the analysis may be simplified by adopting any appropriate curve, including a linearised approximation (e.g. bi-linear or tri-linear), provided that the approximate curve lies wholly below the design moment-rotation characteristic. |  |  |  |
| - Type of joint model |  |  |  |
| - Elastic global analysis |  |  |  |
| The joints should be classified according to their rotational stiffness, see EN 1993-1-8, 5.2.2. The joints shall have sufficient strength to transmit the forces and moments acting at the joints resulting from the analysis. In the case of a semi-rigid joint, the rotational stiffness Sj corresponding to the bending moment Mj,Ed should generally be used in the analysis. If Mj,Ed does not exceed 2 / 3 Mj,Rd the initial rotational stiffness Sj,ini may be taken in the global analysis, see Figure 18. As a simplification to EN 1993-1-8, 5.1.2(3), the rotational stiffness may be taken as Sj,ini /  in the analysis, for all values of the moment Mj,Ed, as shown in Figure 18 where  is the stiffness modification coefficient from Table 2. For joints connecting H or I sections Sj is given in EN 1993-1-8, 6.3.1. |  |  |  |
| - Rotational stiffness to be used in elastic global analysis |  |  |  |
| - Stiffness modification coefficient  |  |  |  |
| - Rigid-plastic global analysis |  |  |  |
| The joints should be classified according to their strength, see EN 1993-1-8, 5.2.3. For joints connecting H or I sections Mj,Rd is given in EN 1993-1-8, 6.2. For joints connecting hollow sections the method given in section 7 of EN 1993-1-8 may be used. The rotation capacity of a joint shall be sufficient to accommodate the rotations resulting from the analysis. For joints connecting H or I sections the rotation capacity should be checked according to EN 1993-1-8, 6.4. |  |  |  |
| - Elastic-plastic global analysis |  |  |  |
| The joints should be classified according to both stiffness (see EN 1993-1-8, 5.2.2) and strength (see EN 1993-1-8, 5.2.3). For joints connecting H or I sections Mj,Rd is given in EN 1993-1-8, 6.2, Sj is given in EN 1993-1-8, 6.3.1 and Cd is given in EN 1993-1-8, 6.4. For joints connecting hollow sections the method given in section 7 of EN 1993-1-8 may be used. The moment rotation characteristic of the joints should be used to determine the distribution of internal forces and moments. |  |  |  |
| As a simplification, the bi-linear design moment-rotation characteristic shown in Figure 19 may be adopted. The stiffness modification coefficient  should be obtained from Table 2. |  |  |  |
| - Simplified bi-linear design moment-rotation characteristic |  |  |  |
| - Global analysis of lattice girders |  |  |  |
| The provisions given in EN 1993-1-8, 5.1.5 apply only to structures whose joints are verified according to section 7 of EN 1993-1-8. |  |  |  |
| The distribution of axial forces in a lattice girder may be determined on the assumption that the members are connected by pinned joints (see also 2.7). |  |  |  |
| Secondary moments at the joints, caused by the rotational stiffness of the joints, may be neglected both in the design of the members and in the design of the joints, provided that both of the following conditions are satisfied: |  |  |  |
| - The joint geometry is within the range of validity specified in EN 1993-1-8, Table 7.1, Table 7.8, Table 7.9 or Table 7.20 as appropriate; - The ratio of the system length to the depth of the member in the plane of the lattice girder is not less than the appropriate minimum value. For building structures, the appropriate minimum value may be assumed to be 6. Larger values may apply in other parts of EN 1993. |  |  |  |
| The moments resulting from transverse loads (whether in-plane or out-of-plane) that are applied between panel points, should be taken into account in the design of the members to which they are applied. Provided that the conditions given in EN 1993-1-8, 5.1.5 are satisfied: |  |  |  |
| - The brace members may be considered as pin-connected to the chords, so moments resulting from transverse loads applied to chord members need not be distributed into brace members, and vice versa; - The chords may be considered as continuous beams, with simple supports at panel points. |  |  |  |
| Moments resulting from eccentricities may be neglected in the design of tension chord members and brace members. They may also be neglected in the design of connections if the eccentricities are within the following limits: |  |  |  |
| where e = eccentricity defined in Figure 20; d0 = diameter of the chord; h0 = depth of the chord, in the plane of the lattice girder. |  |  |  |
| When the eccentricities are within the limits mentioned above, the moments resulting from the eccentricities should be taken into account in the design of compression chord members. In this case the moments produced by the eccentricity should be distributed between the compression chord members on each side of the joint, on the basis of their relative stiffness coefficients I / L, where L is the system length of the member, measured between panel points. |  |  |  |
| When the eccentricities are outside the limits mentioned above (Eqs. and ), the moments resulting from the eccentricities should be taken into account in the design of the connections and the compression chord members. In this case the moments produced by the eccentricity should be distributed between all the members meeting at the joint, on the basis of their relative stiffness coefficients I / L. |  |  |  |
| The stresses in a chord resulting from moments taken into account in the design of the chord, should also be taken into account in determining the factors km, kn and kp used in the design of the connections, see EN 1993-1-8, Table 7.2 to Table 7.5, Table 7.10 and Table 7.12 to Table 7.14. The cases where moments should be taken into account are summarized in Table 3. |  |  |  |
| - Eccentricity of joint |  |  |  |
| - Allowance for bending moments |  |  |  |
| - Classification of joints |  |  |  |
| - General |  |  |  |
| Later in this lecture, it is shown that the joints need to be modelled for the global frame analysis and that three different types of joint modelling are introduced: simple, semi-continuous and continuous. |  |  |  |
| It will also been explained that the type of joint modelling to be adopted is dependent both on the type of frame analysis and on the class of the joint in terms of stiffness and/or strength. |  |  |  |
| Classification criteria are used to perform this task and these are described below. |  |  |  |
| The details of all joints shall fulfil the assumptions made in the relevant design method, without adversely affecting any other part of the structure. Joints may be classified by their stiffness (see EN 1993-1-8, 5.2.2) and by their strength (see EN 1993-1-8, 5.2.3). |  |  |  |
| - Classification by stiffness |  |  |  |
| A joint may be classified as rigid, nominally pinned or semi-rigid according to its rotational stiffness, by comparing its initial rotational stiffness Sj,ini with the classification boundaries (Figure 21) given in EN 1993-1-8, 5.2.2.5. Rules for the determination of Sj,ini for joints connecting H or I sections are given in EN 1993-1-8, 6.3.1. Rules for the determination of Sj,ini for joints connecting hollow sections are not given in this Standard. A joint may be classified on the basis of experimental evidence, experience of previous satisfactory performance in similar cases or by calculations based on test evidence. For sake of simplicity, the stiffness boundaries (Figure 21) have been derived so as to allow a direct comparison with the initial design joint stiffness, whatever the type of joint idealisation that is used afterwards in the analysis. |  |  |  |
| ### Nominally pinned joints A nominally pinned joint shall be capable of transmitting the internal forces, without developing significant moments which might adversely affect the members or the structure as a whole. A nominally pinned joint shall be capable of accepting the resulting rotations under the design loads. |  |  |  |
| ### Rigid joints Joints classified as rigid may be assumed to have sufficient rotational stiffness to justify analysis based on full continuity. |  |  |  |
| ### Semi-rigid joints A joint which does not meet the criteria for a rigid joint or a nominally pinned joint should be classified as a semi-rigid joint. Semi-rigid joints provide a predictable degree of interaction between members, based on the design moment-rotation characteristics of the joints. Semi-rigid joints should be capable of transmitting the internal forces and moments. |  |  |  |
| - Classification boundaries |  |  |  |
| It is while stressing that a classification based on the experimental joint M –  characteristics is not allowed, as design properties only are of concern. |  |  |  |
| The stiffness and strength boundaries for the joint classification are given as follows: |  |  |  |
|  | - Rigid if Sj,ini ≥ kb EIb / Lb where kb = 8 for frames where the bracing system reduces the horizontal displacement by at least 80%, kb = 25 for other frames, provided that in every storey Kb / Kc ≥ 0,1*) - Semi-rigid, all joints in zone 2 should be classified as semi-rigid. Joints in zones 1 or 3 may optionally also be treated as semi-rigid. - Nominally pinned if Sj,ini ≤ 0,5 EIb / Lb - *) For frames where Kb / Kc < 0,1, the joints should be classified as semi-rigid. |  |  |
| - Classification of joints by stiffness |  |  |  |
| Where Kb = the mean value of Ib / Lb for all the beams at the top of that storey; Kc = the mean value of Ic / Lc for all the columns in that storey; Ib = the second moment of area of a beam; Ic = the second moment of area of a column; Lb = the span of a beam (centre-to-centre of columns); Lc = the storey height of a column. |  |  |  |
| Column bases may be classified as rigid provided the following conditions are satisfied: |  |  |  |
| - In frames where the bracing system reduces the horizontal displacement by at least 80% and where the effects of deformation may be neglected; |  |  |  |
| If |  |  |  |
| If and |  |  |  |
| If and |  |  |  |
| - Otherwise if Sj,ini ≥ 30 EIc / Lc. |  |  |  |
| where is the slenderness of a column in which both ends are assumed to be pinned. |  |  |  |
| - Classification by strength |  |  |  |
| A joint may be classified as full-strength, nominally pinned or partial strength by comparing its design moment resistance Mj,Rd with the design moment resistances of the members that it connects (Figure 22). When classifying joints, the design resistance of a member should be taken as that member adjacent to the joint. |  |  |  |
| - Strength classification boundaries |  |  |  |
| Classification by strength: |  |  |  |
| - Full strength joints |  |  |  |
| Where Mb,pl,Rd is the design plastic moment resistance of a beam, Mc,pl,Rd is the design plastic moment resistance of a column. |  |  |  |
| Full-strength joint: |  |  |  |
| The design resistance of a full strength joint shall be not less than that of the connected members. A joint may be classified as full-strength if it meets the criteria given in Figure 23. |  |  |  |
| Partial strength joint: |  |  |  |
| A joint which does not meet the criteria for a full-strength joint or a nominally pinned joint should be classified as a partial-strength joint. |  |  |  |
| Pinned joint: |  |  |  |
| A nominally pinned joint shall be capable of transmitting the internal forces, without developing significant moments which might adversely affect the members or the structure as a whole. A nominally pinned joint shall be capable of accepting the resulting rotations under the design loads. A joint may be classified as nominally pinned if its design moment resistance Mj,Rd is not greater than 0,25 times the design moment resistance required for a full-strength joint, provided that it also has sufficient rotation capacity. |  |  |  |
| - Ductility classes |  |  |  |
| Experience and proper detailing result in so-called pinned joints which exhibit a sufficient rotation capacity to sustain the rotations imposed on them. |  |  |  |
| For moment resisting joints the concept of ductility classes is introduced to deal with the question of rotation capacity. |  |  |  |

| - Modelling of Joints |  |  |
| --- | --- | --- |
| - General |  |  |
| Joint behaviour affects the structural frame response and shall therefore be modelled, just as for beams and columns, for the frame analysis and design. Traditionally, the following types of joint modelling are considered: |  |  |
| For rotational stiffness: | For resistance: |  |
| - Rigid; | - Full-strength; |  |
| - Pinned. | - Partial-strength; |  |
|  | - Pinned. |  |
| When the joint rotational stiffness is of concern, the wording rigid means that no relative rotation occurs between the connected members whatever be the applied moment. The wording pinned postulates the existence of a perfect (i.e. frictionless) hinge between the members. In fact these definitions may be relaxed, as explained earlier. Indeed rather flexible but not fully pinned joints and rather stiff but not fully rigid joints may be considered as effectively pinned and sufficiently rigid respectively. The stiffness boundaries allowing one to classify joints as rigid or pinned have been examined. |  |  |
| For joint resistance, a full-strength joint is stronger than the weaker of the connected members, which is in contrast to a partial-strength joint. In the everyday practice, partial-strength joints are used whenever the joints are designed to transfer the internal forces but not to resist the full capacity of the connected members. A pinned joint is considered to transfer no moment. Related classification criteria have also been presented. |  |  |
| Consideration of rotational stiffness and joint resistance properties leads to three significant joint models: |  |  |
| - Rigid/full-strength; |  |  |
| - Rigid/partial-strength; |  |  |
| - Pinned. |  |  |
| However, as far as the joint rotational stiffness is considered, joints designed for economy may be neither rigid nor pinned but semi-rigid. There are thus new possibilities for joint modelling: |  |  |
| - Semi-rigid/full-strength; |  |  |
| - Semi-rigid/partial-strength. |  |  |
| With a view to simplification, Eurocode 3 Part 1-8 account for these possibilities by introducing three joint models (Table 4): |  |  |
| - Continuous: covering the rigid/full-strength case only; |  |  |
| - Semi-continuous: covering the rigid/partial-strength, the semi- rigid/full-strength and the semi-rigid/partial- strength cases; |  |  |
| - Simple: covering the pinned case only. |  |  |
| - Types of joint modelling |  |  |
| The following meanings are given to these terms: |  |  |
| - Continuous: the joint ensures a full rotational continuity between the connected members; |  |  |
| - Semi-continuous: the joint ensures only a partial rotational continuity between connected members; |  |  |
| - Simple: the joint prevents from any rotational continuity between the connected members. |  |  |
| The interpretation to be given to these wordings depends on the type of frame analysis to be performed. In the case of an elastic global frame analysis, only the stiffness properties of the joint are relevant for the joint modelling. In the case of a rigid-plastic analysis, the main joint feature is the resistance. In all the other cases, both the stiffness and resistance properties govern the manner in which the joints should be modelled. These possibilities are illustrated in Table 5. |  |  |
| - Joint modelling and frame analysis |  |  |
| - Modelling and sources of joint deformability |  |  |
| The difference between the loading of the connection and that of the column web in a beam-to-column joint requires, from a theoretical point of view, that account be taken separately of both deformability sources when designing a building frame. |  |  |
| However doing so is only feasible when the frame is analysed by means of a sophisticated computer program which enables a separate modelling of both deformability sources. For most available software, the modelling of the joints has to be simplified by concentrating the sources of deformability into a single rotational spring located at the intersection of the axes of the connected members. |  |  |
| - Simplified modelling according to EN 1993-1-8, 5.3 (5) |  |  |
| As a simplified alternative, a single-sided joint configuration may be modelled as a single joint, and a double-sided joint configuration may be modelled as two separate but inter-acting joints, one on each side. As a consequence a double-sided beam-to-column joint configuration has two moment-rotation characteristics, one for the right-hand joint and another for the left-hand joint. |  |  |
| - Simplified modelling for joints |  |  |
| - Concentration of the joint deformability |  |  |
| For normal practice taking account of both the flexural behaviour of the connection and the shear (major axis beam-to-column joint) separately or out-of-plane behaviour of the column web panel (minor axis beam-to-column joint configurations or beam-to-beam configurations) is not feasible. This section is aimed at explaining how to concentrate the two deformabilities into a single flexural spring located at the intersection of the axes of the connected members. |  |  |
| ### Major axis beam-to-column joint configurations |  |  |
| In a single-sided configuration, only one joint is concerned. The characteristic shear-rotation deformability curve of the column web panel (Figure 25b) is first transformed into a Mb –  curve through the use of the transformation parameter  This parameter, defined in Figure 26a, relates the web panel shear force to the (load-introduction) compressive and tensile forces connection. |  |  |
| The Mb –  spring characteristic which represents the joint behaviour is shown in Figure 25c; it is obtained by summing the contributions of rotation, from the connection (c) and from the shear panel (). The Mj –  characteristic of the joint rotational spring located at the beam-to-column interaction is assumed to identify itself to the Mb – - characteristic obtained as indicated in Figure 25c. |  |  |
| - Flexural characteristic of the spring |  |  |
| - Definition of the transformation parameter  for major axis joints |  |  |
| In a double-sided configuration, two joints - the left one and the right one – are involved. The derivation of their corresponding deformability curves is conducted similarly as in a single-sided configuration by using transformation parameters 1 and 2 Figure 26b). |  |  |
| Because the values of the  parameters can only be determined once the internal forces are known, their accurate determination requires an iterative process in the global analysis. For practical applications, such an iterative process is hardly acceptable provided safe  values are available. These values may be used a priori to model the joints and, on the basis of such joint modelling, the frame analysis may be performed safely in a non-iterative way. |  |  |
| The recommended but approximate values of , where 1 is taken as equal to 2 for double-sided configurations, are given in EN 993-1-8, Table 5.4. They vary from  = 0 (double-sided joint configuration with balanced moments in the beams) to  = 2 (double-sided joint configuration with equal but unbalanced moments in the beams). These two extreme cases are illustrated in Figure 27. |  |  |
| - Extreme cases for  values |  |  |
| - Def. of the transformation parameter  for minor axis joints |  |  |
| Minor axis beam-to-column joint configurations and beam-to-beam configurations |  |  |
| Similar concepts as those developed before are referred to for minor axis beam-to-column joint configurations and beam-to-beam configurations. The definition of the transformation parameter is somewhat different (see Figure 28). |  |  |
| Approximate values of  (assuming 1 = 2) for these joint configurations may also be recommended, in a similar way than what is done for major axis beam-to-column joint configurations. |  |  |
| - Basis of design |  |  |
| - Assumptions |  |  |
| The design methods given in EN 1993-1-8 assume that the standard of construction is as specified in the execution standards given in EN 1993-1-8, 1.2 and that the construction materials and products used are those specified in EN 1993 or in the relevant material and product specifications (e.g. EN 10210 or 10219). |  |  |
| - General requirements |  |  |
| All joints shall have a design resistance such that the structure is capable of satisfying all the basic design requirements given in EN 1993-1-8 and EN 1993-1-1. |  |  |
| The partial safety factors M for joints are given in Table 6. |  |  |
| - Partial safety factors for joints |  |  |
| Values for γM may be defined in the National Annex. Recommended values are as follows: γM2 = 1,25; γM3 = 1,25 for hybrid connections or connections under fatigue loading and γM3 = 1,1 for other design situations; γM4 = 1,0; γM5 = 1,0; γM6,ser = 1,0 and γM7 = 1,1. |  |  |
| - The applied forces and moments |  |  |
| The forces and moments applied to joints at the ultimate limit state shall be determined according to the principles in EN 1993-1-1. |  |  |
| - Resistance of joints |  |  |
| The resistance of a joint shall be determined on the basis of the resistances of its basic components. Linear-elastic or elastic-plastic analysis may be used in the design of joints. |  |  |
| Where fasteners with different stiffness are used to carry a shear load the fasteners with the highest stiffness should be designed to carry the design load. |  |  |
| - Design assumptions |  |  |
| Joints shall be designed on the basis of a realistic assumption of the distribution of internal forces and moments. The following assumptions should be used to determine the distribution of forces: |  |  |
| - The internal forces and moments assumed in the analysis are in equilibrium with the forces and moments applied to the joints; - Each element in the joint is capable of resisting the internal forces and moments; - The deformations implied by this distribution do not exceed the deformation capacity of the fasteners or welds and the connected parts; - The assumed distribution of internal forces shall be realistic with regard to relative stiffness within the joint; - The deformations assumed in any design model based on elastic-plastic analysis are based on rigid body rotations and/or in-plane deformations which are physically possible; - Any model used is in compliance with the evaluation of test results (see EN 1990). |  |  |
| - Joints loaded in shear subject to impact, vibration and/or load reversal |  |  |
| Where a joint loaded in shear is subject to impact or significant vibration one of the following jointing methods should be used: |  |  |
| - Welding; - Bolts with locking devices; - Preloaded bolts; - Injection bolts; - Other types of bolt which effectively prevent movement of the connected parts; - Rivets. |  |  |
| - Eccentricity at intersections |  |  |
| Where there is eccentricity at intersections, the joints and members should be designed for the resulting moments and forces, except in the case of particular types of structures where it has been demonstrated that it is not necessary, see EN 1993-1-8, 5.1.5. |  |  |
| In the case of joints of angles or tees attached by either a single line of bolts or two lines of bolts any possible eccentricity should be taken into account in accordance with EN 1993-1-8, 2.7(1). In-plane and out-of-plane eccentricities should be determined by considering the relative positions of the centroidal axis of the member and of the setting out line in the plane of the connection (Figure 29). For a single angle in tension connected by bolts on one leg the simplified design method given in EN 1993-1-8, 3.10.3 may be used. |  |  |
| - Setting out lines |  |  |
| Where 1 = Centroidal axes, 2 = Fasteners, 3 = Setting out lines |  |  |
| - Connections made with bolts, rivets or pins |  |  |
| All bolts, nuts and washers should comply with EN 1993-1-8, 1.2.4 Reference Standards: Group 4. The rules in this Standard are valid for the bolt classes given in Table 7. The yield strength fyb and the ultimate tensile strength fub for bolt classes 4.6, 5.6, 6.8, 8.8 and 10.9 are given in Table 7. These values should be adopted as characteristic values in design calculations. |  |  |
| - Nominal values of the yield strength and the ultimate tensile strength for bolts |  |  |
| - Categories of bolted connections |  |  |
| - Shear connections |  |  |
| Bolted connections loaded in shear should be designed as one of the following: |  |  |
| ### Category A: Bearing type In this category, bolts from class 4.6 up to and including class 10.9 should be used. No preloading and special provisions for contact surfaces are required. The design ultimate shear load should not exceed the design shear resistance, obtained from EN 1993-1-8, 3.6, nor the design bearing resistance, obtained from EN 1993-1-8, 3.6 and 3.7. |  |  |
| ### Category B: Slip-resistant at serviceability limit state In this category preloaded bolts in accordance with EN 1993-1-8, 3.1.2(1) should be used. Slip should not occur at the serviceability limit state. The design serviceability shear load should not exceed the design slip resistance, obtained from EN 1993-1-8, 3.9. The design ultimate shear load should not exceed the design shear resistance, obtained from EN 1993-1-8, 3.6, nor the design bearing resistance, obtained from EN 1993-1-8, 3.6 and 3.7. |  |  |
| ### Category C: Slip-resistant at ultimate limit state In this category preloaded bolts in accordance with EN 1993-1-8, 3.1.2(1) should be used. Slip should not occur at the ultimate limit state. The design ultimate shear load should not exceed the design slip resistance, obtained from EN 1993-1-8, 3.9, nor the design bearing resistance, obtained from EN 1993-1-8, 3.6 and 3.7. In addition for a connection in tension, the design plastic resistance of the net cross-section at bolt holes Nnet,Rd, (see EN 1993-1-1, 6.2), should be checked, at the ultimate limit state. |  |  |
| - Tension connections |  |  |
| Bolted connection loaded in tension should be designed as one of the following: |  |  |
| ### Category D: non-preloaded In this category bolts from class 4.6 up to and including class 10.9 should be used. No preloading is required. This category should not be used where the connections are frequently subjected to variations of tensile loading. However, they may be used in connections designed to resist normal wind loads. |  |  |
| ### Category E: preloaded In this category preloaded 8.8 and 10.9 bolts with controlled tightening in conformity with EN 1993-1-8, 1.2.7 Reference Standards: Group 7 should be used. |  |  |
| - Categories of bolted connections |  |  |
| - Welded connections |  |  |
| - General |  |  |
| The provisions in this section apply to weldable structural steels conforming to EN 1993-1-1 and to material thicknesses of 4 mm and over. The provisions also apply to joints in which the mechanical properties of the weld metal are compatible with those of the parent metal. For welds in thinner material reference should be made to EN 1993-1-3 and for welds in structural hollow sections in material thicknesses of 2.5 mm and over guidance is given section 7 of EN 1993-1-8. |  |  |
| For stud welding reference should be made to EN 1994-1-1. Further guidance on stud welding can be found in EN ISO 14555 and EN ISO 13918. |  |  |
| Welds subject to fatigue shall also satisfy the principles given in EN 1993-1-9. |  |  |
| Quality level C according to EN ISO 5817 is usually required, if not otherwise specified. The frequency of inspection of welds should be specified in accordance with the rules in EN 1993-1-8, 1.2.7 Reference Standards: Group 7. The quality level of welds should be chosen according to EN ISO 5817. For the quality level of welds used in fatigue loaded structures, see EN 1993-1-9. |  |  |
| Lamellar tearing shall be avoided. Guidance on lamellar tearing is given in EN 1993-1-10. |  |  |
| - Welding consumables |  |  |
| All welding consumables should conform to the relevant standards specified in EN 1993-1-8, 1.2.5 Reference Standards; Group 5. The specified yield strength, ultimate tensile strength, elongation at failure and minimum Charpy V-notch energy value of the filler metal, should be equivalent to, or better than that specified for the parent material. Generally it is safe to use electrodes that are overmatched with regard to the steel grades being used. |  |  |
| - Geometry and dimensions |  |  |
| - Type of weld |  |  |
| This Standard covers the design of fillet welds, fillet welds all round, butt welds, plug welds and flare groove welds. Butt welds may be either full penetration butt welds or partial penetration butt welds. Both fillet welds all round and plug welds may be either in circular holes or in elongated holes. The most common types of joints and welds are illustrated in DIN EN ISO 17659. |  |  |
| ### Fillet welds Fillet welds may be used for connecting parts where the fusion faces form an angle of between 60° and 120°. Angles smaller than 60° are also permitted. However, in such cases the weld should be considered to be a partial penetration butt weld (for more see EN 1993-1-8, 4.3.2). |  |  |
| ### Butt welds A full penetration butt weld is defined as a weld that has complete penetration and fusion of weld and parent metal throughout the thickness of the joint. A partial penetration butt weld is defined as a weld that has joint penetration which is less than the full thickness of the parent material (for more see EN 1993-1-8, 4.3.4). |  |  |
| ### Plug welds Plug welds may be used: |  |  |
| - To transmit shear; - To prevent the buckling or separation of lapped parts; - To inter-connect the components of built-up members. |  |  |
| but should not be used to resist externally applied tension. (for more see EN 1993-1-8, 4.3.5) |  |  |
| ### Flare groove welds For solid bars the design throat thickness of flare groove welds, when fitted flush to the surface of the solid section of the bars, is defined in Figure 4.2 of EN 1993-1-8. The definition of the design throat thickness of flare groove welds in rectangular hollow sections is given in EN 1993-1-8, 7.3.1 (7). |  |  |
| - Design resistance of a fillet weld |  |  |
| - Length of welds |  |  |
| The effective length of a fillet weld l should be taken as the length over which the fillet is full-size. This maybe taken as the overall length of the weld reduced by twice the effective throat thickness a. Provided that the weld is full size throughout its length including starts and terminations, no reduction in effective length need be made for either the start or the termination of the weld. |  |  |
| A fillet weld with an effective length less than 30 mm or less than 6 times its throat thickness, whichever is larger, should not be designed to carry load. |  |  |
| - Effective throat thickness |  |  |
| The effective throat thickness, a, of a fillet weld should be taken as the height of the largest triangle (with equal or unequal legs) that can be inscribed within the fusion faces and the weld surface, measured perpendicular to the outer side of this triangle, see Figure 30. |  |  |
| The effective throat thickness of a fillet weld should not be less than 3 mm. In determining the design resistance of a deep penetration fillet weld, account may be taken of its additional throat thickness, see Figure 31, provided that preliminary tests show that the required penetration can consistently be achieved. |  |  |
| - Throat thickness of a fillet weld |  |  |
| - Throat thickness of a deep penetration fillet weld |  |  |
| - Design Resistance |  |  |
| ### General |  |  |
| The design resistance of a fillet weld should be determined using either the Directional method given in EN 1993-1-8, 4.5.3.2 or the Simplified Method. |  |  |
| ### Simplified method for design resistance of fillet weld |  |  |
| Alternatively to EN 1993-1-8, 4.5.3.2 the design resistance of a fillet weld may be assumed to be adequate if, at every point along its length, the resultant of all the forces per unit length transmitted by the weld satisfy the following criterion: |  |  |
| where Fw.Ed is the design value of the weld force per unit length; Fw.Rd is the design weld resistance per unit length. |  |  |
| Independent of the orientation of the weld throat plane to the applied force, the design resistance per unit length Fw,Rd should be determined from: |  |  |
| where fvw.d = design shear strength of the weld. |  |  |
| The design shear strength fvw.d of the weld should be determined from: |  |  |
| where fu and w are defined in EN 1993-1-8, 4.5.3(7). |  |  |
| - Design resistance of fillet welds all round |  |  |
| The design resistance of a fillet weld all round should be determined using one of the methods given in 7.4.3. |  |  |
| - Design resistance of butt welds |  |  |
| The design resistance of a full penetration butt weld should be taken as equal to the design resistance of the weaker of the parts connected, provided that the weld is made with a suitable consumable which will produce all-weld tensile specimens having both a minimum yield strength and a minimum tensile strength not less than those specified for the parent metal. |  |  |
| - Design Resistance of basic components |  |  |
| - Column web panel in shear |  |  |
| The design methods given in EN 1993-1-8, 6.2.6.1(2) to 6.2.6.1(14) are valid provided the column web slenderness satisfies the condition d / tw  69, where d is the clear depth of the web, tw is the thickness of the web and  = (235/fy)0,5. For a single-sided joint, or for a double-sided joint in which the beam depths are similar, the design shear resistance Vwp,Rd of an unstiffened column web panel, subject to a design shear force Vwp,Ed should be obtained using: |  |  |
| where Avc = shear area of the column, see EN 1993-1-1. |  |  |
| The design shear resistance may be increased by the use of stiffeners or supplementary web plates. Where transverse web stiffeners are used in both the compression zone and the tension zone, the design plastic shear resistance of the column web panel Vwp,Rd may be increased by Vwp,add,Rd given by: |  |  |
| where ds = distance between the centrelines of the stiffeners; Mpl.fc.Rd = design plastic moment resistance of a column flange, Mpl.st.Rd = design plastic moment resistance of a stiffener. |  |  |
| In welded joints, the transverse stiffeners should be aligned with the corresponding beam flange. When diagonal web stiffeners are used the design shear resistance of a column web should be determined according to EN 1993-1-1. In double-sided beam-to-column joint configurations without diagonal stiffeners on the column webs, the two beams are assumed to have similar depths. |  |  |
| Where a column web is reinforced by adding a supplementary web plate, see Figure 32, the shear area Avc may be increased by bs twc. If a further supplementary web plate is added on the other side of the web, no further increase of the shear area should be made. |  |  |
| Supplementary web plates may also be used to increase the rotational stiffness of a joint by increasing the stiffness of the column web in shear, compression or tension, see EN 1993-1-8, 6.3.2(1). |  |  |
| The steel grade of the supplementary web plate should be similar to that of the column. |  |  |
| The width bs should be such that the supplementary web plate extends at least to the toe of the root radius. |  |  |
| The length ls should be such that the supplementary web plate extends throughout the effective width of the web in tension and compression, see Figure 32. |  |  |
| The thickness ts of the supplementary web plate should be not less than the column web thickness twc. |  |  |
| The welds between the supplementary web plate and profile should be designed to resist the applied design forces. |  |  |
| The width bs of a supplementary web plate should be less than 40ts. |  |  |
| Discontinuous welds may be used in non corrosive environments. |  |  |
| (a) layout (b) Note: Weldabilty at the corners should be taken into account Examples of cross-section with longitudinal welds |  |  |
| - Examples of supplementary web plates |  |  |
| - Column web in transverse compression |  |  |
| The design resistance of an unstiffened column web subject to transverse compression should be determined from: |  |  |
| where  = reduction factor to allow for the possible effects of interaction with shear in the column web panel according to Table 9; beff,c,wc = effective width of column web in compression. |  |  |
| For a welded connection: |  |  |
| ac, rc and ab are as indicated in EN 1993-1-8, Figure 6.6. For rolled I or H sections, s = rc and for welded sections s = 2 ac . |  |  |
| For bolted end-plate connection: |  |  |
| sp is the length obtained by dispersion at 45° through the end-plate (at least tp and, provided that the length of end-plate below the flange is sufficient, up to 2 tp). |  |  |
| For bolted connection with angle flange cleats: |  |  |
| λp= plate slenderness: |  |  |
| For rolled I or H section column: |  |  |
| For welded I or H section column: |  |  |
| kwc = reduction factor and is given in EN 1993-1-8, 6.2.6 (2) |  |  |
| - Reduction factor  for interaction with shear |  |  |
|  |  | EN 1993-1-8 |
| - Rotational stiffness |  |  |
| - Basic model |  |  |
| The rotational stiffness of a joint should be determined from the flexibilities of its basic components, each represented by an elastic stiffness coefficient ki obtained from EN 1993-1-8, 6.3.2. These elastic stiffness coefficients are for general application. |  |  |
| For bolted end-plate connections with more than one row of bolts in tension, the stiffness coefficients ki for the related basic components should be combined. For beam-to-column joints and beam splices a method is given in EN 1993-1-8, 6.3.3 and for column bases a method is given in EN 1993-1-8, 6.3.4. |  |  |
| In a bolted connection with more than one bolt-row in tension, as a simplification the contribution of any bolt-row may be neglected, provided that the contributions of all other bolt-rows closer to the centre of compression are also neglected. The number of bolt-rows retained need not necessarily be the same as for the determination of the design moment resistance. |  |  |
| Provided that the axial force NEd in the connected member does not exceed 5 % of the design resistance Npl.Rd of its cross-section, the rotational stiffness Sj of a beam-to-column joint or beam splice, for a moment Mj,Ed less than the design moment resistance Mj,Rd of the joint, may be obtained with sufficient accuracy from: |  |  |
| where ki is the stiffness coefficient for basic joint component i; z is the lever arm, see EN 1993-1-8, 6.2.7; µ is the stiffness ratio Sj,ini / Sj , see EN 1993-1-8, 6.3.1 (6); The initial rotational stiffness Sj,ini of the joint is given by expression with µ = 1,0. |  |  |
| The rotational stiffness Sj of a column base, for a moment Mj,Ed less than the design moment resistance Mj,Rd of the joint, may be obtained with sufficient accuracy from EN 1993-1-8, 6.3.4. The stiffness ratio µ should be determined from the following: |  |  |
| If |  |  |
| If |  |  |
| in which the coefficient  is obtained from Table 10. |  |  |
| - Value of the coefficient  |  |  |
| The basic components that should be taken into account when calculating the stiffness of a welded beam-to-column connection and a bolted angle flange cleat are given in Table 11. Similarly, the basic components for a bolted end-plate connection and a base plate are given in EN 1993-1-8, Table 6.10. In both of these tables the stiffness coefficients, ki, for the basic components are defined in EN 1993-1-8, Table 6.11. |  |  |
| For beam-to-column end plate joints the following procedure should be used for obtaining the joint stiffness. The equivalent stiffness coefficient, keq, and the equivalent lever arm, zeq, of the connection should be obtained from EN 1993-1-8, 6.3.3. The stiffness of the joint should then be obtained from EN 1993-1-8, 6.3.1(4) based on the stiffness coefficients, keq (for the connection), k1 (for the column web in shear), and with the lever arm, z, taken equal to the equivalent lever arm of the connection, zeq. |  |  |
| - Joints with welded connections or bolted angle flange cleat connections |  |  |
| - Rotation capacity |  |  |
| - General |  |  |
| In the case of rigid plastic global analysis, a joint at a plastic hinge location should have sufficient rotation capacity. |  |  |
| The rotation capacity of a bolted or welded joint should be determined using the provisions given in EN 1993-1-8, 6.4.2 or 6.4.3. The design methods given in these clauses are only valid for S235, S275 and S355 steel grades and for joints in which the axial force NEd in the connected member does not exceed 5% of the design plastic resistance Npl,Rd of its cross-section. |  |  |
| As an alternative to EN 1993-1-8, 6.4.2 and 6.4.3 the rotation capacity of a joint need not be checked provided that the design moment resistance Mj,Rd of the joint is at least 1,2 times the design plastic moment resistance Mpl,Rd of the connected member. |  |  |
| In cases not covered by EN 1993-1-8, 6.4.2 and 6.4.3 the rotation capacity may be determined by testing in accordance with EN 1990, Annex D. Alternatively, appropriate calculation models may be used, provided that they are based on the results of tests in accordance with EN 1990. |  |  |
| - Bolted joints |  |  |
| A beam-to-column joint in which the design moment resistance of the joint Mj,Rd is governed by the design resistance of the column web panel in shear, may be assumed to have adequate rotation capacity for plastic global analysis, provided that d / tw  69 where d is the clear depth of the web, tw is the thickness of the web and  = (235/fy)0,5. |  |  |
| A joint with either a bolted end-plate or angle flange cleat connection may be assumed to have sufficient rotation capacity for plastic analysis, provided that both of the following conditions are satisfied: |  |  |
| a) The design moment resistance of the joint is governed by the design resistance of either: - The column flange in bending or; - The beam end-plate or tension flange cleat in bending. b) The thickness t of either the column flange or the beam end-plate or tension flange cleat (not necessarily the same basic component as in (a)) satisfies: |  |  |
| where d is the bolt diameter, fub is the ultimate bolt strength and fy = yield strength of the relevant basic component |  |  |
| A joint with a bolted connection in which the design moment resistance Mj,Rd is governed by the design resistance of its bolts in shear, should not be assumed to have sufficient rotation capacity for plastic global analysis. |  |  |
| - Welded Joints |  |  |
| The rotation capacity Cd of a welded beam-to-column connection may be assumed to be not less that the value given by the following expression provided that its column web is stiffened in compression but unstiffened in tension, and its design moment resistance is not governed by the design shear resistance of the column web panel, see EN 1993-1-8, 6.4.2 (1): |  |  |
| where hb is the depth of the beam; hc is the depth of the column. |  |  |
| An unstiffened welded beam-to-column joint designed in conformity with the provisions of this section, may be assumed to have a rotation capacity Cd of at least 0,015 radians. |  |  |
| - Hollow section joints |  |  |
| - General |  |  |
| - Scope |  |  |
| This section gives detailed application rules to determine the static design resistances of uniplanar and multiplanar joints in lattice structures composed of circular, square or rectangular hollow sections, and of uniplanar joints in lattice structures composed of combinations of hollow sections with open sections. |  |  |
| The static design resistances of the joints are expressed in terms of maximum design axial and/or moment resistances for the brace members. |  |  |
| These application rules are valid both for hot finished hollow sections to EN 10210 and for cold formed hollow sections to EN 10219, if the dimensions of the structural hollow sections fulfil the requirements of this section. |  |  |
| For hot finished hollow sections and cold formed hollow sections the nominal yield strength of the end product should not exceed 460 N/mm². For end products with a nominal yield strength higher than 355 N/mm ², the static design resistances given in this section should be reduced by a factor 0,9. |  |  |
| The nominal wall thickness of hollow sections should not be less than 2.5 mm. |  |  |
| The nominal wall thickness of a hollow section chord should not be greater than 25 mm unless special measures have been taken to ensure that the through thickness properties of the material will be adequate |  |  |
| For fatigue assessment see EN 1993-1-9. |  |  |
| The types of joints covered are indicated in Figure 33. |  |  |
| - Field of application |  |  |
| The application rules for hollow section joints may be used only where all of the conditions given in EN 1993-1-8, 7.1.2 (2) to 7.1.2 (8) are satisfied. |  |  |
| The compression elements of the members should satisfy the requirements for Class 1 or Class 2 given in EN 1993-1-1 for the condition of pure bending. |  |  |
| The angles i between the chords and the brace members, and between adjacent brace members, should satisfy: i  30°. |  |  |
| The ends of members that meet at a joint should be prepared in such a way that their cross-sectional shape is not modified. Flattened end connections and cropped end connections are not covered in this section. |  |  |
| In gap type joints, in order to ensure that the clearance is adequate for forming satisfactory welds, the gap between the brace members should not be less than (t1 + t2). |  |  |
| In overlap type joints, the overlap should be large enough to ensure that the interconnection of the brace members is sufficient for adequate shear transfer from one brace to the other. In any case the overlap should be at least 25%. |  |  |
| Where overlapping brace members have different thicknesses and/or different strength grades, the member with the lowest ti fyi value should overlap the other member. |  |  |
| Where overlapping brace members are of different widths, the narrower member should overlap the wider one. |  |  |
| - Types of joints in hollow section lattice girders |  |  |
| - Design of hollow section connections |  |  |
| - General |  |  |
| The design values of the internal axial forces both in the brace members and in the chords at the ultimate limit state should not exceed the design resistances of the members determined from EN 1993-1-1. |  |  |
| The design values of the internal axial forces in the brace members at the ultimate limit state should also not exceed the design resistances of the joints given in section 7.4 (welded joints between CHS members), 7.5 (welded joints between CHS or RHS brace members and RHS chord members) or 7.6 (welded joints between CHS or RHS brace members and I or H section chords) as appropriate. |  | EN 1993-1-8 |
| The stresses 0,Ed or p,Ed in the chord at a joint should be determined from: |  |  |
| Where |  |  |
| - Failure modes for hollow section connections |  |  |
| The design joint resistances of connections between hollow sections and of connections between hollow sections and open sections, should be based on the following failure modes as applicable: |  |  |
| a) Chord face failure (plastic failure of the chord face) or chord yielding (plastic failure of the chord cross-section); |  |  |
| b) Chord side wall failure (or chord web failure) by yielding, crushing or instability (crippling or buckling of the chord side wall or chord web) under the compression brace member; |  |  |
| c) Chord shear failure; |  |  |
| d) Punching shear failure of a hollow section chord wall (crack initiation leading to rupture of the brace members from the chord member); |  |  |
| e) Brace failure with reduced effective width (cracking in the welds or in the brace members); |  |  |
| f) Local buckling failure of a brace member or of a hollow section chord member at the joint location. |  |  |
| The phrases printed in boldface type in this list are used to describe the various failure modes in the tables of design resistances given in 7.4 to 7.7. |  |  |
| Figure 34 illustrates failure modes (a) to (f) for joints between CHS brace and chord members. |  |  |
| - Failure modes for joints between CHS members |  |  |
| Figure 35 illustrates failure modes (a) to (f) for joints between RHS brace and chord members. |  |  |
| - Failure modes for joints between RHS members |  |  |
| Figure 36 illustrates failure modes (a) to (f) for joints between CHS or RHS brace members and I or H section chord members. |  |  |
| - Failure modes for joints between CHS or RHS brace members and I or H section chord members |  |  |
| - References |  |  |
| - DIN EN 1993-1-10:2005. Eurocode 3: Design of steel structures – Part 1-10: Material toughness and through-thickness properties; German version EN 1993-1-10:2005 - DIN EN 1994-1-1:2004. Eurocode 4: Design of composite steel and concrete structures – Part 1-1: General rules and rules for buildings; German version EN 1994-1-1:2004 - DIN EN ISO 13918:2006. Welding - Studs and ceramic ferrules for arc stud welding (ISO/DIS 13918:2005); German version prEN ISO 13918:2005 / Note: Intended as replacement for DIN EN ISO 13918 (1998-12). - DIN EN ISO 14555:2006. Welding – Arc stud welding of metallic materials (ISO 14555:2006); German version EN ISO 14555:2006 - DIN EN ISO 17659:2004. Welding – Multilingual terms for welded joints with illustrations (ISO 17659:2002); Trilingual version EN ISO 17659:2004 - EN 10210-1:2006. Hot finished structural hollow sections of non-alloy and fine grain structural steels – Part 1: Technical delivery requirements. Brussels: European Committee for Standardisation. - EN 10210-2:2006. Hot finished structural hollow sections of non-alloy and fine grain structural steels – Part 2: Tolerances, dimensions and sectional properties. Brussels: European Committee for Standardisation. - EN 10219-1:2006. Cold formed welded structural hollow sections of non-alloy and fine grain steels – Part 1: Technical delivery requirements. Brussels: European Committee for Standardisation. - EN 10219-2:2006. Cold formed welded structural hollow sections of non-alloy and fine grain steels – Part 2: Tolerances, dimensions and sectional properties. Brussels: European Committee for Standardisation. - EN 1990:2002. Basis of structural design. Brussels: European Committee for Standardisation. - EN 1993-1-1:2005. Eurocode 3: Design of steel structures – Part 1.1: General rules and rules for buildings. - EN 1993-1-8:2005. Design of steel structures – Design of joints. Brussels: European Committee for Standardisation. - EN 1993-1-9:2005. Eurocode 3: Design of steel structures – Part 1.9: Fatigue. - EN ISO 5817:2007. Welding - Fusion-welded joints in steel, nickel, titanium and their alloys (beam welding excluded) - Quality levels for imperfections (ISO 5817:2003, corrected version:2005, including Technical Corrigendum 1:2006) - Kurobane, Y., Packer, J.A., Wardenier, J., Yeomans, N. 2004. Cidect Design Guide 9 – For Structural Hollow Section Column Connections. CIDECT Construction with hollow steel sections. TÜV Verlag Köln. |  |  |