- Module 7

Worked examples

Lecture 17Frame analysis – column member check

- Contents:

1.	Description of the studied frame

1.1.	Geometry

1.2.	Loading

1.2.1.	Basic loading

1.2.2.	Frame imperfection

1.2.3.	Load combination cases

1.2.4.	Partial safety factors on resistance – Initial choice of profiles

2.	Structural analysis

2.1.	Determination of internal forces

2.2.	Determination of buckling factors

2.3.	Cross-section classification

3.	Member verification

3.1.	According to Method 1

3.2.	According to Method 2

3.3.	Remarks

4.	Cross-section check

5.	References

| - Description of the studied frame |  |
| --- | --- |
| - Geometry |  |
| The structure of a three bay – four storey building is shown on Figure 1; it is composed of rectangular tubular columns and of open section beams. |  |
| - Frame geometry |  |
| The frame is 24 m wide, each span being 8 m; its total height is 12,8 m, each storey being 3,2 m high. The spacing of the frames is equal to 6 m in the perpendicular direction. |  |
| The structure is assumed to be braced out of the plane of the 6 m spans and to be unbraced in the plane of the 8 m spans. In the longitudinal direction of the building, i.e. in the direction perpendicular to the frame plane, bracing exists so that the columns are restrained in the direction of the 6 m spans at the floor levels and at the roof. Lateral restraint is provided to the beams by the concrete floor and roof slabs. The column bases (foundation level) are assumed to be nominally pinned. Although the structure is a 3D frame, it is treated as two plane frames, for sake of simplicity. |  |
| The total height of the building is close to the maximum length allowed for transportation (about 12 m). Therefore, it was decided to use continuous columns throughout the full height of the building. |  |
| The members are standard sections, made of S355 steel according to EN 10025. Finally, the structural joints are assumed to be rigid. |  |
| - Loading |  |
| - Basic loading |  |
| The values for the characteristic permanent and variable actions are given in Table 1. |  |
| - Vertical loading |  |
| Self weight and permanent actions; Variable actions (imposed loads). Roof level; 17,6 kN/m; 6 kN/m. Floor level; 17,6 kN/m; 16,5 kN/m. |  |
| The wind loads are applied as point loads of respectively 6,72 kN at the roof level and 13,44 kN at the 1st, 2nd and 3rd floor levels. They are realistic for the example. |  |
| The basic loading cases shown schematically in Figure 2 have been considered in appropriate combinations. It should be noted that Eurocode 1 permits the designer to assume uniform floor loading when designing columns, thus reducing the number of load cases. | EN 1991-1-1 6.2.2(1) |
| - Loading cases |  |
| - Frame imperfection |  |
| Frame imperfections are considered by means of equivalent horizontal loads. The formulae for the determination of the initial sway imperfection are given in Table 2. |  |
| - Global frame imperfections |  |
| - EC3 EN. with:. | EN 1993-1-1 5.3.2(3)a |
| In this frame, m = 4 (number of full height columns per floor) and h = 12,8 m (height of the structure), so that: |  |
| The equivalent horizontal load HEd = VEd  at each storey of the frame is derived from the initial sway  and the total design vertical load VEd in any storey for a given load case . The relevant values are listed in Table 3 for all the basic loading cases. |  |
| - Equivalent horizontal forces |  |
| Basic loading case (cf. Figure 2); Storey; VEd [kN]; HEd [kN]. G; - Roof; 422,4; 1,11. - 3rd floor; 422,4; 1,11. - 2nd floor; 422,4; 1,11. - 1st floor; 422,4; 1,11. I1; - Roof; 144,0; 0,38. - 3rd floor; 396,0; 1,04. - 2nd floor; 396,0; 1,04. - 1st floor; 396,0; 1,04. I2; - Roof; 48,0; 0,13. - 3rd floor; 264,0; 0,70. - 2nd floor; 132,0; 0,35. - 1st floor; 264,0; 0,70. I3; - Roof; 96,0; 0,25. - 3rd floor; 132,0; 0,35. - 2nd floor; 264,0; 0,70. - 1st floor; 132,0; 0,35. |  |
| They must of course be multiplied by the appropriate partial safety factors on actions. |  |
| - Load combination cases |  |
| Load combinations for the ultimate limit states are derived according to EN 1990 (cf. Table 3, Table 4 and Figure 2). However, as previously said, EN 1991–1–1 permits to assume that the floor is uniformly distributed when designing columns in multi-storey buildings. This reduces the number of load cases. |  |
| - Load combination cases at ULS |  |
| Load combination; Ultimate limit state. Case 1; 1,35 G + 1,5 W. Case 2; 1,35 G + 1,5 I1. Case 3; 1,35 G + 1,5 I2. Case 4; 1,35 G + 1,5 I3. Case 5; 1,35 G + 1,5 W + 1,05 I1. Case 6; 1,35 G + 0,9 W + 1,5 I1. Case 7; 1,35 G + 1,5 W + 1,05 I2. Case 8; 1,35 G + 0,9 W + 1,5 I2. Case 9; 1,35 G + 1,5 W + 1,05 I3. Case 10; 1,35 G + 0,9 W + 1,5 I3. |  |
| - Partial safety factors on resistance – Initial choice of profiles |  |
| For this worked example, the values of the partial safety factors on resistance are assumed to be M0 = M1 = 1,0 for resistance of cross-sections as well as for buckling resistance of members. The profiles of Figure 3 are adopted. |  |
| - Profile distribution |  |
| - Structural analysis |  |
| - Determination of internal forces |  |
| A linear elastic analysis is here conducted at ultimate limit states. The detailed results of the structural frame analysis are not systematically reported in the next sections, but reference is made to the key values of the internal forces and displacements, when required. |  |
| First, the classification of the frame under a given load combination must be investigated. This is achieved by computing the ratio cr of the elastic critical load Fcr of the frame for the sway buckling mode and the total design vertical load FEd acting on the frame: | EN 1993-1-1 5.2.1(4) |
| - where: FEd is the design value of the total vertical load; Fcr is the elastic critical load of the frame for the sway buckling mode; i designates the i-th storey; H,Ed is the horizontal displacement at the top of the storey relative to the bottom of this storey obtained from a first order elastic analysis; h is the storey height; HEd is the total horizontal reaction at the bottom of the storey; VEd is the total vertical reaction at the bottom of the storey. |  |
| The values of cr for each load combination are reported in Table 5. They have been respectively computed by means of the simplified formula of Eq. and the OSSA2D software, developed at the Department ArGenCo of the Liège University. |  |
| A quite good agreement between these two approaches can be observed in Table 5, as the maximum difference is less than 6%. |  |
| As all the values of the ratio Fcr / FEd are smaller than 10, the frame is classified as “sway” for all the load combinations, according to EN 1993–1–1. As a result, the 2nd order sway effects must be considered in the design and analysis process. |  |
| Three approaches may be contemplated for this purpose: |  |
| - Perform a second-order analysis including sway effects and check the member instability with non-sway buckling lengths (if these checks are not automatically achieved through the introduction of member imperfections into the second-order frame analysis); - Perform a first order analysis with amplification of the sway moments (provided that 3 ≤ Fcr / FEd ≤ 10) and check the member instability with non-sway buckling lengths; - Perform a first order analysis and check the member instability with sway buckling lengths. |  |
| For sake of simplicity, elastic analyses are considered here and the second method, i.e. the first order analysis with amplification of the sway moments, is selected. In EC3, the application of this procedure to multi-storey frames is limited to situations where the storeys have a similar distribution of vertical loads, distribution of horizontal loads and distribution of frames stiffness with respect to the applied storey shear forces. These conditions are assumed to be reasonably fulfilled in the present example. The sway moments are those associated with the horizontal translation of the top of the storey relative to the bottom of the same storey. In this worked example (symmetrical structure which is symmetrically loaded), the sway moments are due only to the horizontal loads, including the equivalent forces for sway imperfections. |  |
| - Frame classification |  |
| Load combination #; Storey; HEd [mm]; VEd [kN]; HEd [kN]; cr; cr exact; % cr; . 1; Roof; 1,03; 570; 12; 63,27; 5,44; 5,9; 1,210. 3rd; 3,25; 1140; 33; 28,70. 2nd; 5,85; 1711; 55; 17,54. 1st; 18,65; 2281; 77; 5,76. 2; Roof; -0,15; 786; 2; 56,77; 2,90; -3,2; 1,554. 3rd; 0,52; 1950; 5; 16,25. 2nd; 0,59; 3115; 8; 14,15. 1st; 3,00; 4279; 11; 2,81. 3; Roof; -0,17; 642; 2; 48,99; 3,71; -1,4; 1,376. 3rd; 0,46; 1608; 4; 18,36. 2nd; 0,50; 2377; 6; 16,81. 1st; 2,30; 3343; 9; 3,66. 4; Roof; -0,10; 714; 2; 83,40; 3,85; -4,7; 1,374. 3rd; 0,41; 1482; 4; 20,47. 2nd; 0,43; 2449; 6; 19,49. 1st; 2,30; 3217; 8; 3,67. 5; Roof; 1,01; 721; 12; 52,57; 3,37; 5,4; 1,392. 3rd; 3,37; 1707; 35; 19,34. 2nd; 6,03; 2694; 57; 11,32. 1st; 19,64; 3680; 80; 3,55. 6; Roof; 0,54; 786; 8; 60,86; 2,90; 4,4; 1,493. 3rd; 2,26; 1950; 23; 16,92. 2nd; 3,90; 3115; 38; 10,11. 1st; 13,23; 4279; 54; 3,03. 7; Roof; 0,99; 621; 12; 60,78; 4,10; 5,7; 1,300. 3rd; 3,32; 1468; 34; 22,36. 2nd; 5,97; 2177; 56; 13,82. 1st; 19,15; 3024; 79; 4,34. 8; Roof; 0,52; 642; 8; 74,34; 3,71; 5,2; 1,344. 3rd; 2,20; 1608; 22; 20,26. 2nd; 3,81; 2377; 37; 12,89. 1st; 12,54; 3343; 51; 3,90. 9; Roof; 1,04; 671; 12; 54,09; 4,22; 5,6; 1,289. 3rd; 3,29; 1380; 34; 23,87. 2nd; 5,92; 2227; 56; 13,65. 1st; 19,14; 2936; 78; 4,46. 10; Roof; 0,59; 714; 8; 60,11; 3,85; 4,7; 1,330. 3rd; 2,15; 1482; 22; 22,13. 2nd; 3,74; 2449; 37; 12,81. 1st; 12,53; 3217; 51; 4,03. |

| As can be seen in Table 5 for load combination #2, a value cr = 2,81 < 3 is reached, which means that for this particular situation, a complete (numerical) second-order analysis should be ideally performed. However, as the value of cr is quite close to the limit of 3, the amplification procedure is nevertheless illustrated in this example. |  |
| --- | --- |
| The value of the amplification factor to be applied to the sway moments is computed for each load combination case, in accordance with Eurocode 3. The results are given in Table 6. | EN 1993-1-1 5.2.2(5) |
| - Amplification factor |  |
| Load combination; cr. 1; 5,76; 1,210. 2; 2,81; 1,554. 3; 3,66; 1,376. 4; 3,67; 1,374. 5; 3,55; 1,392. 6; 3,03; 1,493. 7; 4,34; 1,300. 8; 3,90; 1,344. 9; 4,46; 1,289. 10; 4,03; 1,330. |  |
| In this particular example, a practical way of accounting for the amplification factors consists in amplifying the value of the partial safety factors to be applied to the horizontal actions: |  |
| - because these only ones contribute to the sway moments (symmetrical structure under symmetrical gravity loads); - because the principle of superposition applies in the case of linear first order structural analysis. |  |
| As a result, the internal forces and moments may simply be evaluated, for all the load combination cases at ultimate limit states, by applying the partial safety factors given in Table 7 to the external loads acting on the structure. The factors in the column entitled Wind are the “amplified partial safety factors” to be applied to wind and imperfection forces (obtained, for each combination, by multiplying the relevant partial safety factor (1,5 or 0,9) by the amplification factors listed in Table 6). |  |
| - Modified load factors including amplification effects |  |
| Combination #; ; Self weight; I1; I2; I3; Wind. 1; 1,210; 1,35; 1,815. 2; 1,554; 1,35; 1,5. 3; 1,376; 1,35; 1,5. 4; 1,374; 1,35; 1,5. 5; 1,392; 1,35; 1,05; 2,087. 6; 1,493; 1,35; 1,5; 1,344. 7; 1,300; 1,35; 1,05; 1,949. 8; 1,344; 1,35; 1,5; 1,210. 9; 1,289; 1,35; 1,05; 1,934. 10; 1,330; 1,35; 1,5; 1,197. |  |
| In the present worked example, the resistance and the stability of the 260x180x8 outer column (S355) are checked. It can be shown that combination n°5 leads to the most unfavourable design situation; the corresponding distributions of internal forces are given in Figure 4. |  |
| - Internal forces in the right outer column for load combination n°5 (resp. NEd and My,Ed, including P- 2nd order effects) |  |
| Therefore, the verification is performed at the lower storey level for NEd = 598 kN and My,Ed = 116,25 kNm. The maximum shear force in the column is 36,3 kN. |  |
| - Determination of buckling factors |  |
| For the out-of-plane instability of the column, the buckling length is equal to the system length, and so lz = 3,2 m. The in-plane buckling length ly of the lower column is then evaluated. Because of the rigid character of the structural joints, the ly buckling length depends on the stiffness of the adjacent members. The procedure of former EC3-ENV is applied here. | ENV 1993-1-1 Annex E |
| Kc: column stiffness coefficient: |  |
| K1: stiffness coefficient for the adjacent upper column segment: |  |
| K11, K12: effective stiffness coefficient for the beams assumed to bend with a single curvature in an assumed non-sway frame buckling mode (see § 2.1): |  |
| The distribution factor at the upper extremity of the column segment is then defined as: |  |
| Finally the buckling length coefficient is calculated through (2 = 1): |  |
| and then: |  |
| In accordance with Eurocode 3, the imperfection factors for flexural buckling about strong axis is y = z = 0,21 (hot finished profile). |  |
| Then, the classical buckling coefficients can be calculated: |  |
|  | EN 1993-1-1 6.3.1 |
| - Cross-section classification |  |
| The geometrical properties of an RHS 260x180x8 cross-section are as follows: |  |
| The steel grade is S355, and the flange thickness is less than 40 mm, thus fy = 355 N/mm2 and  is defined as follows: |  |
| Because the worse situation for the flanges is pure compression, the flange classification is as follows: |  |
| Flange:  The flanges are Class 1. | EN 1993-1-1 Table 5.2(2) |
| For the web, the plate slenderness is: |  |
|  | EN 1993-1-1 Table 5.2(1) |
| Even if we assume a web under full compression, which is the worst situation, the web is Class 2 since the limit of cw / t for a web to be Class 2 is 38  = 30,9. Therefore, the web is at least Class 2. |  |
| All elements being at least Class 2, use of plastic cross-section resistances is allowed. |  |
|  | EN 1993-1-1 Table 5.2(1) |
| - Member verification |  |
| - According to Method 1 | EN 1993-1-1 Annex A |
| The geometrical characteristics wy and wz are as follows: |  |
| and the “amplification factors” y and z: |  |
| In the present stage, the member must be supposed as non-sway since the P- sway effects have been accounted for in the analysis. Therefore, only the P- effects still need to be included in the verification. |  |
| When the P- effects develop, both the bending moment at the top of the column and the maximum second-order bending moment in span are changing, due to local “member” second-order effects. In the present example, the value of the bending moment at top of the member can only decrease since the adjacent beams do not loose their stiffness due to P- effects while the column does. Then, it is safe to assume that this bending moment remains constant. So, the member is supposed to be simply-supported with applied triangular bending moment My,Ed = 116,25 kNm. |  |
| Then, the M – N plasticity interaction factors Cyy and Czy are evaluated: |  |
| with . |  |
| Finally, the member checks for strong and weak axes respectively are: |  |
|  Satisfactory. |  |
| As a last step in the verification process, the resistance of the cross-sections at the end of the column segment must not be forgotten (cf. §4). |  |
| - According to Method 2 | EN 1993-1-1 Annex B |
| The ny and nz factors are as follows: |  |
| The coefficient of equivalence Cmy is taken for the non-sway single-span member: |  |
| for : |  |
| kzy may be taken equal to 0 when uniaxial bending and compression My,Ed applies. Finally, the member checks for strong and weak axes respectively are: |  |
|  Satisfactory. |  |
| As a last step in the verification process, the resistance of the cross-sections at the end of the column segment must not be forgotten (cf. §4). |  |
| - Remarks |  |
| Whenever both Methods 1 and 2 lead to the same conclusion (i.e. the design of the member is safe), differences in the final results appear. |  |
| First, it is to be noted that for the major axis in-plane verifications (Eqs. and ), the results are relatively similar; the Method 2 formula however offers more carrying capacity, which is mainly due to the fact that specific kyy interaction factors are provided by Method 2 for hollow sections. |  |
| Then, when comparing Eqs. and , a significant difference arises, both in terms of results and of theoretical principles. Indeed, in particular cases, Method 2 assumes that no interaction occurs between major axis bending and weak axis buckling, unlike Method 1 which proposes a moderate influence of the My,Ed term. |  |
| - Cross-section check | EN 1993-1-1 6.2.9 |
| Because both Method 1 and 2 make use of the Cm concept, it is necessary to check the cross-section resistance of the end-sections of the member. Resistance formulae of EN 1993–1–1 are used here. |  |
| - Shear resistance: |  |
| Shear area: |  |
| Shear resistance: |  |
| So that: |  |
| As this ratio is less than 0,5, the effect of shear on the moment resistance is neglected. |  |
| - Resistance of the cross-sections at the extremities of the lower column segment: |  |
| The criteria to be fulfilled is: |  |
| For a RHS section: |  |
| For load combination n°5: |  |
| thus aw = 0,5 |  |
| wherefrom: |  |
|  Satisfactory. |  |

- References

- Eurocode 1: EN 1991–1–1: Eurocode 1: Actions on structures – Part 1–1: General actions - Densities, self-weight, imposed loads for buildings.

- Eurocode 3: ENV 1993–1–1: Eurocode 3: Design of steel structures. Part 1–1: General rules and rules for buildings.

- Eurocode 3: EN 1993–1–1: Eurocode 3: Design of steel structures. Part 1–1: General rules and rules for buildings.