- Module 2

Structural analysis and design

Lecture 5Frame classification and joint representation

- Contents:

1.	Frame classification

1.1.	Braced and unbraced

1.2.	Sway and non-sway frames

2.	Assessment of the elastic critical load for a frame in the sway mode

2.1.	Approximate procedure

2.2.	Grinter frame procedure

2.3.	Bifurcation analysis and step-by-step second-order analysis procedures

3.	Imperfections

3.1.	Frame imperfections

3.1.1.	Global geometric imperfection for frames

3.1.2.	Closed system of equivalent horizontal forces

3.2.	Imperfections for analysis of bracing systems

3.3.	Local member imperfections

3.4.	Combined global and local imperfections

4.	Joint representation for frame analysis

4.1.	Traditional approach

4.2.	The semi-rigid approach

4.2.1.	Classification by rigidity

4.2.2.	Classification of joints by ductility

4.2.3.	Classification of joints by strength

4.3.	Choice of joint model for analysis

5.	References

| - Frame classification |  |
| --- | --- |
| - Braced and unbraced |  |
| When bracing is provided it is normally used to prevent, or at least to restrict, sway in multi-storey frames. Common bracing systems are trusses or shear walls (Figure 1). |  |
| - Common bracing systems |  |
| For a frame to be classified as a braced frame, it must possess a bracing system which is adequately stiff. |  |
| When it is justified to classify the frame as braced, it is possible to analyse the frame and the bracing system separately as follows: |  |
| - The frame without the bracing system can be treated as fully supported laterally and as having to resist the action of the vertical loads only; - The bracing system resists all the horizontal loads applied to the frames it braces, any vertical loads applied to the bracing system and the effects of the initial sway imperfections from the frames it braces and from the bracing system itself. |  |
| - It should be noted that in a frame with a truss type or frame type bracing system some members participate in the bracing system in addition to being part of the frame structure (without bracing). |  |
| - For frames without a bracing system and also for frames with a bracing system but which is not sufficiently stiff to allow classification of the frame as braced, the structure is classified as unbraced. In all case of unbraced frames, a single structural system, consisting of the frame and of the bracing when present, shall be analysed for both the vertical and horizontal loads acting together as well as for the effects of imperfections. |  |
| - The existence of a bracing system in a structure does not guarantee that the frame structure is to be classified as braced. Only when the bracing system reduces the horizontal displacements by at least 80% can the frame be classified as braced. |  |
| - If no bracing system is provided: the frame is unbraced; - When a bracing system is provided the following applies: when br > 0,2 unbr: the frame is classified as unbraced, - when br  0,2 unbr: the frame is classified as braced, - where: - br is the lateral flexibility of the structure with the bracing system. - unbr is the lateral flexibility of the structure without the bracing system. |  |
| - Sway and non-sway frames |  |
| - The term non-sway frame is applicable when the frame response to in-plane horizontal forces is sufficiently stiff for it to be acceptable to neglect any additional forces or moments arising from horizontal displacements of its nodes. The global second-order effects (i.e. the P- sway effects) may be neglected for a non-sway frame. | Part 1-1 5.2.1 |
| - When the global second-order effects are not negligible, the frame is said to be a sway frame. |  |
| - Braced and unbraced frame |  |
| - Normally a frame with bracing is likely to be classified as non-sway, while an unbraced frame is likely to be classified as sway. However, it is important to note that it is possible for an unbraced frame to be classified as non-sway (this is often the case of one storey portal frame buildings) while a frame with bracing may be classified as sway (possible for multi-storey buildings) (see Figure 2). |  |
| - When a frame is classified as non-sway, a first-order analysis may always be used. | Part 1-1 5.2.1(3) |
| - When a frame is classified as sway, a second-order analysis shall be used. A procedure involving iterations on a first-order elastic analysis is usually adequate for this purpose (see Lecture 4 “Structural idealisation and analysis”). Furthermore, on condition that the structure meets certain conditions, a first-order analysis (without any iteration process) may be used either by making a nominal correction to member end forces to allow for the global second-order effects or by analysing for vertical loads and for sway load effects (to be magnified for design) separately (see Lecture 6 “Choice of structural analysis and consequences on design”). | Part 1-1 5.2.2(4) |
| It should be noted that bracing systems which are themselves frames (or sub-frames) may also be classified as sway or non-sway. |  |
| - The classification of a frame structure (or bracing system) as sway or non-sway is based on the value of the ratio of the design value of the total vertical load FEd applied to the structure to its elastic critical value Fcr producing sway instability (failure in the sway mode). | Part 1-1 5.2.1(3) |
| - Obviously, the closer the applied load is to the critical load, the greater is the risk of instability and the greater are the global second-order effects on the structure (the P- effects). |  |
| - The classification rule is as follows: | Part 1-1 5.2.1(3) |
| - For elastic design: |  |
| the structure is classified as non-sway |  |
| the structure is classified as sway |  |
| - For plastic design: |  |
| the structure is classified as non-sway |  |
| the structure is classified as sway |  |
| - This rule can also be expressed in the following way: |  |
| - For elastic design: |  |
| the structure is classified as non-sway |  |
| the structure is classified as sway |  |
| - For plastic design: |  |
| the structure is classified as non-sway |  |
| the structure is classified as sway |  |
| - Where substantiated by more accurate approaches, the National Annexes to Eurocode 3 Part 1-1 may give a lower limit for cr for certain types of frames. |  |
| - Assessment of the elastic critical load for a frame in the sway mode |  |
| - Approximate procedure |  |
| - For portal frames with shallow roof slopes and beam-and-column type plane frames in building structures, the elastic critical buckling load in the sway mode may be calculated by the following procedure, provided that the axial compression in the beams or rafters is not significant: | 5.2.1(4)B |
| - The frame is analysed by a first-order elastic analysis for the specific load combination. The horizontal displacement of each storey due to the design loads (both horizontal and vertical) is determined; - The elastic critical load of the frame in the sway mode under the specific load combination case may be estimated from the following relationship: |  |
| where i designates the ith storey, and: |  |
| VEd is the design value of the total vertical reaction at the foundation, H,Ed is the horizontal (sway) displacement at the top of the ith storey, relative to the bottom of the ith storey, h is the ith storey height, HEd is the total horizontal reaction at the bottom of the ith storey. |  |
| Note: It can be observed that the approximation given above is based on an estimate of the ratio of the moment at a column base respectively due to the global second-order effects (the P- effect, given here as Vi i) and to the first-order sway moment (Hi hi). |  |
| - Grinter frame procedure |  |
| The principle used in this procedure is to replace an actual beam-and-column type plane frame in buildings, with beams connecting each column at each storey level, by a particular kind of frame known as the equivalent Grinter frame [1]. The elastic critical load for the Grinter frame is easy to determine and the value obtained is normally close to that for the actual frame it replaces. For multi-bay frames with semi-rigid joints (see Section 4 below), it is best to first find an intermediate single-bay substitute frame with rigid joints before determining the equivalent Grinter frame. |  |
| A multi-story multi-bay frame with rigid or semi-rigid joints is first replaced by an equivalent substitute single bay frame with (i) rigid joints and (ii) columns and beams with equivalent stiffness (Figure 3a and b). This equivalent frame is defined such that the lateral displacements of each storey are as similar as possible to that of the original frame. Consequently, the elastic critical load for both structures should be similar. |  |
| It is assumed that the columns behave elastically and are continuous over their whole height. Accordingly the stiffness of the column at each storey is obtained as follows: |  |
| where Kc,j is the column j stiffness coefficient, Ic,j / Lc,j. |  |
| The equivalent stiffness coefficient of the beam with linear restraints at each storey is obtained as follows: |  |
| where Kb,equi,i = Ib,equi,i / Lb,i in which: |  |
| and (i = 0 for a rigid joint) |  |
| EIb,i / Lb,i is the flexural stiffness of the considered beam i, and Sj,ini is the initial joint stiffness at the end of the considered beam in the actual structure. For a beam in which the joint stiffness are not the same at each end, either the lowest joint stiffness is used (conservative) or else, an assessment can be made to determine an appropriate single value for the individual equivalent beam stiffness. | Part 1-8 6.3 |
| As the so-defined substitute frame has rigid joints, the associated Grinter frame can now be derived (Figures 3b and 3c). Since the lateral displacements at each storey of the real, the substitute and the equivalent Grinter frames are similar, the values of the elastic critical loads for all three frames can be expected to be also similar. |  |
| The stiffness of the members in the Grinter frame are the following: |  |
| The elastic critical load of the actual frame with semi rigid joints can be computed by referring to the associated Grinter frame. The computation steps are as follows: |  |
| - In each storey, the critical load of the column stub Fcr* is computed on the basis of its buckling length in the sway mode taking the end restraints into account; - Each column stub of the Grinter frame is thus characterised by a value of Fcr*. The lowest of all these values, Fcr,min*, is selected as being a safe lower bound for the elastic critical load of the whole Grinter frame and thus of the whole actual frame. | ENV 1993 Annex E |
| Fehler! Es ist nicht möglich, durch die Bearbeitung von Feldfunktionen Objekte zu erstellen. a – Actual frame b – Substitute frame c – Grinter frame (semi-rigid rigid joints) (rigid joints) - From an actual frame to a Grinter frame |  |
| - Bifurcation analysis and step-by-step second-order analysis procedures |  |
| Special computer programmes exist for obtaining the elastic critical load of frames. Joints, when considered as semi-rigid, are characterised by their initial stiffness in these analyses. |  |
| In addition, for specific frame configurations, special charts are available which permit rapid determination of the elastic critical load value (see references [1], [2]). |  |
| - Imperfections |  |
| In the structural analysis, effects of imperfections have to be considered, including residual stresses and geometrical imperfections such as lack of verticality, lack of straightness, lack of flatness, lack of fit, and any minor eccentricities present in joints. | Part 1-1 5.3.1 |
| Equivalent geometric imperfections reflecting the effects of all type of imperfection are often used in the structural analysis; one usually distinguishes: |  |
| - Global imperfections for frames (frame imperfections); - Global imperfections for bracing systems; - Local imperfections for individual members (member imperfections). |  |
| - Frame imperfections | Part 1-1 5.3.2 |
| The effects of global frame imperfections must be accounted for in the global analysis. The shape of these imperfections may be derived from the elastic buckling mode of the structure in the plane of buckling considered. Alternatively the form of an equivalent geometric imperfection, i.e. an initial sway (see Figure 4a) may be adopted. |  |
| These initial sway imperfections should apply in all relevant directions, but need only to be considered in one direction at a time. |  |
| Care must obviously be taken of the influence that the direction of the global imperfection may have on the resulting values of forces and moments in the frame; and the worst situation should be considered. |  |
| The frame imperfections are treated as a load case to be used in conjunction with all the critical load combinations acting on the frame. Both in and out of plane buckling including torsional buckling with symmetric and asymmetric buckling shapes should be taken into account in the most unfavourable direction and form. |  |
| These global imperfections can also be accounted for by introducing equivalent lateral loads at the floor levels. These ones should be applied at each floor and roof level. |  |
| The following paragraphs describe the two possible ways for introducing frame imperfections. |  |
| a – Global frame imperfection b – Local member imperfection - Global and local imperfections |  |
| - Global geometric imperfection for frames |  |
| The frame imperfections are quantified in terms of an initial sway rotation of the frame relative to the foundation of the columns (Figure 5a). The initial sway imperfections are determined directly from the following formula: |  |
|  | Part 1-1 5.3.2(3) |
| where: , but (reduction factor for height h), (reduction factor for the number of columns in a row), h is the full height of the considered frame in meters, m is the number of columns in a row including those columns which carry a vertical load NEd not less than 50% of the average value of the column in the vertical plane considered. |  |
| There are situations in which sway imperfections may be disregarded. The following criterion: HEd ≥ 0,15 VEd is suggested in Eurocode 3 where it is also explained how Eq. may be applied to determine values of horizontal forces acting on floor diaphragms. | Part 1-1 5.3.2(4)B 5.3.2(5)B |
| - Closed system of equivalent horizontal forces |  |
| The alternative method for introducing the global imperfections of the frame, which may be more convenient than the introduction of the “out of plumb” or sway type of imperfection, is to use a closed system of equivalent horizontal forces instead. | Part 1-1 5.3.2(7) |
| The procedure used for determining these forces is the same as that for obtaining the equivalent lateral load to account for the P- sway effects (see the Lecture 4 “Structural idealisation and analysis”). The equivalent horizontal forces at each roof and floor level are calculated by multiplying the proportion of the vertical load applied at the level by the initial sway imperfection. They may be applied in any horizontal direction, but only in one sway direction at time (see Figure 5b). |  |
| a – Sway imperfection  b - Equivalent forces - Global frame imperfections |  |
| The equivalent horizontal forces obtained by multiplying the vertical reactions by the initial sway imperfections are applied at the supports (in the opposite direction to those applied at the other levels) so that the equivalent horizontal forces on the entire frame form a closed system, i.e. the net equivalent horizontal force applied to the entire structure becomes zero. |  |
| - Imperfections for analysis of bracing systems | Part 1-1 5.3.3 |
| Bracing systems which are required to provide lateral stability within the length of beams or compression members must also be analysed with allowance being made for an equivalent geometric imperfection of the members to be restrained. |  |
| The initial bow imperfections for that case are determined using a specific procedure which is not described here. | Part 1-1 5.3.3(1) |
| The initial bow imperfections may be replaced by an equivalent stabilising force. In addition, when the restrained members are spliced, the bracing system shall be capable of resisting a local force applied to it at that point by each beam or compression member which is spliced. | Part 1-1 5.3.3(2) 5.3.3(4) 5.3.3(5) |
| - Local member imperfections |  |
| Similarly to global sway imperfections, local member imperfections must basically be accounted for in the global analysis, as they generate additional P- effects. The shape of these imperfections may be derived from the elastic buckling mode of the structure in the plane of buckling considered. Alternatively an equivalent geometric imperfection, i.e. an initial bow imperfection (see Figure 4b) may be adopted. | Part 1-1 5.3.4(2) 5.3.2(1) 5.3.2(3) |
| These initial member imperfections should apply in all relevant directions, but need only to be considered in one direction at a time. | Part 1-1 |
| Values of e0 / L (Figure 4b) to apply are specified in the National Annexes, but recommended values are however suggested in Eurocode 3. They depend on the relevant member buckling curve and the type of structural analysis used. | 5.3.2(8) 5.3.2(3) |
| The effects of member imperfections may be neglected when carrying out the global analysis of frames, except in some specific cases of slender members. For those cases for which it can be neglected, the effect is presumed to be included in the appropriate buckling verification formula. |  |
| In practice, it is recognised that bow imperfections must be integrated in the structural analysis, additionally to global sway imperfections, in the case of members subjected to axial compression which have at least one moment-resisting joint at one end and for which: | Part 1-1 5.3.2(6) |
| where: NEd is the design value of the compression force and λ (Class 1, 2 or 3 sections) is the in-plane non-dimensional slenderness calculated for the member considered as hinged at its ends. |  |
| Care should be obviously taken of the influence that the direction of the initial bow imperfection may have on the resulting values of forces and moments in the member; and the worst situation should be considered. |  |
| - Combined global and local imperfections |  |
| For cases where global sway imperfections and local member imperfections are to be considered simultaneously in the structural analysis, the definition of both the shape and the amplitude of the imperfections may result from the application of the guidelines given in the previous paragraphs. |  |
| One of the possible solutions is to adopt the shape of the elastic critical mode of the structure as a unique global and local imperfection. Special attention has obviously be paid to the value of the amplitude of this imperfection. In Eurocode 3, relevant formulae are suggested which are not explicitly reported here. | Part 1-1 5.3.2(11) |
| - Joint representation for frame analysis |  |
| - Traditional approach |  |
| In the traditional approach to the design of building structures, the joints between the members are considered as being either rigid or pinned. |  |
| This approach is particularly appropriate for framed structures which are classified as braced non-sway where most of the beam-to-column joints are not required to transmit any moment. Those joints that must transmit moment are usually part of the bracing system and as such need to be quite rigid. |  |
| It is also useful when the so called “wind moment method” is used. In this latter approach, the beam-to-column joints are considered to transmit no moments for vertical loading cases but to transmit moments for wind loading alone. However, it is not clear that this approach is acceptable within the context of the application of Eurocode 3 Part 1-1. It can be regarded as a simple way for accounting for the actual semi-rigid behaviour of joints which is treated in the following section. |  |
| - The semi-rigid approach |  |
| - Classification by rigidity | Part 1-8 5.2.2 |
| The rotational behaviour of actual joints has often to be recognised as being intermediate between the two extreme situations of rigid and pinned joints respectively. |  |
| In the following, examples of joints between a beam and a column are used for illustration. The elastic rotations which result at a typical beam-to-column joint of a frame, due to loads acting on the beam for example, are shown in Figure 6. |  |
| When all the different parts of the joint are sufficiently stiff, there will be almost no difference between the respective rotations at the ends of the members connected at the joint. The joint can be considered as rigid (Figure 6a). A rigid joint experiences a single global rigid-body rotation which is the nodal rotation in the commonly used analysis methods for framed structures. |  |
| Should the joint have a very small flexural stiffness, then the beam will behave much as a simply supported beam, whatever the behaviour of the other connected member(s) (Figure 6b). The joint can be considered to be a nominally pinned joint, the relative rotation between the beam and the column being almost the same as that for the simply supported beam end. |  |
| - Classification of joints according to stiffness |  |
| For intermediate cases (non-zero finite joint stiffness), the transmitted moment will result in a difference between the absolute rotations of the two connected members (Figure 6c). Then the joint is said to be “semi-rigid”. |  |
| The simplest way to represent the joint in the global analysis is by an equivalent rotational spring between the ends of the two connected members. For the beam-to-column joint shown, the spring is placed at the beam end. The rotational stiffness Sj of this spring is the parameter that links the transmitted moment Mj with the relative rotation j, which is the difference between the absolute rotations of the two connected members. The greater the joint stiffness is compared to that of the connected member, the more rigid the joint is. The three categories of joint are represented as shown in Figure 7. |  |
| - Modelling of different joint categories in the elastic range of behaviour |  |
| In a frame with semi-rigid joints the loading will create both a bending moment Mj and a relative rotation j between the connected members. The moment and relative rotation are related through a constitutive law which depends on the joint properties. This is illustrated in Figure 7, where a linear-elastic joint behaviour is assumed. The effect, at the global analysis stage, of having semi-rigid joints instead of rigid or pinned joints will be to modify not only the displacements but also the distribution and magnitude of the internal forces in the structure. |  |
| - Classification of joints by ductility |  |
| As regards ductility or rotation capacity, joints can be classified in a manner similar to member cross-section classification, according to their ability to resist premature local instability and, more specifically, premature brittle failure (especially due to bolt failure). The ductility (or lack of ductility) of the joints used in a frame may have consequences on the type of global analysis that can be allowed. |  |
| One practical application of such a classification for joints is to check whether a plastic global analysis may be conducted up to the formation of a plastic collapse mechanism in the structure which might involve plastic hinges in some joints. | Part 1-8 5.1.3 & 5.1.4 |
| - Classification of joints by strength | Part 1-8 5.2.3 |
| Joints may also be classified according to their strength. |  |
| For the strength classification, joints are classified as “full-strength” or “partial-strength” according to whether their resistance is, respectively, at least equal to or less than that of the connected member with the lowest resistance. If the joint is a splice joint in a beam of constant section, the comparison is made with the moment resistance of the beam. For a typical beam-to-column joint, the comparison is made with the member having the lowest moment resistance at the joint location. In addition, when the joint cannot develop significant moments it can be classified as a “nominally pinned” joint. |  |
| The use of the strength classification compliments that of the ductility classification when using global plastic analysis. One can use it to foresee the possibility that a plastic hinge is likely to occur in the joint during the global analysis. |  |
| When full-strength joints are used, one can usually assume that no hinges occur in them. However, to be certain that this is in fact the case, it is required that a joint which is 20% over-strength be used. This is a good precaution since it is often difficult to design full strength joints which are also ductile. Member resistances on the other hand are often significantly higher than the design value due to over-strength material for example. | Part 1-8 6.4.1(3) |
| Hinges may occur in partial-strength joints. When a plastic hinge occurs in a partial-strength joint, so as to permit the increase of loads with the joint acting as a hinge once its plastic bending resistance has been reached, the joint must also be ductile. Fortunately, it is usually easy to design partial-strength joints which are ductile. The use of partial-strength ductile joints permits the easy identification of plastic hinge locations in floor beams for example, and this allows a very simple rigid-plastic design procedure for dimensioning beams particularly in braced non-sway frames. |  |
| - Choice of joint model for analysis |  |
| The extent to which joint behaviour is accounted for in a precise manner or not in the global analysis is treated in the Lecture 4 section related to the type of framing adopted for analysis. |  |
| There three types of framing are introduced, respectively called simple, continuous and semi-continuous. |  |
| The selection of the type of framing to be used in the structural analysis is achieved according to Table 1. | Part 1-8 5.1.1(4) |
| - Selection of joint model for frame analysis |  |
| Methods of global analysis; Classification of joints. Elastic; Nominally pinned; Rigid; Semi-rigid. Rigid-plastic; Nominally pinned; Full-strength; Partial-strength. Elastic-plastic; Nominally pinned; Rigid and full-strength; Semi-rigid and partial-strength Semi-rigid and full-strength Rigid and partial-strength. Type of joint model; Simple; Continuous; Semi-continuous. |  |
| When the joints are to be modelled as semi-rigid, more or less sophisticated idealisations of the moment–rotation characteristic curves may be adopted, ranging from the linear-elastic type to the full non-linear type. The use of a linear-elastic global analysis model requires that the joint behaviour be idealised as linear-elastic also. For elastic-perfectly plastic analysis a bi-linear joint model is needed. Therefore, the type of analysis used has a direct impact on the degree of complexity of the joint idealisation to be adopted, particularly when plastic analysis is used and hinges are allowed to occur in the joints. | Part 1-8 5.1 |

- References

- Wood, R.H., Effective lengths of columns in multi-storey buildings, The Engineer N° 7,8 and 9, Vol. 52, 1974.

- Petersen,C., “Stahlbau”, F.Vieweg & Sohn, Braunschweig, Wiesbaden, 1998.

- ENV 1993-1-1, Eurocode 3: Design of steel structures - Part 1-1: General rules and rules for buildings, May 2005.

- EN 1993-1-8. 2005. Eurocode 3: Design of steel structures – Part 1.8: Design of joints.