- Module 1

Introduction to the new Eurocodes

Lecture 3Introduction to Eurocode 3 Part 1-1

- Contents:

1.	Scope of Eurocode 3

2.	Scope of EN 1993-1-1

3.	Terminology

4.	Symbols

5.	Basis of design

6.	Material properties

7.	Durability

8.	Structural analysis

9.	Ultimate Limit States (ULS)

10.	Serviceability Limit States (SLS)

11.	References

| - Scope of Eurocode 3 |  |
| --- | --- |
| EN 1993 (Eurocode 3) applies to the design of buildings and civil engineering works in steel. It complies with the general principles and requirements given in EN 1990. It is concerned only with requirements for resistance, serviceability, ductility and fire resistance of steel structures. It is intended to be used in conjunction with EN 1990, EN 1991, EN 1992 to EN 1999, when steel components or steel structures refer to, and with standards for construction products relevant for steel structures. |  |
| Eurocode 3 is subdivided in six main parts. The master part is Part 1-1, designated as EN 1993-1-1. Therefore, present lecture focuses on this part only. |  |
| - Scope of EN 1993-1-1 |  |
| EN 1993-1-1 gives the basic design rules for steel structures with material thickness larger than 3 mm (for smaller thickness, it should be referred to EN 1993-1-3). Also, supplementary provisions for the structural design of steel buildings are given therein; these provisions are indicated by the letter ‘B’ after the paragraph number. |  |
| The arrangement of sections within EN 1993-1-1 is based on design criteria such as deflection, tension, compression, bending, shear, buckling etc., rather than on individual element types such as beams, columns, etc. |  |
| Eurocode 3 is organised into a number of chapters as follows: |  |
| Chapter 1 General |  |
| Provides additional clauses to those given in EN 1990: outlines especially the scope of Eurocode 3, gives the normative references, defines specific terms, introduces a list of symbols and specifies the conventions for member axes. |  |
| Chapter 2 Basis of design |  |
| Provides additional clauses to those given in EN 1990: outlines design requirements and principles of limit state design, introduces basic variables and describes the verification by the partial safety factor method and the design assisted by testing. |  |
| Chapter 3 Materials |  |
| Deals with material properties of products made of low alloy structural steels: specifies characteristic strengths for structural steel, fasteners and welding consumables. |  |
| Chapter 4 Durability |  |
| Mostly refers to EN 1990 and gives general rules for durability. |  |
| Chapter 5 Structural analysis |  |
| Comments on structural modelling prior to global analysis, specifies frame and member imperfections, provides guidance for the methods of global frame analysis and introduces the concept of cross-section classification. |  |
| More details about this subject are provided in Module 2 ‘Structural analysis and design’, see Lectures 4 to 6 of the present set of lectures. |  |
| Chapter 6 Ultimate limit states |  |
| Sets out detailed rules regarding the resistance of cross-sections and the buckling resistance of members with special attention paid to built-up compression members. Module 3 ‘Design of members’ (Lectures 6 to 11) deals with the detailed design rules to check the ultimate limit states. |  |
| Chapter 7 Serviceability limit states |  |
| Sets out limits on vertical and horizontal deflections (also includes guidance on dynamic effects). |  |
| There are a number of Annexes within EN 1993-1-1. All these annexes are ‘informative’ (i.e. they are not mandatory) and provide useful information or facilitate the design. It remains however to the National Annexes to decide whether their status is kept ‘informative’ or made ‘normative’. |  |
| - Annexes A and B: introduces the interaction factors for the verification of members subject to combined compression axial force and bending; - Annex AB: provides additional design provisions on how to account for material non linearity in global analysis, and simplified provisions for the design of continuous floor beams; - Annex BB: deals with buckling of components in building structures (members in lattice and triangulated structures, continuous restraints, stable lengths of segment containing plastic hinges for out of plane buckling). |  |
| - Terminology | Part 1-1 |
| The Eurocodes define a number of terms which, although often used generally in a rather loose way, have more precise meanings in the context of European standards. |  |
| Eurocode 3 includes especially a number of specific definitions as follows: | 1.5 |
| - Frame: whole or portion of a structure consisting in an assembly of directly connected structural elements; - Sub-frame: part of a larger frame treated in isolation for structural global analysis; - Type of framing: - Simple: frame in which the joints do not resist bending moments; - Continuous: frame in which only the structural properties of members (and not those of joints) need to be considered in the global analysis; - Semi-continuous: frame in which both member and joint structural properties need explicit consideration in the global analysis. - Global analysis: determination of consistent internal forces and moments which are in equilibrium with a particular set of actions; - System length: distance in a given plane between two adjacent points where a member is braced against lateral displacement in this plane, or between one such point and the end of this member; - Buckling length: system length of an equivalent pin-ended member which has the same buckling resistance as the actual member or segment of it; - Shear lag effects: non uniform distribution of direct stresses in wide flanges as a result of shear deformation; - Capacity design: design method aimed at achieving the plastic deformation capacity of a member by providing additional strength in its connections and/or in other parts connected to it; - Uniform member: member with a constant cross-section along its whole length. |  |
| - Symbols | Part 1-1 |
| A complete list of symbols is included in EN 1993-1-1. The most common of these are listed below. | 1.6 |
| Symbols of a general nature: |  |
| L, l Length, span, system length |  |
| F Load (in any generality) |  |
| H Horizontal load |  |
| R Resistance, reaction |  |
| M, N, V Internal forces: bending moment, axial force, shear force |  |
| X Material property |  |
| b Width |  |
| d Depth, diameter |  |
| h Height, depth |  |
| k, k Buckling coefficient for direct stresses , shear stresses  |  |
| r Radius |  |
| t Thickness |  |
|  Deflection |  |
|  Strain |  |
|  Direct stress, shear stress |  |
|  Sway imperfection |  |
|  Slenderness ratio |  |
|  Reduction factor for buckling |  |
|  Partial safety factor |  |
|  Stress or strain ratio |  |
| Symbols relating to cross-section properties: |  |
| A Cross-sectional area |  |
| i Radius of gyration |  |
| I Second moment of area |  |
| W Section modulus |  |
| Symbols relating to material properties: |  |
| E Modulus of elasticity (Young’s modulus) |  |
| G Shear modulus |  |
| f Strength |  |
|  Poisson’s ratio |  |
| EN 1993-1-1 also makes extensive use of subscripts. These can be used to clarify the precise meaning of a symbol. Some common subscripts are as follows: |  |
| d Design |  |
| E Effect |  |
| el Elastic |  |
| k Characteristic |  |
| LT Lateral torsional |  |
| pl Plastic |  |
| Normal symbols may also be used as subscripts, for example: |  |
| Rd Design resistance |  |
| Ed Design values of internal force or moment |  |
| Subscripts can be arranged in sequence when necessary, separated by a comma. For example: |  |
| Npl,Rd Design plastic axial resistance |  |
| The following convention is adopted for the member axes: | 1-7 |
| x-x Longitudinal axis of a member |  |
| y-y Axis of a cross-section (parallel to the flanges for an I section) |  |
| z-z Axis of a cross-section (parallel to the web for an I section) |  |
| Note: for angle sections the y-y and z-z axes are respectively parallel to the smaller and larger legs, and the major and minor axes of the cross-section are given by the following notation: |  |
| u-u Major principal axis |  |
| v-v Minor principal axis |  |
| - Basis of design | Part 1-1 |
| Additional clauses to those given in EN 1990 are provided. In this section, the design requirements and principles of limit state design are outlined and the basic variables are introduced. Also, both the verification by the partial safety factor method and the design assisted by testing are briefly described. | 2.1 to 2.5 |
| - Material properties | Part 1-1 |
| Five nominal steel grades (specified in EN 10025) are specifically included in EN 1993-1-1. They are: | 3.2.1 |
| - S235 with a nominal yield strength of 235 N/mm2; - S275 with a nominal yield strength of 275 N/mm2; - S355 with a nominal yield strength of 355 N/mm2; - S420 with a nominal yield strength of 420 N/mm2; - S460 with a nominal yield strength of 460 N/mm2. |  |
| Note that these specified yield strengths are reduced for elements of higher thickness. Values for the yield strength fy and the ultimate tensile strength fu can be obtained either by adopting the values fy = Reh and fu = Rm from the product standards (e.g. EN 10210 or EN 10219) or using simplified values given in EN 1993 Part 1-1. |  |
| Higher strength steels with a yield strength up to 690 N/mm² are dealt with in EN 1993-1-12. |  |
| For all structural steels the specified modulus of elasticity is 210 kN/mm2, the Poisson’s ratio in the elastic range is 0,3 and the coefficient of linear thermal expansion is taken as 1210-6 1/°K (for temperatures not exceeding 100°C). |  |
| Guidance is also given on material requirements for: | Part 1-1 |
| - Ductility (plastic analysis); - Fracture toughness; - Through-thickness properties. | 3.2.2 3.2.3 3.2.4 |
| Reference is made to EN 1993-1-8 regarding requirements for connecting devices and weld consumables. | 3.3 |
| - Durability | Part 1-1 |
| General rules for durability are given. However the basic requirements in this respect are set out in the standard EN 1990 ‘Execution of steel structures’. | 4 |
| - Structural analysis | Part 1-1 |
| Global analysis should be based on a calculation model and basic assumptions that are appropriate for the limit state under consideration, and likely to reflect the anticipated type of behaviour of the structure and its components (cross-sections, members, joints, bearings). The method used for the global analysis should be consistent with the design assumptions. | 5.1.1 |
| The effects of the behaviour of the joints on the distribution of internal forces and bending moments within a structure may generally be disregarded, but they should be taken into account when they are significant. Therefore, it is distinguished between three models of joint behaviour: simple, continuous and semi-continuous, the characterisation of each being referred to in EN 1993-1-8. | 5.1.2 |
| Where significant, ground-structure interaction should be accounted for. | 5.1.3 |
| Global analysis may be conducted either as elastic analysis or plastic analysis. In an elastic analysis, only the stiffness of the members and joints is concerned; strength is not managed within the analysis and should therefore be checked once the distribution of internal forces and bending moments is available. In a plastic analysis, stiffness, strength and ductility all together are involved. | 5.4 |
| General practice is to calculate the internal forces and bending moments according to an elastic global analysis. That does not prevent from basing the resistance of the cross-sections on their plastic resistance. |  |
| Depending on the effects of the deformed geometry of the structure, the internal forces and moments are determined based simply on a first-order analysis or they may require that second-order effects are duly computed. A criterion is given that permits to decide when the action effects or the structural behaviour are significantly modified or increased by the effects of the deformed geometry. | 5.2.1 |
| According to both the type of frame and global analysis, second order effects and imperfections may be accounted for through several methods. These methods are briefly commented on. | 5.2.2 |
| Either a second order analysis is carried out, in which case the internal forces and bending moments yet include the second order effects. Or, alternatively, the internal forces and bending moments are obtained from a first order analysis and they are implemented afterwards with additional ones that represent the second order effects but are got similarly from first order analysis. |  |
| Allowance for the effects of imperfections should be incorporated in the global analysis. Such imperfections are especially residual stresses and geometric imperfections; it is usual to consider the effects of both kinds of imperfections through those of single so-called equivalent geometric imperfections. |  |
| The following imperfections should be accounted for: | 5.3 |
| - Global imperfections for frames and bracing systems, in the form of lack of verticality (out of plumb) of the columns; - Local imperfections for individual members, in the form of lack of straightness of the compression members. |  |
| The imperfections which are possibly included in the resistance formulae for member design need not be modelled for the global analysis. |  |
| Cross-section classification is of paramount importance. It aims at identifying in which extent the resistance and rotation capacity of cross-sections is limited by local buckling resistance. Four classes of sections are defined. A cross-section is classified according to the least favourable class of its compression parts. The classification of a compression part of a cross-section depends on the width-to-thickness ratio (b/t ratio) of that part. The limiting proportions for Class 1, 2 and 3 compressions parts are obtained from tables given in EN 1993-1-1; the compression part which fails to satisfy the limits for Class 3 should be taken as Class 4. | 5.5.2 |
| Plastic global analysis is subordinated to specific cross-section requirements. | 5.6 |
| - Ultimate Limit States (ULS) |  |
| The ultimate limit state is concerned with the resistance of the structure to collapse. That is generally checked by considering the strength of individual elements subjected to forces determined from a suitable analysis. In addition, the overall stability of the structure must be checked. |  |
| EN 1993-1-1 provides different analytical approaches ranging from simple elastic analysis (assuming that the joints of the frame are all nominally pinned) to sophisticated elastic-plastic of semi-continuous frames (assuming partial-strength and semi-rigid joints). Full details are given elsewhere in this set of lectures (see Module 2 ‘Structural analysis and design’). |  |
| The ultimate limit state is examined under factored load conditions. In general, the effects on individual structural elements will be determined through the structural analysis, and each element is then treated as an isolated component for design. Details of individual design checks depend on the type of member (e.g. beam, column, …) and are described in other lectures (see lectures of Module 3 ‘Design of members’). |  |
| Framed structures must be especially checked for: |  |
| - Resistance of cross-sections; - Resistance of members to instability assessed by member checks; - Resistance of joints. | 6.2 6.3 |
| The resistance of joints is precisely the topic of EN 1993-1-8. |  |
| A whole section is dedicated to uniform built-up laced and battened compression members, where additional checks or some alterations in the usual checks are required. |  |
| - Serviceability Limit States (SLS) |  |
| Vertical and horizontal deflections and dynamic effects govern the serviceability limit states. |  |
| At the serviceability limit states, the calculation of vertical and/or horizontal deflections of a member or structure is seldom meaningful in itself since the design assumptions are rarely realised. This happens because, for example: |  |
| - The actual loading may be quite different from the assumed design loading; - A beam is seldom ‘simply supported’ or with ‘fixed ends’ but, in reality, is usually in some intermediate conditions; - The steelwork is very often stiffened by other building components such as floors and walls, roof and cladding, the effects of which are normally not included in the global analysis. |  |
| The calculated deflection is, however, valuable as an index of the stiffness of a member or structure, i.e. to assess whether adequate provision is made in relation to the limit state of deflection or local damage. For this purpose, sophisticated analytical methods are seldom justified. Whatever methods are adopted to assess the resistance and stability of a member or structure, the calculation of deflections should relate to elastic behaviour of the structure. For example, when analysis for strength is based on rigid-elastic or elastic-perfectly plastic concepts, an elastic analysis should also be considered to check deflections. |  |
| In determining the deflection it may be necessary to consider the effects of a preliminary camber, permanent loads and variable loads separately. |  |
| Calculated deflections should be compared with specified maximum values, which will depend upon circumstances. Limiting vertical deflections for beams and horizontal deflections for frames are recommended in EN 1990; the limit values may be specified in the National Annex. |  |
| The dynamic effects to be considered at the serviceability limit state are vibrations caused by machinery and self-induced vibrations, e.g. vortex shedding. Resonance can be avoided by ensuring that the natural frequencies of the structure (or any part of it) are sufficiently different from those of the excitation source. Both the oscillation and vibration of structures on which the public can walk, run or jump should be limited to avoid significant discomfort for the users. Some limits for the vibration of floors are recommended in EN 990; they may be specified in the National Annex. |  |

- References

- EN 1990: Eurocode - Basis of structural design. CEN, Brussels, April 2002.

- EN 1991: Eurocode 1 Actions on structures – Part 1-1: Actions on structures - densities, self-weight and imposed loads. CEN, Brussels, April 2002.

- EN 1993-1-1: Eurocode 3: Design of Steel Structures – Part 1-1: General rules and rules for buildings. CEN, Brussels, May 2005.

- EN 1993-1-8: Eurocode 3: Design of Steel Structures – Part 1-8: Design of Joints. CEN, Brussels, May 2005.