- Module 3

Design of members

Lecture 11Members in bending and axial force

- Contents:

1.	Introduction

2.	Cross-section resistance

2.1.1.	Bending and axial force in Class 1 and 2 cross-sections

2.1.2.	Bending and axial force in Class 3 cross-sections

2.1.3.	Bending and axial force in Class 4 cross-sections

3.	Member behaviour: theoretical background – Elastic 2nd order in-plane behaviour

4.	Extension to spatial behaviour and elastic-plastic behaviour

4.1.	Member under NEd + My,Ed + Mz,Ed

4.2.	Elastic-plastic behaviour

4.2.1.	Member under NEd + MEd

4.2.2.	Extension to beam-columns subjected to NEd + My,Ed + Mz,Ed

5.	Design formulae in EN 1993–1–1

5.1.	Format of presentation of the design rules in EN 1993–1–1

5.2.	“Method 1” set of design formulae

5.2.1.	Factors of EN 1993–1–1 and more appropriate format

5.2.2.	Cm factor

5.2.3.	Plasticity factors

5.2.4.	Continuity aspects

5.2.5.	Members with Class 3 and 4 cross-sections

5.3.	“Method 2” set of design formulae

5.3.1.	Factors of EN 1993–1–1

5.3.2.	More appropriate format for members with Class 1 and 2 cross-sections

a)	Axial Compression and strong-axis bending (NEd + My,Ed)

b)	Axial compression and weak-axis bending (NEd + Mz,Ed)

c)	Axial Compression plus biaxial bending (NEd + My,Ed + Mz,Ed)

d)	Equivalent uniform moment factors Cm

5.3.3.	Members with Class 3 and 4 cross-sections

6.	References

| - Introduction |  |
| --- | --- |
| Beam-columns are defined as members subjected to combined bending and axial force. In principle, all members in frame structures are actually beam-columns, since the particular cases of beams (NEd = 0) and columns (MEd = 0) are seen to be two extreme situations. Usually, beam-columns refer to members under bending and axial compression. However, members under bending and tension can also be seen as beam-columns; these members are consequently not prone to flexural buckling, and must be treated accordingly, i.e. their carrying capacity is mainly governed by cross-section resistance in the case of hollow sections members (except very rare situations where the h / b ratio is important thus the possibility for lateral torsional buckling to occur). |  |
| Members under bending and compression develop a specific load-carrying behaviour in the different ranges of slenderness. At very low slenderness the cross-sectional resistance dominates, described by the well-known interaction formulae for elastic or plastic limit states (see Section 2). With increasing slenderness, pronounced second-order effects appear, which are significantly influenced by both geometrical imperfections and residual stresses. In the high slenderness range, member buckling is dominated by elastic behaviour; the larger the slenderness, the greater the dominance. |  |
| Depending on the emphasis given to the different ranges, different concepts of interaction formulae have been proposed in the past, from the exponential form (stressing the plastic cross-sectional behaviour) to the linear-additive form (derived from linear-elastic buckling response). The present approach of EN 1993–1–1 has been based on the latter concept, where the effects of the axial force and the bending moments are linearly summed and the non-linear effects are accounted for by specific interaction factors (see Sections 3 and 4). The preference for this concept results from its user-friendliness, since it allows the evaluation of the individual effects of the axial force and the bending moments, which is of advantage in practical design work. |  |
| Two different formats of the interaction formulae are provided in EN 1993–1–1, called Method 1 and Method 2 (see respectively section 5.2 and 5.3). The main difference between them is the kind of presentation of the different structural effects, either by specific coefficients in Method 1 or by one compact interaction factor in Method 2. This makes Method 1 more adaptable to identifying and accounting for the structural effects, while Method 2 is mainly focussed on the direct design of standard cases. |  |
| - Cross-section resistance |  |
| This paragraph only provides additional information on the resistance of sections submitted to combined axial force and bending (eventually biaxial bending and axial force). Sections submitted to bending only or bending and shear are dealt with in Lecture 9. |  |
| - Bending and axial force in Class 1 and 2 cross-sections |  |
| This section deals with the resistance of so-called “plastic” cross-sections, i.e. sections where a full plastic distribution of stresses at failure is possible. According to Eurocode 3, such sections are denominated as Class 1 and 2 cross-sections (see Lecture 7). |  |
| - Class 1 or 2 cross-sections (RHS): |  |
| When an axial force is present coincidently with bending, allowance should be made for its effect on the section resistance to bending, which is designated as MN,Rd. | Part 1-1 6.2.9.1(1) |
| Resistance to combined bending and axial force is satisfied if the design resistance moment MEd at each cross-section is not larger than the reduced design resistance moment MN,Rd: | Part 1-1 6.2.9.1(2) |
| For cross-sections where fastener holes are not to be accounted for, the reduced design plastic resistance moment MN,Rd for RHS hollow sections with uniform thickness is approximated as follows: | Part 1-1 6.2.9.1(5) |
| for bending about the major axis |  |
| for bending about the minor axis |  |
| where: |  |
| - Class 1 or 2 circular hollow sections (CHS): |  |
| - No indication is given in EN 1993-1-1; in [7], the following interaction formula is suggested: |  |
| - where n is still given by Eq. . |  |
| When biaxial bending is of concern, beams bent about both axes of the cross-section have a plastic neutral axis inclined to the rectangular axes by an amount which depends on the ratio of the applied moments and the precise shape of the cross-section. The general format for biaxial bending is: |  |
|  | Part 1-1 6.2.9.1(6) |
| in which  and  are constants, which may defined as follows: |  |
| but  =  ≤ 6 for RHS |  |
| for CHS or conservatively be taken as unity. |  |
| - Bending and axial force in Class 3 cross-sections |  |
| In the absence of shear forces, the maximum longitudinal stress in Class 3 cross-sections shall satisfy the criterion: |  |
|  | Part 1-1 6.2.9.2(1) |
| where x,Ed is the design value of the local longitudinal stress due to moment and axial force. Another writing of the same criterion for sections under biaxial bending and axial force is: |  |
| - Bending and axial force in Class 4 cross-sections |  |
| The same principles as for Class 3 cross-section apply to Class 4 sections (Eq. ), provided that x,Ed is calculated on the basis of an effective cross-section. In the same way as previously, a section under biaxial bending and axial force must satisfy the criterion: |  |
|  | Part 1-1 6.2.9.3(2) |
| where eN is the shift of the relevant centroidal axis when the cross-section is subjected to compression only. |  |

| - Member behaviour: theoretical background – Elastic 2nd order in-plane behaviour |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Let us consider a simply supported prismatic member under axial compression NEd (cf. Figure 1). This member has an initial geometrical imperfection v0(x), that is assumed to be sinusoidal with a maximum value of e0,d at mid-span: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is quite a common choice for the representation of imperfections. When the axial force NEd is applied on the member, an additional deflection v(x) appears on the member. Because of the support conditions, this additional deflection that is associated with instability can be written as: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where  represents the maximum value of the additional deflection (at mid-span). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Simply supported member with initial imperfection |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In this case, the classical elastic flexural equilibrium equation, accounting for the initial imperfection, becomes: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where I represents the second moment of area in the plane of bending. Replacing Eqs. and in Eq. allows the determination of the expression of  as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where Ncr is the critical flexural buckling load: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Then, the total deflection at mid-span may be expressed as: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As can be seen, the maximum deflection vmax depends on the value of the axial compression through the well-known amplification factor K: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A second-order in-plane elastic check of the most heavily loaded cross-section on the member (at mid-span) then becomes: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where e0,d is denoted as the equivalent geometrical imperfection. When the axial force increases until collapse, NEd is just equal to Nb,Rd =  NRd (see Lecture 10), and the left-hand side of Eq. is just equal to 0. This allows to determine the value of e0,d as a function of : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When the member is subject to additional first order moments, the format of Eq. can be extended as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where represents the second-order maximum bending moment induced by the additional first order bending moment. Indeed, because Eq. represents a second-order cross-sectional check of the most heavily loaded section, it becomes necessary to determine its location, in order to evaluate . When a first order bending moment MEd exists, an additional lever arm to the axial force NEd arises, causing an amplification of the deflection and of the bending moment, in the same way as for the initial imperfection e0,d (cf. Figure 2). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Actual 2nd order bending moment and the associated sinusoidal equivalent |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In order to avoid the determination of the location of the cross-section most heavily loaded by second-order effects, the equivalent moment concept is used. This consists of replacing the actual first order bending system on the member already subjected to the same axial force by a sinusoidal first order bending moment (the equivalent one, cf. Figure 2), that produces the same amplified bending moment. The latter is usually expressed as Cm MEd. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The second-order maximum bending moment can then expressed as: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| using the same amplification factor as for buckling (cf. Eq. ). Then, an elastic second-order check of the most loaded cross-section on the member can be expressed as: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| the primary bending moment being induced by end moments and/or transverse loading. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The proposed format of Eq. is absolutely general, and has been used in many codes. Another way of writing this is: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The latter format appears to be more convenient, because it allows the classical buckling term to appear separately, and has then been preferred in the formulae. Nevertheless, it does not completely isolate the terms depending on the axial compression NEd. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Extension to spatial behaviour and elastic-plastic behaviour |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Member under NEd + My,Ed + Mz,Ed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When biaxial bending occurs, the axial force amplifies the moments about both of the principal axes y – y and z – z of the cross-section. This results in a complex coupling between instabilities in both principal planes. However, this coupling is generally disregarded in codes for practical reasons, and because unsafe cases are very rare. At this point, the proposed formulae diverge slightly from a strict theoretical approach. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Accordingly, biaxial bending is accounted for using a doublet of formulae, obtained by simply adding a second bending term in an Eq. -type formula to give: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From a physical point of view, Eqs. and respectively check the member resistance about y – y and z – z axes, assuming that no instability interaction between principal planes occurs, as described earlier. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For continuity aspects, it is desirable that Eqs. and should reduce to well-known criteria when the loading becomes more simple. It is obvious here that the particular case of buckling is covered, for both principal planes. In the case of NEd = 0, Eqs. and reduce to: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| because Cmy and Cmz tends to 1,0 (cf. section 5.2.2 for example). Eq. is the general biaxial bending criterion for linear elastic analysis of the cross-section. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Elastic-plastic behaviour |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Now that the general elastic format has been established for both plane and spatial behaviour, it becomes possible to extend it to elastic-plastic behaviour but this section is limited to Class 1 and 2 cross-sections. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The generalisation from a theoretical elastic format to an elastic-plastic one implies that several elastic concepts such as the amplification factor, equivalent moment or buckling length concepts are supposed to be still applicable in the inelastic range. In addition, it should be noted here that the extension to elastic-plastic behaviour is not treated here in a fully theoretical manner, because no simple mechanical model seems to be available. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In principle, all types of cross-sections may be able to develop a certain amount of plastic behaviour, due to the non-linear ductile behaviour of steel; this depends on the cross-section geometry. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presently, EN 1993–1–1 adopts a concept of classes for cross-sections: Class 1 and 2 sections are assumed to exhibit a plastic resistance, while Class 3 and 4 are restricted to elastic behaviour, eventually penalised by local buckling or shear lag effects for Class 4 sections that limit their resistance to a so-called “effective” one. This concept still applies for member resistance, though its appropriateness may be much more disputable. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Besides this, the fact that member behaviour is influenced by member instability has to be taken into account when dealing with elastic-plastic effects: the higher the slenderness, the lower the capacity and the less pronounced is the plastic behaviour. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section 4.2.1 gives a general background how the way to incorporate these effects in the elastic in-plane formula of section 4.1, while section 4.2.2 extends these principles to members under three dimensional loading. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Member under NEd + MEd |  |  |  |  |  |  |  |  |  |  |  |  |  |
| To allow yielding along the member, the general format of Eq. is kept, but with C Mpl,Rd replacing Mel,Rd |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Indeed, as explained above, the full plastic bending resistance Mpl,Rd may not be reached but only an intermediate elastic-plastic value given by C Mpl,Rd because of instability effects. Then, factor C aims to take account of the plasticity effects in the interaction between mono-axial bending and axial force along the member. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This C factor must obviously depend on the axial force, member slenderness and bending moment distribution, because all of these factors play a role on the extent of yielding. C should however tend to unity when the axial force is small enough for pure bending to govern the member resistance. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When the member length tends to zero, Eq. tends to: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| because the amplification factor vanishes, and  =  = 1. Eq. must be compared with Eurocode 3 plane N – M cross-sectional criterion. As a consequence, C has to be such that Eq. will produce a good approximation to Eurocode 3 cross-section checks (illustrated in Figure 3). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Eurocode 3 N – M cross-section resistance interaction criterion |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In addition, C should tend to the inverse of the shape factor when the member slenderness is important, in order for C Mpl,Rd to be equal to Mel,Rd. Indeed, for such cases, the behaviour of the member is almost elastic. Then, the general format of factor C should be: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Extension to beam-columns subjected to NEd + My,Ed + Mz,Ed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When a member is subjected to both biaxial bending and axial compression, the general format given in section 4.1, based on elastic second-order theory, is only valid for Class 3 cross-sections. When sections of Class 1 or 2 are of concern, the inelastic effects on the member resistance can be accounted for by means of the following expressions: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eqs. and are then able to deal with inelastic effects for N – M elastic-plastic member interaction. It is however necessary to introduce indices in these C factors, Cyy and Czz dealing with the N – M plasticity effects when the bending plane is the same as the buckling plane, while the factors Cyz and Czy concern the plane perpendicular to the plane of buckling. General expressions for Cii and Cij factors must necessarily be different, because of the smaller effects of plasticity on the M – N interaction when the plane of bending and the plane considered for buckling are not coincident. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When the axial force NEd is negligible or when the member length is very small (stocky members), then the influence of instability vanishes. The problem then reduces to one of pure resistance, and the cross-section My,Ed – Mz,Ed interaction directly governs the response of the beam. Eqs. and then reduce to: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| while, according to ENV 1993–1–1, the cross-sectional resistance to biaxial bending is given as: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where Mpl,y,Rd and Mpl,z,Rd are the design bending resistances about respectively y – y and z – z.  and  are factors that depend on the cross-sectional shape, and whose values are at least equal to 1 in order to take the benefits of plasticity effects. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * and * should then be factors that allow account to be taken of the material non-linear behaviour in the same way as  and . |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The bi-linear criterion of Eqs. and may allow quite a good approximation of the convex criterion of Eq. while keeping a linear biaxial bending interaction format for the formulae. Figure 4 gives a geometrical interpretation of this approximation. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Biaxial bending interaction |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Design formulae in EN 1993–1–1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Format of presentation of the design rules in EN 1993–1–1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EN 1993–1–1 proposes two alternative methods for the design of beam-columns: Method 1 is an accurate fully-theoretically derived set of formulae, while Method 2, using the same theoretical aspects, proposes a simpler and more user-friendly set. EN 1993–1–1 uses a general format for the interaction formulae, with global factors relating to either Method 1 or 2. These factors are given in Annexes A or B accordingly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The general format of the interaction formulae for bending and axial compression of a member not prone to torsional deformations is as follows: |  |  |  |  |  |  |  |  |  |  | EN 1993-1-1 6.3.3(4) |  |  |
| where: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - NEd, My,Ed and Mz,Ed are the design values of the compression force and the maximum moments about the y – y and z – z axis along the member, respectively; -  My,Ed and  Mz,Ed are the moments due to the shift of the centroidal axis for Class 4 cross-sections, cf. Table 1; - y and z are the reduction factors due to flexural buckling; - kyy, kyz, kzy and kzz are the interaction factors. They are relative to either Method 1 or 2 (cf. sections 5.2 and 5.3, respectively). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| It is also important to notice that besides this stability check, the resistance check of the member end sections has to be carried out. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Values for NRk = Ai fy, Mi,Rk = Wi fy and  Mi,Ed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class; 1; 2; 3; 4. Ai; A; A; A; Aeff. Wy; Wpl,y; Wpl,y; Wel,y; Weff,y. Wz; Wpl,z; Wpl,z; Wel,z; Weff,z.  My,Ed; 0; 0; 0; eN,y NEd.  Mz,Ed; 0; 0; 0; eN,z NEd. |  |  |  |
| The terms given in EN 1993–1–1 by  My,Ed and  Mz,Ed do not occur for doubly-symmetric cross-sections, since eN,y and eN,z are zero. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - “Method 1” set of design formulae |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Factors of EN 1993–1–1 and more appropriate format |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Table 2 summarizes the expressions of the kij factors of Eqs. and : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Interaction factors kij for Method 1 |  |  |  |  |  |  |  |  |  |  | EN 1993-1-1 Annex A |  |  |
| Class 3 or 4 cross-sections; Class 1 or 2 cross-sections. |  |  |  |
| In Table 2, factors y, z, wy and wz must be calculated as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Class 3 and 4 cross-sections, it is necessary to use wy = wz = 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Cii and Cij factors of Table 2 must be calculated as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In Eqs. to , represents the highest slenderness of the member, i. e. or . |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The equivalent moment factors Cmi must be calculated according to Table 3: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Equivalent moment factors Cmi |  |  |  |  |  |  |  |  |  |  | EN 1993-1-1 Annex A |  |  |
| Moment diagram; Equivalent moment factor. General case; Mi,Ed (x) is the maximum moment My,Ed or Mz,Ed for Cmy and Cmz respectively  is the associated maximum member displacement along the member (i.e. respectively z and y). |  |
| This set of formulae is provided by the Annex A of Eurocode 3 (Method 1) for members prevented against lateral torsional buckling or sections for which It > Iy, i.e. usual hollow sections for construction. For the rare situations where lateral torsional buckling might be relevant, reference should be made to the dedicated formulae of Annex A of Eurocode 3. It is to be noted that the recommendations from CIDECT [2] to determine if the member is prone or not to lateral torsional buckling are somewhat different (note, however, that the recommendations from CIDECT are relative to “bending-only” situations while Annex A relates to “bending and compression” cases). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The general format of Method 1 presented in EN 1993–1–1 can be rearranged as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This set of formulae is exactly equivalent to the one provided by the combined Eqs. , and Table 2, but allows for a much clear understanding of the effects covered and the meaning of each factor. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cm factor |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As explained in section 3, when regarding second-order effects, the determination of the location of the most heavily loaded section often appears to be very complicated if the member is a beam-column. Therefore, it is usual to resort to the equivalent moment concept. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This Cm factor is probably the most important one in beam-column problems, and the quality of the expression for Cm directly governs the accuracy of the interaction formulae. As a consequence, numerous approximations have been proposed by many researchers covering several loading conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The general expression of Cm for equivalence with a sinusoidal bending moment is derived by rearranging Eq. : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where represents the actual first order maximum value of the bending moment and the second-order one. Obviously, the values of Cm are dependent of the axial force NEd. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When the bending on the member is due to transverse loads or to end moments with transverse loads, it appears difficult to propose exact theoretical expressions for Cm, because the second-order bending moment diagram shape can be very variable. Nevertheless, it is possible to resort to the following expression, provided that the beam is in single curvature: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where M0 represents the maximum first order bending moment and v0 the maximum first order deflection. For the practical case of a concentrated load at mid-span, it gives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and for a uniformly distributed load: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As can be seen, the influence of the axial force NEd remains quite small in these particular cases: the reason being that the first order bending moment diagrams are almost sinusoidal – as was assumed in developing Cm. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When a linear first order bending moment is applied to the member with MEd at one end and  MEd at the other end (–1 ≤  ≤ 1), the equivalent moment is such that the theoretical expression for Cm for a sinusoidal equivalent moment is: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eq. is particularly important in practice because bending in beam-columns is often due to beams intersecting with columns, or to axial force applied with eccentricities, causing linearly distributed moments. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Several approximate expressions have been proposed by various authors for this particular case. However, unlike the above they are related to the case of a uniform bending moment. Therefore, usually the Cm factors are called equivalent uniform moment factors. The most common one is probably the one due to Austin [3]: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A more accurate alternative proposed by Campus and Massonnet [4] is written as: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The main advantage of Eqs. and lies in their simple and practical format. Nevertheless, they propose values of Cm that are independent of the axial force NEd which is clearly not true (cf. Eq. ). In addition, it should be noticed here that Eqs. and cannot be directly compared to the theoretical expression of Eq. . Indeed, they are both relative to a constant reference equivalent moment, while Eq. is relative to a sinusoidal one. In the particular case of a constant bending moment, the amplification factor is: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| whilst for a sinusoidal bending moment, the true amplification factor is: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Both expressions give quite similar results, as reported on Figure 5, but the differences become significant when the axial force approaches the critical state. The equivalence between Cm and Cm* is then: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A confusion between Cm* and Cm has often been made in the literature, and Eqs. and have been associated with Eq. amplification factor, mainly because of practical considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Amplification factors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Finally, it must be noted here that in some cases, the axial force may not be big enough for the maximum second-order bending moment to be observed in the span but rather at one end. This arises when NEd is lower than the following limit: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The maximum second-order bending moment then remains at one end. In this case, another expression for Cm is proposed: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As a consequence, resorting to the equivalent moment concept requires the additional end sections cross-section resistance check. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For the important practical case of linearly distributed bending moments, Method 1 proposes the following expression for Cm, which is due to Villette [5], [6]: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As in the theoretical Eq. , expression depends on the NEd / Ncr ratio, which is not the case in most proposals; it is therefore particularly efficient. Figure 6 compares approximation with , and shows quite good agreement between both expressions. The dashed values of do not need to be considered, because they relate to cases where the maximum second-order moment appears on the member end, i.e. NEd is lower than Nlim. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cm coefficient for linear bending moments |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When the member is subjected to transverse loads or transverse loads and end moments, the general expression is kept: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The expressions of Cm given in this subsection correspond to values of Cm,0 in EN 1993–1–1 rules, i.e. for cases where no torsional deformation are likely to occur. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Plasticity factors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In order to develop the elastic theoretical shape to include the inelastic phenomena, the formulae resort to several plasticity coefficients. First, the formulation uses * and * coefficients for biaxial bending interaction. Method 1 gives greater resistance from plastic interaction than the from the elastic criterion by the following values: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with w defined as the ratio between the plastic modulus Wpl and the elastic one Wel: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| w has an upper bound value of 1,5, because this is a maximum value for the range of realistic structural sections and for which it has been verified. * and * coefficients then give the best possible fit with the non-linear interaction of Eq. , for cross-section properties. However, there is a discontinuity between Class 3 and 1 or 2 cross-sections. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In addition, the plasticity coefficients Cii and Cij have to be defined. Method 1 adopts the following expressions: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where represents the highest relative slenderness of the member. As explained in 4.2.1, these C factors allow for the plasticity effects in the interaction between mono-axial bending and axial force. But because of instability effects, i.e. the influence of the slenderness, the axial force NEd plays a role in the extent of yielding at the ultimate limit state, and the beam may not reach the full plastic bending resistance Mpl,Rd. This effect is accounted for in Eqs. and through the presence of and NEd / Npl,Rd. The contribution of bending moments along the member also influences the phenomenon, and this is included in Method 1 by the Cm factors in Eqs. and . |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The form of these factors has been chosen so that they reduce to the cross-section check of a rectangular cross-section when the length of the member tends to zero (cf. section 5.2.4). These Cii and Cij values have to be bounded such as in Eqs. and in order that the bending resistance moment C Mpl,Rd is not to be lower than the elastic one Mel,Rd (cf. section 4.2.1). The format of Eqs. and obviously fulfils the general format of Eq. . |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Finally, it is necessary to stress that the factor (w – 1) in Eqs. and represents the maximum possible increase in bending resistance from pure elasticity to full plasticity; other coefficients in these expressions have been obtained by calibration. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Continuity aspects |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As explained before, because the formulae of Method 1 are intended to be general and to include limiting cases, they must exhibit a minimum of discontinuities, i.e. between the most general format and the one used in simpler loading situations, between member design checks (stability) and section design checks (cross-sectional resistance), and from plastic to elastic behaviour when the slenderness increases. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Firstly, because the general case of Eqs. and is not the most common in practice, the formulae can be reduced to simpler expressions. For the particular case of strong axis bending and axial compression for a member that can exhibit a spatial behaviour, Method 1 gives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eq. appears much more simple than Eq. , because the factor y simplifies with the amplification factor. As Eq. clearly shows, the strong axis bending has an influence on the weak axis buckling. This effect is usually disregarded in most design codes. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Secondly, the proposed interaction formulae should reduce to cross-sectional criteria when the member slenderness tends to zero. As explained in section 1, the objective of the present new interaction formulae is not to propose new criteria for cross-sectional interactions; the Eurocode 3 criteria apply to both Methods 1 and 2. Nevertheless, because of the basic mechanical behaviour itself, the formulae must ensure a good agreement with Eurocode 3 cross-sectional checks when for zero member length. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The way this aspect is treated for My – Mz interaction was shown in section 4.2.2. It is necessary to check if factors Cii and Cij reduce so that the formulae approach the usual N – M cross-section checks. For example, in the particular case of in-plane N – Mz interaction, Eq. simplifies to: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In Eq. , the factor 2 is the theoretical value that leads to the exact N – M interaction for rectangular cross-sections. Eq. leads to an approximate cross-sectional criterion that is represented in Figure 7. As can be seen, the proposed expression gives satisfactory results, compared to the reference Eurocode 3 formula. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - N – M cross-sectional resistance interaction criterions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Finally, the necessary continuity between pure plasticity when the axial force is negligible and pure elastic behaviour when the slenderness is important is ensured, because of the way the proposal has been built. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Members with Class 3 and 4 cross-sections |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Because the formulae are based on elastic 2nd order considerations, the Class 3 and 4 cross-section cases are obviously covered by Method 1 set of formulae. The following expressions apply for Class 3 cross-sections: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where the elastic-plastic bending resistances Cii Mpl,Rd and Cij Mpl,Rd are replaced by the pure elastic ones. In addition, a linear interaction between My,Ed and Mz,Ed must be considered, i.e. * = * = 1. The  values are here calculated on the basis of elastic cross-section properties. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In the particular case of Class 4 cross-sections, the formulae write: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where Meff,Rd represents the effective bending resistance of the cross-section, and eN,y the shift of centroid in the y – y direction due to effective cross-section concept. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - “Method 2” set of design formulae |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Factors of EN 1993–1–1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Interaction factors kij for Method 2 |  |  |  |  |  |  |  |  |  |  | EN 1993-1-1 Annex B |  |  |
| Class 3 or 4 cross sections; Class 1 or 2 cross sections. Option for axial compression and uniaxial bending My,Ed: kzy = 0. |  |
| The equivalent moment factors Cm correspond to members with both ends supported. For members with flexibly restrained ends or even one free end, which may buckle in a sway mode, the Cm factor should be taken equal to 0,9. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Equivalent moment factors Cm |  |  |  |  |  |  |  |  |  |  | EN 1993-1-1 Annex B |  |  |
|  | Moment diagram | Range |  |  |  |  | Cmy and Cmz and CmLT |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Uniform load |  | Concentrated load |  |  |  |  |
|  |  | –1    1 |  |  |  |  | 0,6 + 0,4  0,4 |  |  |  |  |  |  |
|  |  | 0  s  1 |  |  | –1    1 |  | 0,2 + 0,8s  0,4 |  | 0,2 + 0,8s  0,4 |  |  |  |  |
|  |  | –1  s < 0 |  |  | 0    1 |  | 0,1 – 0,8s  0,4 |  | –0,8s  0,4 |  |  |  |  |
|  |  |  |  |  | –1   < 0 |  | 0,1 (1–) – 0,8s  0,4 |  | 0,2(– ) – 0,8s  0,4 |  |  |  |  |
|  |  | 0  h  1 |  |  | –1    1 |  | 0,95 + 0,05h |  | 0,90 + 0,10h |  |  |  |  |
|  |  | –1  h  0 |  |  | 0    1 |  | 0,95 + 0,05h |  | 0,90 + 0,10h |  |  |  |  |
|  |  |  |  |  | –1    0 |  | 0,95 + 0,05h (1 + 2) |  | 0,90 + 0,10h (1 + 2) |  |  |  |  |
|  | For members with sway mode, the equivalent uniform moment factor should be taken Cmy = 0,9 or Cmz = 0,9 respectively. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cmy and Cmz shall be obtained according to the bending moment diagram between the relevant braced points as follows: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | moment factor |  |  | bending axis |  | points braced in direction |  |  |  |  |  |  |  |
|  | Cmy |  |  | y – y |  | z – z |  |  |  |  |  |  |  |
|  | Cmz |  |  | z – z |  | y – y |  |  |  |  |  |  |  |
| - More appropriate format for members with Class 1 and 2 cross-sections |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Axial Compression and strong-axis bending (NEd + My,Ed) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Buckling mode y – y: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Buckling mode z – z: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coefficients: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and: (cf. Table 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The interaction factor ky of Eq. has been determined on the basis of GMNIA-results for different sections and moment diagrams calculated with different values of the parameters and ny and the uniform moment factor Cmy. These calculations were based on extensive parametric studies by limit load calculations in [7]. Graphs for two different sections under uniform moment are shown in Figure 8 [8], [9]. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Numerically derived interaction factor ky for constant moment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The ky-factors clearly illustrate the characteristic physical behaviour of plastic members with increasing slenderness. In the range of , the cross-section interaction causes ky-values below 1,0. Then, with increasing slenderness the second-order effects increase with the axial compression parameter ny, leading to increasing ky-factors. This increase is only partly the same as that resulting from the analytical amplification factor given in Eq. , since a large part of the amplifying effect is already accounted for by using the y-factor in the term for the axial compression. Therefore, the increase in ky stops at about and then should progressively decrease with increasing slenderness (cf. Figure 9c). However, as Figure 8 shows, the ky-factor remains fairly constant in this slenderness range. This is due to the fact, that, in this range, the buckling behaviour tends more and more to elastic behaviour, so that the ky-factor has to compensate also for the difference between My,el and My,pl. This is because My,pl is used in the interaction formula throughout the whole slenderness range. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As illustrated, the interaction factor ky is influenced to a limited amount by the cross-section shape of the member. However, distinct differences occur just in the range of higher values of ny, where the moment-term is low, so that the results are hardly affected. Accordingly, it was not necessary to distinguish between the section-shapes considered here. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The ky-factor calculated from the design formulae is presented in Figure 9a. The graph is bilinear with a plateau starting at , which makes it easy to use in practical work. The comparison with the ky-factor of the previous ENV 1993–1–1 (Figure 9b) shows the more economical effects. The factor has been statistically evaluated [10] with the available test data and the GMNIA-results and is given by Eq. . |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Comparison of interaction factor ky for constant moment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For non-uniform moment diagrams the Cm-factor according to Austin has been used for linear distributions. For different variable moment distributions the Cm-factors are detailed in next paragraph d). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A further view on the typical effect of the ky-factor is presented in Figure 10 for a triangular moment diagram. A comparison is made for the combined factor “Cmy ky” determined by calculation from GMNIA-results for an IPE 400 with that of the new proposal of Eurocode 3 and with the previous formula of the ENV 1993–1–1. The beneficial effect of the moment diagram is obvious in the plots of the GMNIA and the new proposal. The latter also includes the effects of the cross-section check at the end of the member, so that it is comparable with the numerical results. The ENV–formula, however, predicted values higher than one, which is in contrast with the physical behaviour and needed correction for economy. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Numerically derived interaction factors ky for triangular moment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Members susceptible to lateral buckling deformations may generally fail by the in-plane or the out-of-plane buckling mode. Their buckling capacity is predicted by Eqs. and , where each equation describes one of the two failure modes. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For representation by design rules, this spatial behaviour has been split up into two different branches described by the two Eqs. and . The latter is a linear approximation with the meaning that out-of-plane buckling is affected just by 60% of the in-plane bending moment. The value of 60% is conservative, however; its specific magnitude will be explained later in context with biaxial bending. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Standard sections – such as RHS or I- and H-profiles – under N + My, it is proposed to reduce the out-of-plane buckling check to axial compression alone. The reasoning for this simplification is the fact that for open sections like I-and H-profiles, the out-of-plane buckling check is performed by the lateral-torsional buckling check anyway, and that for closed sections like RHS-profiles, the deviations to the unconservative side are small and limited to just a small area of load-configurations N – My in the interaction diagram. This simplified check follows the traditional design practice of many codes, which due to the just small effect of the strong-axis moment on lateral flexural buckling neglect the bending effect completely. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Axial compression and weak-axis bending (NEd + Mz,Ed) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In a way which is analogous to that for strong-axis bending, the interaction formula of Eq. describes the buckling behaviour under N + Mz. The interaction factor kz has been calculated from GMNIA-results and plotted as illustrated in Figure 11. Again, the typical form of the kz-curves appears, with the cross-section shape having only a minor effect. The cross-section interaction in the range of is, understandably, much more pronounced. The rest of the curves are quite similar in shape to the ky-curves. The kz-factor has again been determined as a bilinear graph with the plateau starting at (cf. Figure 11c). The kz-factor is given by Eq. , which is the same as ky. The factor Cmz relates to the shape of the diagram of Mz; it is also defined by the Austin formula Eq. . |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Numerically derived interaction factor kz for constant moment Mz,Ed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Axial Compression plus biaxial bending (NEd + My,Ed + Mz,Ed) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Buckling mode y – y: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Buckling mode z – z: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The interaction formulae follow from extension of the equations for mono-axial bending and compression by the addition of the terms for Mz. For the z – z buckling mode the full value of the term of Mz according to Eq. has been added, while for the y – y mode the term has been reduced by the factor 0,6. This results in Eqs. and , which have the following characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - They are “symmetrical” in format concerning the axes y and z, which is significant for the transition of RHS to SHS; - They allow the exploitation of the plastic cross-section interaction of bi-linear bending with qualitatively good approximation through the use of the factors 0,6 with the two bending terms. As NEd → 0 the equations become: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Equivalent uniform moment factors Cm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The equivalent uniform moment factor Cm has been derived by calculation from GMNIA-results for members with different varying moment diagrams compared with uniform moments [9]. Accordingly, the effects of plasticity and imperfections are included. Understandably, the calculation is influenced by a number of parameters, which makes the presentation in design rules quite complex. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Given the objective of user-friendly design rules, the Cm-factors have been presented in simple form, even if conservative values had to be accepted in some cases. They are given in Table 5 by formulae. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For members under linearly varying moment diagrams the Cm-factor has been described by the so-called Austin-formula, which has been widely used in international codes before. The validity of this formula has been recently checked and justified in [11] for a wide range of parametric cases. The formula describes a bi-linear shape, limited by a lower bound of 0,4, which in comparison with numerical calculations has turned out as quite conservative in cases of bi-triangular moment diagrams. Nevertheless, it was found sensible to not exploit the remaining capacity in these cases. So, the limitation by 0,4 has been kept. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The uniform moment factors discussed so far have been related to members with both ends simply supported. In case of members with one end unrestrained, so that a sway buckling mode may occur, these values are not valid and should be replaced by the values Cm = 0,9, which is a good approximation to the factor for such members. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Finally, a general remark should be made that all the Cm-factors have been related to the maximum value of bending moment My or Mz in the member considered. This means that My and Mz have to be introduced into the interaction formulae as their maximum values, even if the maximum values do not occur in the same place. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Members with Class 3 and 4 cross-sections |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Buckling mode y – y: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Buckling mode z – z: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The coefficients , , ny, nz, Cmy, Cmz are identical with those in sections 5.3.1 and 5.3.2 above. For Class 4 sections the section properties Npl,Rd and Mel,y,Rd, Mel,z,Rd need to be replaced by the resistances calculated from the properties of the effective section Aeff and Weff. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The design formulae and are closely related to the analytical derivation given in section 4.1. They are based on the linear interaction of the internal section forces. They further use interaction factors ky and kz, which are bi-linear approximations of the theoretical elastic interaction factors and which – due to this bilinear simplification – are somewhat conservative in the range of higher slenderness. |  |  |  |  |  |  |  |  |  |  |  |  |  |

- References

- Eurocode 3: EN 1993–1–1: Eurocode 3: Design of Steel Structures Part 1.1: General rules and rules for buildings.

- Rondal J., Würker K.-G., Dutta D., Wardenier J. and Yeomans N. Structural Stability of Hollow Sections, CIDECT, 1992.

- Austin W.J.; Strength and design of steel beam-columns. Proceedings of ASCE, Journal of the Structural Division, Vol. 87, N° ST4, April 1981.

- Byfield M.P., Nethercot D.A.; An analysis of the true bending strength of steel beams, Proceedings Inst. Civ. Engrs. Structs. & Bldgs.; 1998, 128, May, pp. 188-197.

- Villette M. Boissonnade N., Muzeau J.P., Jaspart J.P.; Development of a comprehensive formula for the design of beam-columns. Internal report, Baudin-Châteauneuf, LERMES-CUST, University of Liege, 2000.

- Villette M. Analyse critique du traitement de la barre comprimée et fléchie et propositions de nouvelles formulations. PhD Thesis, Faculté des Sciences Appliquées, Université de Liège, 2004.

- Ofner R.; Traglasten Von Stäben aus Stahl bei Druck und Biegung, PhD Thesis, Institut für Stahlbau, Holzbau und Fläschentragwerke, TU Graz, Heft 9, 1997.

- Greiner R., Ofner R., Salzgeber G.; Flexural buckling of beam-columns, ECCS-Validation Group, Report 3, October 1998.

- Greiner R., Ofner R., Salzgeber G.; LT-buckling of beam-columns, ECCS-Validation Group, Report 5, September 1999.

- Lindner J., Gietzelt R.; Zweiachsige Biegung mit Längskraft - Ein ergänzter Bemessungsvorschlag (biaxial bending and compression – a completed design concept), Stahlbau 54 (1985), pp. 265-272.

- Kaim P.; Spatial buckling behaviour of steel members under bending and compression, PhD-thesis, Institute for Steel, Timber and Shell Structures, Graz University of Technology, H. 12, 2004.

- Trahair N.S. and Bradford M.A., “The Behaviour and Design of Steel Structures”, E&F Spoon, 1994.