- Module 2

Structural analysis and design

Lecture 4Structural idealisation and analysis

- Contents:

1.	Introduction to frame behaviour

1.1.	Scope

1.2.	Load displacement relationship of frames

2.	Frame idealisation and basic concepts of structural analysis

2.1.	Modelling of building structures for analysis

2.1.1.	Structural concept

2.1.2.	Spatial behaviour

2.1.3.	Resistance to horizontal forces

2.1.4.	Ground-structure interaction

2.1.5.	Modelling of frames

2.1.6.	Joint modelling

2.2.	Modelling of lattice girders for analysis

2.3.	Allowance for imperfections

2.4.	Structural components

2.4.1.	Members in bending

2.4.2.	Members in tension and members in tension and bending

2.4.3.	Members in compression

2.4.4.	Members in bending and compression

2.4.5.	Joints

2.5.	Basic concepts of structural analysis

3.	Methods of global structural analysis

3.1.	General

3.2.	Second-order effects

3.3.	Slope-deflection method

3.3.1.	First-order elastic analysis

3.3.2.	Second-order analysis

3.3.3.	Equivalent lateral load approach for second-order analysis

3.3.4.	Modified slope-deflection method

4.	Global elastic structural analysis

4.1.	First-order theory

4.1.1.	Assumptions, limitations, section and joint requirements

4.1.2.	Structural analysis

4.1.3.	Structural design

4.2.	Second-order theory

4.2.1.	Assumptions, limitations, section and joint requirements

4.2.2.	Structural analysis

4.2.3.	Structural design

5.	Global plastic structural analysis

5.1.	Introduction

5.2.	Elastic-perfectly plastic analysis (second-order theory)

5.2.1.	Assumptions, limitations, section and joint requirements

5.2.2.	Structural analysis and design

5.3.	Elastic-plastic non-linear analysis (second-order theory)

5.3.1.	Assumptions, limitations, section and joint requirements

5.4.	Rigid-plastic analysis (first-order theory)

5.4.1.	Assumptions, limitations, section and joint requirements

5.4.2.	Structural analysis

5.4.3.	Structural design

6.	References

| - Introduction to frame behaviour |  |
| --- | --- |
| - Scope |  |
| Global frame analysis aims at determining the distribution of the internal forces and the corresponding deformations in a structure subjected to a specified loading. | Part 1-1 5 |
| Achieving this purpose requires the respect of design requirements and the adoption of adequate models which incorporate assumptions about the behaviour of the structure and in particular of its components (members and joints). The main purpose of this lecture is to address the question of modelling and analysis of structural behaviour for practical design purposes. | Part 1-1 2 |
| - Load displacement relationship of frames |  |
| The response of a structure to loading applied to it can be expressed by the relationship between a load parameter and a significant displacement parameter. An example of the behaviour of a typical sway frame under increasing load is shown in Figure 1. |  |
| The load parameter  is most often a multiplier (or load factor) applied to all the load components so as to produce monotonic and proportional increase in the entire loading on the structure, while the displacement parameter is taken here as the lateral displacement at the top floor level. The resulting curve can be considered to characterise the overall structural behaviour. In this case, the slope of the curve is a measure of the lateral stiffness of the frame structure. |  |
| - Load-displacement response of a framed structure |  |
| One observes that the response of the structure is quasi-linear up to a certain point (the linear limit). Once the linear limit is reached, the positive slope of the rising part of the curve gradually reduces due to a combination of three kinds of non-linearity: geometrical non-linearity, joint non-linearity and material non-linearity. Joint non-linearity usually manifests itself at relatively low levels of load. Geometrical non-linearity expresses the influence of the actual deformed shape of the structure on the distribution of the internal forces. Typically it becomes evident well before the onset of material yielding, i.e. material non-linearity. Beyond the latter, the response becomes progressively non-linear as the load increases up to a maximum. Once the maximum load is reached, equilibrium would require a decrease in the magnitude of the loads as deformations increase. |  |
| The slope of the curve (i.e. the stiffness) is zero at the peak load and then it becomes negative indicating that the structure is henceforward unstable. The peak load, often termed the ultimate load, is the point of imminent structural collapse in the absence of the possibility of load shedding. |  |
| - Frame idealisation and basic concepts of structural analysis | Part 1-1 5.1 |
| - Modelling of building structures for analysis |  |
| Global analysis of frames is conducted on a model based on many assumptions including those for the structural model, the geometric behaviour of the structure and of its members and the behaviour of the sections and of the joints. | Part 1-1 5.1.1 |
| Once the analysis is achieved, a number of design checks of the frame and its components (members and joints) must be performed. These checks depend on the type of analysis performed and the type of cross-section verification (i.e. ultimate limit state criteria) used. |  |
| Guidelines for the simplified modelling of building structures for analysis, and in some cases for the verification of individual members, were given in the informative Annex H [1] of the second amendment to Eurocode 3 ENV, Part 1-1. Simplified models for buildings subjected to predominantly static loading were proposed which may be adopted as alternatives to more sophisticated models. It was not covering methods intended either for seismic design nor for “stressed skin” design which were dealt with in other specific Eurocodes [2], [3]. | ENV 1993 Annex H |
| While the essential aspects of frame modelling are cited in the lectures of this module, reference to the relevant parts of Eurocode 3 Part 1-1 and to Annex H should be made by the designer. |  |
| - Structural concept |  |
| The layout of the structure should be based on the requirements for the intended use of the building, including resistance to the actions that are likely to occur. |  |
| One is required to identify the following categories of structural elements: | ENV 1993 Annex H |
| - Main structural elements: including main frames, their joints and their foundations that form the routes by which vertical and horizontal forces acting on the building are transferred to the ground; - Secondary structural elements: such as secondary beams or purlins, that transfer loads to the main structural elements; - Other elements: elements that only transfer loads to the main or secondary elements. Examples are sheeting, roofing and partitions. |  |
| In cases where these three categories of elements are subject to different safety requirements, they should be modelled separately, if necessary. |  |
| - Spatial behaviour |  |
| As an alternative to analysing the main structure as a one three-dimensional framework, it may be analysed as two series of independent plane frames running in two horizontal directions approximately at right angles to each other, see Figure 2, provided each such plane frame has sufficient out-of-plane restraint to ensure its lateral stability. | ENV 1993 Annex H |
| - Reduction of a three-dimensional framework to plane frames |  |
| - Resistance to horizontal forces |  |
| This aspect is treated in Lecture 5 on “Frame classification and joint representation” where the Braced/ Unbraced and Sway/ Non-sway classifications are explained. |  |
| When the configuration of the structure is such that the building is sensitive to possible eccentricity of horizontal loading, relative to the centre of resistance to twisting of the structure, the effects of applying only part of the horizontal loading should also be taken into account. | ENV 1993 Annex H 5.2.4.3(5) |
| - Ground-structure interaction | Part 1-1 5.1.3 |
| Whether ground-structure interaction should be accounted for or not depends on the significance of the effects on the internal forces and moments in the structural elements of the foundation settlements resulting from the loading on the ground. | ENV 1993 Annex H |
| The following procedure is proposed in Annex H of ENV 1993 for examining ground-structure interaction: |  |
| - As a first step, the structure may be analysed assuming that the ground is rigid. From this analysis, the loading on the ground should be determined and the resulting settlements should be calculated; - The resulting settlements are applied to the structure in the form of imposed deformations and the effects on the internal forces and moments should be evaluated; - When the effects are significant, the ground-structure interaction should be accounted for. This may be done by using equivalent springs to model the soil behaviour. |  |
| No criteria are given in Annex H or elsewhere in Eurocode 3 to decide on the significance of the ground-structure interaction. It is suggested here that when they do not reduce the resistance of the structure by more than 5%, they can be considered to be insignificant and can be neglected in design. This criteria is the same criteria as that given for the classification of rigid joints. |  |
| - Modelling of frames |  |
| The following guidelines are taken from Annex H of ENV 1993. | ENV 1993 Annex H |
| - The members and joints should be modeled for global analysis in a way that appropriately reflects their expected behaviour under the relevant loading; - The basic geometry of a frame should be represented by the centrelines of the members; - It is normally sufficient to represent the members by linear structural elements located at their centrelines, disregarding the overlapping of the actual widths of the members; - Alternatively, account may be taken of the actual width of all or some of the members at the joints between members. Methods which may be used to achieve this are proposed in Annex H. | 2.3.1(4) |
| - Joint modelling |  |
| In Eurocode 3, the expression joint modelling is used to distinguish between the various ways that joint behaviour can be considered for global analysis. | Part 1-1 5.1.2 |
| It is recognised that in general, due to joint deformations and/or lack of resistance, deflected shapes of members will be discontinuous at the joints. Depending upon the effects of this discontinuity, a distinction can be made between the following cases: |  |
| - The discontinuity may be neglected and the behaviour of the joint has no effect on the analysis, i.e. the joints are assumed to be rigid and/or full strength, and the frame may be analysed as continuous. This is called continuous joint modelling; - The discontinuity may be taken into account by assuming a pinned (hinged) joint model, taking advantage of possible rotations without considering joint moment resistances. This is called simple joint modelling; - The discontinuity at the joints may also be accounted for by using semi-continuous joint modelling. This is where a joint model (i.e. a semi-rigid and/or partial strength joint) is used in which its moment-rotation behaviour has to be more precisely taken into account in the structural analysis. |  |
| The use of a continuous and of a simple type of joint must be justified by an appropriate choice of joint class (“rigid/full strength” and “simple” joints respectively). This aspect of joint classification will be addressed in Lecture 5 on “Frame classification and joint representation”. |  |
| Whilst it is likely that for the analysis of many typical frames, a choice of only one of these joint modelling possibilities will be made throughout the frame for the beam to column joints, the use of different joint modelling types for various parts of a given frame can be envisaged. |  |
| Joint modelling is introduced in Lecture 5 on “Frame classification and joint representation”. |  |
| - Modelling of lattice girders for analysis |  |
| - The distribution of axial forces in a lattice girder may be determined on the assumption that the brace and chord members are connected by pinned joints (simple joint modelling) even though the external members (chord members) are physically continuous. | Part 1-8 5.1.5(2) |
| - Secondary moments at the joints, caused by the rotational stiffness of the joints, may be neglected both in the design of the members and in the design of the joints provided the following conditions are fulfilled: | Part 1-8 5.1.5(3) |
| - The joint geometry is within the range of validity specified in Lectures 12 to 14; - The ratio of the system length to the depth of the member in the plane of the lattice girder is not less than an appropriate minimum value. For building frames, the appropriate minimum value may be assumed to be 6. Larger values may apply in other types of structures. |  |
| - The moments resulting from transverse loads (whether in-plane or out-of-plane) that are applied between panel points should be taken in the design of the members to which they are applied. Provided that the here above specified conditions are satisfied, | Part 1-8 5.1.5(4) |
| - The brace members may be considered as pin-connected to the chords, so moments resulting from transverse loads applied to chord members need not to be distributed into brace members, and vice-versa; - The chords may be considered as continuous beams, with simple support at panel joints. |  |
| - The detailing of the joints may lead to eccentricities between the axes of the connected members. These eccentricities result in bending moments in the members and in the joints. This basically requires to model the joint in a way that allows for this. Simplified methods that allow to consider the effects of such eccentricities in the analysis are however described in Eurocode 3 Part 1-8 [14]. | Part 1-8 |
| - Moments resulting from eccentricities may be neglected in the design of the tension chord members and brace members. They may also be neglected in the design of joints, if the eccentricities are within the following limits: | Part 1-8 5.1.5(5) |
| where: e is the eccentricity defined in Figure 3; d0 is the diameter of the chord; h0 is the depth of the chord, in the plane of the lattice girder. |  |
| - When the eccentricities are within these limits, the moments resulting from the eccentricities should be taken into account in the design of the compression chord members. In this case the moments produced by the eccentricities should be distributed between the compression chord members on each side of the joint, on the basis of their relative stiffness I / L, where L is the system length of the member, measured between panel points. | Part 1-8 5.1.5(6) |
| - When the eccentricities are outside these limits, the moments resulting from the eccentricities should be taken into account in the design of the joints and of the compression chord members. In this case, the moments produced by the eccentricity should be distributed between all the members meeting at the joint, again on the basis of their relative stiffness I / L. | Part 1-8 5.1.5(7) |
| - Eccentricity of joints |  |
| - The stresses in a chord resulting from such moments should also be considered in the design of the joints by determining the factors km , kn and kp. | Part 1-8 5.1.5(8) |
| - Table 1 summarises the cases where moments should be considered. |  |
| - Allowance for bending moments - Type of component; - Source of the bending moment. - Secondary effects; - Transverse loading; - Eccentricity. - Compression chord; - Not if the conditions from Part 1-8 5.1.5(3) specified above are satisfied; - Yes; - Yes. - Tension chord; - No. - Brace member; - No. - Joint; - Not if the conditions from Part 1-8 5.1.5(5) specified above are satisfied. |  |
| - Allowance for imperfections |  |
| Appropriate allowance must be made to account for the effects of practical imperfections in the global analysis, in the analysis of bracing systems and in member design. | Part 1-1 5.3.1 |
| - Practical imperfections, which include residual stresses, are geometrical imperfections such as lack of verticality, lack of straightness, lack of flatness, lack of fit and unavoidable eccentricities present in joints of the non-loaded structure. |  |
| Allowance for these imperfections may be achieved by incorporating suitable geometric imperfections with values which reflect all types of imperfections. Eurocode 3 Part 1-1 distinguishes two kinds of imperfections: |  |
| - Global imperfections for frames and bracing (often called sway imperfections); | Part 1-1 5.3.2 & 5.3.3 |
| - Local imperfections for individual members. | Part 1-1 5.3.4 |
| The effects of member imperfections may sometimes be neglected when carrying out the structural analysis. These aspects, as well as the values of the imperfections themselves, are treated in Lecture 5 on “Frame classification and joint representation”. |  |
| - Structural components |  |
| A structure is composed of members and joints (see Figure 4 where two typical structures are shown as examples). Members are elements which are much longer than they are deep, and joints are the zones where two or more members are connected. Members are classified according to the kind of loading they sustain. So, one speaks about members in bending (beams), members in axial compression or tension (columns, chords and braces) and members in bending and axial compression or tension (beam-columns, chords and braces). |  |
| - For typical structures it is usual to dispose the elements so as to avoid the creation of significant torsion forces. In that case, the torsion forces, while never entirely absent, can be neglected in the global analysis. For those cases where significant torsion can be expected, appropriate means should be used to determine the torsion in the affected members (if it has not already been allowed for in the global analysis) and their design should take it into account. |  |
| Joint Chord Brace a – Building frame b – Lattice girder - Structure and structural components |  |
| In the present lecture notes, members with uniform cross-sections along the member length are only considered. Furthermore, members with built-up sections are also not covered as their application in tubular construction is not common. Reference to Eurocode 3 will have therefore to be made for laced, battened or closely spaced members. | Pat 1-1 6.4 |
| - Members in bending |  |
| Most beams are designed to carry loads which produce bending about the major principal axis of the cross section only. Commonly called in-plane bending, this is usually the governing design condition for beams in frames. |  |
| Because unavoidable initial imperfections in the beam geometry and unintentional small eccentricity of the loads on the beam occur, some torsion will be always present in the case of in-plane bending. Under increasing loads, the out-of-plane deformations can become magnified to such an extent that the usefulness limit of the beam may be reached. Failure is then said to occur due to lateral-torsional buckling. For most typical frames this type of failure can usually be avoided by providing adequate lateral restraint to the compression flange (restraint provided in the plane of the floor or by the floor slab itself for instance). |  |
| Beams are made up of plate elements, which may sometimes be sensitive to local buckling. Local buckling in combination with lateral-torsional buckling may be the cause of failure of some steel beams in a structure. Bi-axial bending occurs when there is bending in the member section about both the major and the minor axes simultaneously. As for the in-plane bending case, some torsion is unavoidably present. However bi-axial bending in beams is rare and torsion can usually be ignored in members of most typical frames. The effects of plasticity in the cross section, when it occurs, can accelerate the effects enumerated above. |  |
| Besides that, other failure modes may occur in members with cross-sections characterised by rather slender constitutive walls: |  |
| - Shear buckling of the member web(s); - Flange-induced buckling or; - Web crippling. |  |
| Design formulae allowing to check the resistance and/or the stability of members in bending will be discussed in Lecture 7 on “Local buckling and cross-section classification” and in Lecture 9 on “Members in bending and shear”. | Part 1-1 5.5, 6.2.5 & 6.3.2 |
| - Members in tension and members in tension and bending |  |
| No member instability is likely to develop in such situations and therefore only a lack of resistance of the most highly loaded cross-section along the member length may cause failure. |  |
| The design resistance of tension members is so based on yield in the gross section and/or on rupture in the net section. Special attention has to be paid to where ductile behaviour is required. Inclined guys, cables and slender tie-rods may need special analytical treatment as their tension behaviour, which is strongly influenced by sag due to self-weight, is non-linear. These elements are not covered by the present lecture notes. |  |
| Design formulae allowing to check the resistance of cross-sections in members subject to tension or tension and bending will be presented in Lecture 11 on “Members in bending and axial force”. | Part 1-1 6.2.3 & 6.2.9 |
| - Members in compression |  |
| - Members subject to axial compression, such as columns, chords or braces, may be classified by length. Whilst a short compression member (post or pedestal) fails by crushing or squashing, a long or slender compression member fails by buckling instability. The squash load occurs where there is full yielding in compression throughout the cross-section of the member. The failure load of any compression member that fails by buckling depends on its slenderness and, as a consequence, the buckling load of the member can be significantly lower than its squash load. |  |
| - While residual stresses have little effect on the squash load, they reduce the buckling load. Load eccentricities and member lack-of-straightness reduce the failure load below the squash load and reduce the buckling load. |  |
| Buckling resistance formulae in Eurocode 3 for nominally straight members contain allowances for unavoidable initial member curvature (i.e. lack-of-straightness) and for residual stresses present after manufacture and/or fabrication. |  |
| Design formulae allowing to check the resistance and/or the stability of members in compression will be discussed in Lecture 7 on “Local buckling and section classification” and in Lecture 10 on “Members in compression”. | Part 1-1 5.5, 6.2.4 & 6.3.1 |
| - Members in bending and compression |  |
| Members subject to both significant bending and axial compression are often met in steel construction. A member in bending represents the limiting case where the axial forces can be disregarded and a member in compression the case where bending moments are not significant. The failure of this element may therefore be due to any of the failure modes described earlier, but also to a combination of some of these. |  |
| - Design formulae allowing to check the resistance and/or the stability of members in compression and bending will be discussed in Lecture 7 on “Local buckling and cross-section classification” and in Lecture 11 on “Members in bending and axial force”. | Part 1-1 5.5, 6.2.4, 6.2.5, 6.2.9 & 6.3.3 |
| - Joints |  |
| Historically it has been common practice to assume joints to be either rigid or pinned. Whilst this is still usual today, it is now possible to model joint behaviour more precisely (see Lecture 5 on “Frame classification and joint representation” and Module 4 on “Design of joints”). |  |
| The main forces that a joint between two members must transmit are the shear force and/or the axial force and, when the joint is not pinned, the in-plane bending moment. In frames, the axial forces are usually significant for columns and beam-column members, but usually not for beams. In lattice girders, there are predominant. It has to be noted that any out-of-plane moment and/or out-of-plane shear force as well as any torsion in the connected members must also be transmitted, if relevant. |  |
| In Module 4 on “Design of joints”, more detailed information about the design of structural joints will be provided. | Part 1-8 |
| - Basic concepts of structural analysis |  |
| The primary step in establishing a model of the structure to be analysed is to define the configuration. As the components making up typical structures are mostly made up of straight members between connection points (joints), the geometry of the structure is usually defined by the independent co-ordinates of the joints. |  |
| From these essential pieces of geometric information, the degrees of freedom of the structure are established. To complete this aspect, some additional information must be provided on the relaxation of constraints at particular locations in the structure. These are mostly related to the idealisation of the supports conditions and possibly of the behaviour at some other locations (usually joints) in the structure where relative movement (for instance sway) or rotation (for instance at pinned joints) is allowed. |  |
| - The degrees of freedom, usually defined as the displacements and rotations of the joints, are used in the analysis model to describe the deformed shape of the structure under applied loads. When the joint components are modelled as separate elements, this usually results in an increase of the number of degrees of freedom. |  |
| - Once the geometry of the structure has been fixed, it is necessary to define the material properties (i.e. material behavioural laws for the steel chosen) and the character of the loads. By preliminary design, the member cross-section properties and – if needed - the joint properties are then established. | Part 1-1 3 & 2.3.1 |
| - The solution to any structural analysis problem requires that the structural variables (forces, deformations) satisfy three basic principles or physical laws, which are: |  |
| - Equilibrium: the member and joint forces and the externally applied loads must satisfy the equations of static (or dynamic) equilibrium throughout the structure; - Compatibility: the member deformations must be geometrically compatible with the joint (including the supports) displacements and rotations; that is, structural continuity must be preserved throughout the structure; - Constitutive laws: the member and joint forces (stresses) and the member and joint deformations (strains) must satisfy the behavioural laws for the material of which they are constructed. The fundamental behavioural law for a material is its stress-stain relationship which incorporates the value of Young's Modulus, the yield strength and the ductile capacity of the material. Other laws derived from it to establish the behaviour and the strength of members and joints under combined bending, shear and axial load may also be accounted for in advanced structural analysis procedures. |  |
| - At the core of any structural analysis model is the structural element model representing the behaviour of the basic components (members and joints). For the members, the model expresses the relationships between the displacements and rotations at the end of the member to the forces applied to it (member forces). These relationships satisfy the three basic principles of equilibrium, compatibility and the constitutive laws at all sections along the member. The structural element representing the joint, when included, is developed along similar principles as that for a member although the constitutive laws used are sometimes partly based on empirical and/or on semi-empirical relationships established by calibration with test results. |  |
| - The structural element model used in most computer analysis techniques to represent the behaviour of frame components takes the form of a stiffness matrix. In this way, the solution to the analysis involves satisfying equilibrium and compatibility at the joints between the elements. More than one structural element can be used to represent a single frame component. |  |
| Many analysis programmes in use today assume joints to be either pinned or rigid. However, it is usually possible to introduce joint elements (i.e. semi-continuous joint modelling) and to provide the behavioural model for them. In particular, the influence of the movements of the foundation can be investigated by assigning suitable properties to the support joints. By assuming that the joints at the ends of an element are in fact an extension of the element itself, it is possible to establish an equivalent element which includes the joint properties. This technique is useful for hand calculations and in particular for studying the in-plane stability or resistance of a structure or of an individual member which includes joint behaviour. | Part 1-1 5.1.2 |
| - The most commonly used structural analysis method by computer is based on finding the displacements and rotations at each of the joints which satisfy compatibility, while equilibrium is ensured by means of the application of the constitutive laws. This method is called the displacement method of analysis. The alternative approach consists of finding the member forces which satisfy equilibrium at each of the joints, while ensuring compatibility by the application of the constitutive laws. This latter approach, known as the force method, is well suited to manual analysis procedures such as the moment distribution method. However, it is less well adapted to automated computer techniques than the displacement method. |  |
| - Because of the nature of the many simplifying assumptions on which they are based, many present day manual and computer methods for structural analysis do not detect the onset of structural instability. Instability can come through local plate buckling of a part of the member cross section or of a joint, buckling of a member (including lateral-torsional buckling) or buckling instability of part or of the entire structure. Therefore, in addition to the structural analysis, additional analysis and/or design measures usually have to be undertaken to ensure the structural stability. These measures depend to some extent on the type and complexity of the analysis model used and on the type or types of members and joints used. |  |
| - Methods of global structural analysis |  |
| - General | Part 1-1 5.4 |
| The determination of the actual load-deformation response generally requires the use of a sophisticated analysis method. For practical purposes, assumptions for the structure and the member and joint models are made that permit to obtain a safe bound for the ultimate load. Hence models range from the simple elastic analysis or the rigid-plastic analysis to the most complex, elastic-plastic analysis, which can provide a close representation of the real behaviour of the structure. |  |
| The first important distinction that can be made between the methods of analysis is the one that separates elastic and plastic methods. While elastic analysis can be used in all cases, the use of plastic analysis is subjected to some restrictions. Another important distinction is between those which make allowance for and those which neglect the effects of the actual displaced configuration of the structure. They are referred to as second-order theory and first-order theory based methods respectively. While the second-order theory can be adopted in all cases, the first-order theory may be used only either when the displacement effects of the structural behaviour are negligible or when they can be incorporated in some other way. |  |
| In the following discussion of the methods of analysis, for clarity, reference will be made to two dimensional structures only which are subject to in-plane loading and deformations. |  |
| Although only one loading combination is discussed, it is understood that all loading combinations must be analysed. |  |
| - Second-order effects |  |
| - The deflections due to the external loads modify the structural response and the distribution of the internal forces. Therefore it is necessary to evaluate the degree to which they are significant so that appropriate action can be taken to account for them if needed. |  |
| - For frames, the most significant modifications to the linear response are caused by the sway and the axial loads. To illustrate these modifications the example of a fixed bar cantilever subject to combined axial and transverse loads applied at the free end, shown in Figure 5, is taken. The cantilever is representative of part of the height of a column, i.e. from its base up to the point of inflexion near mid-height, in a frame structure subjected to sway displacement. The lateral displacement of the point of inflexion will be close to half of the relative sway between the floor above and the floor below the column. For clarity, other than the horizontal force at the inflexion point and the corresponding reaction at the base, no other horizontal loads (such as span loading due to wind for instance) is assumed to be applied to the member. |  |
| Note: The axial load has been designated by the symbol P here to relate to commonly used terminology in the literature about second-order effects. It would normally be designated by N in Eurocode 3 Part 1-1. |  |
| - In the presence of the axial load, the lateral (sway) displacement at the top of the member and the curvature of the member itself, second-order effects in the form of secondary moments are induced along the member. The consequences will be that the actual deformations of the column under a given loading are greater than predicted by a first order analysis (i.e. where the secondary effects are neglected). |  |
| - First and second order moments in a beam-column |  |
| - It can be seen that a global second-order moment, commonly referred to as the P- effect, arises throughout the member due to relative lateral (sway) displacement () between the top and the bottom of the member. In addition a local second-order moment, commonly referred to as the P- effect, arises in the axially loaded member due to the deflections () relative to the chord line connecting the member ends. |  |
| For frames, whilst the P- effect still arises when sway deformation is prevented, both the P- effect and the P- effect arise when sway can occur. In the case of sway of typical frames, the P- effect is usually much more significant than the P- effect. It is always necessary to evaluate whether sway effects in a frame are significant or not, whilst account for the P- effect needs only be considered for particularly slender members (see Lecture 5 on “Frame classification and joint representation”). |  |
| - Note: In Eurocode 3 Part 1-1, the symbol  is surprisingly used in one particular formula to denote the sway deflection even if “P- effect” is commonly used to characterise the element response; in the present lecture notes, reference is made to the usual meaning of  i.e. a member deflection. | Part 1-1 5.2.1(4)B |
| Both effects occur independent of whether the axial load is a compression one or a tension one. In terms of member stability and of overall frame stability, secondary effects due to tension forces are beneficial while those due to compression forces are detrimental. |  |
| - Slope-deflection method |  |
| - A basic understanding of elastic frame analysis can be obtained by examining the slope-deflection equations for a simple member in bending, or in compression and bending. Starting from the equations for a first-order theory, how second-order effects can be introduced is then explained. |  |
| - First-order elastic analysis |  |
| - The basic slope-deflection equations express the moment at the end of a member as the superposition of the end moments due to external loads on the member with the ends assumed fixed and of the moments caused by the actual end displacements and rotations. A set of simultaneous equations is written down to express the equilibrium of the joints in which the moments caused by joint displacements and rotations are expressed as linear functions of those same displacements and rotations. These equations are used in establishing the terms related to bending and shear in the stiffness matrix for the member. |  |
| - When using the slope-deflection method to analyse a sway frame, use is made of the compatibility of the member rotations at the joints. In addition equations expressing the lateral equilibrium of each floor are also established. The solution of the full system of equations gives the unknown joint displacements and rotations. When these are back substituted into the original slope-deflection equations for each member one obtains the end moments. The end reactions can then be obtained. |  |
| - The slope-deflection method is in fact an application of the more general displacement method of first-order elastic analysis to plane frames in which the effects of axial and shear strain energy can be disregarded compared to the bending strain energy. This simplification is usually justified when analysing frames. |  |
| - Deformation of beam-column |  |
| - Figure 6 shows the sign convention adopted here, and the symbols used for the moments, shear forces, rotations and sway displacement. Note: the displacement  is much smaller than the member length. |  |
| The basic slope-deflection equations for the member may then be written as follows: |  |
| where, in addition to the symbols defined in Figure 6, |  |
| and MBA are the moments at the joint nodes A and B, MAB and MBA are the fixed-end moments for the member lateral loads alone, is the slope of the chord line AB caused by sway. |  |
| The corresponding shear forces at the member ends are obtained from the following relationships: |  |
| where VAB and VBA are the end shears for a simply supported beam of span L. |  |
| Note: The slope-deflection equations given above are valid for the case of rigid joints. Modified equations are used when the joint flexural stiffness are taken into account [4]. |  |
| The slope-deflection equations, the shear equations and the usual relation between axial load and axial deformation are used for the derivation of the first-order stiffness matrix for each member for analysis programmes [5]. When sway is prevented the contribution of the sway term (6 AB) can be omitted. |  |
| When the slope-deflection or the moment-distribution methods are used for the analysis of sway frames, it is usual to analyse the structure first for all design loads acting but with sway prevented. The effects of sway are then obtained from a separate analysis and the results from both analyses are superimposed. For the separate sway analysis, equations which express the equilibrium between the external horizontal forces acting at each storey (i.e. the sum of the column shears) and the corresponding moments in the columns in the storey are used. The horizontal forces acting on the structure are given by the first analysis as the values for the horizontal reactions (needed to prevent sway) at each floor level, but applied in the opposite direction. The first-order sway equilibrium equation for each floor takes the following form, the summation being over all the columns of the given storey i: |  |
| where Hi is the total external horizontal force acting on the floor i of height hi and A and B denote the two ends of a typical column j. |  |
| The initial fixed-end moments due to sway in each column for this analysis, in which further sway at every floor is prevented, are calculated using the following formula: |  |
| where i is the sway of the floor supported by the columns of storey i relative to the floor below and which is the unknown to be solved. |  |
| The approach involves carrying out a separate analysis for the sway of one floor at a time. The final sways can be resolved using the series of sway equations, there being as many equations as there are unknown floor sways. |  |
| - Second-order analysis |  |
| When the P- and the P- effects are ignored, each structural element is characterised by a linear stiffness matrix for a first-order elastic analysis. The result of this approach will be acceptable only when the column members have relatively low axial loads (i.e. the columns are relatively stocky). |  |
| The importance of the P- axial load effect is controlled by the ratio of the member’s axial load NAx to the Euler buckling load NE. For a member of length L and second moment of area I, the Euler load is given by NE = 2 EI / L2. Account for the P- can be achieved by modifying the terms of the linear stiffness matrix so that they include terms (stability functions) which are functions of the ratio . The axial load effectively modifies the stiffness of the member. |  |
| Note: A distinction is made here between the axial load in the member NAx (taken as acting along the chord line AB) and the load N applied at the member ends as indicated in Figure 6. When there is sway, the member axial load is slightly different from the load N. This difference is usually ignored since sway displacements are relatively small (cos AB ≈ 1 and sin AB ≈ AB). |  |
| The following are the modified slope-deflection equations for the member: |  |
| - The formulae for the terms s, c and m have values of 4, 0,5 and 1 respectively when  = 0, so the equations are then the same as for the first-order analysis. The other terms in the equations are as defined for the first-order analysis (see Figure 6). |  |
| The parameter m indicates that the fixed-end moments are slightly different than for the member without axial load. It can be expressed in terms of the parameters s and c (so-called stability functions). For instance, for a uniformly distributed load it has the value of 6 / [s(1 + c)] (see reference [5]). Its effect is to increase the fixed end moments when the axial load is compressive and to reduce them when it is tensile. The influence of this term is sometimes ignored. |  |
| - Note: Although the formulae for these non-dimensional parameters for a tensile load and for a compressive load are different, they converge to the same values at  = 0 Numerical approximations for them, suited for computer applications, exist which permit having only one expression for both tension and compression formulae (see reference [8]. |  |
| - Compared to those for the first-order analysis, the equations for the end shear forces are also modified as they now include a new term to account for the effect of sway. This new term represents the P- effect. Noting that the axial load N is taken as positive for a compressive axial load, they become as follows: |  |
| - A full treatment of second-order analysis of plane frames can be found in references [6], [7], [8] where it is shown how to extend second-order analysis to failure (including instability) in the elastic-plastic domain. The Eqs. , and indicate the most important modifications to the member shear and bending stiffness terms while a more in depth treatment shows that sway introduces other higher-order terms into the shear and axial load stiffness terms. The resulting modified stiffness matrix for the structure as a whole is non-linear since the stiffness terms are now functions of the actual sway displacements as well as of the member axial loads. In fact, equilibrium of the structure is established for its deformed shape. A second-order analysis is usually carried out by increasing all loads incrementally and convergence by an iteration procedure is usually used [6]. | Part 1-1 5.2.1 & 5.2.2 |
| - It is recognised that for typical frames with sway where very slender columns are not used, since the P- effect is then negligible compared to that of the P- effect, the direct use of the non-linear theory is not required. Simpler approaches, based on iteration of a first-order analysis for instance, can be safely used for these cases. Methods for accounting for the P- effect, which are essentially based on satisfying the modified shear equations given above, are described in the following section. |  |
| - Caution should be exercised when very slender members or curved members are used since the moments occurring along their entire lengths are modified. The simpler methods of second-order analysis are not usually suited for such special cases and the more exact second-order analysis may be needed. The member is best modelled by a number of elements which also allows introducing member imperfections. In this way, moments and forces at the joints thus created along the length of the member as well as at its ends can be obtained from the analysis. |  |
| - Equivalent lateral load approach for second-order analysis |  |
| - From figure 5, it is observed that the end moment at the column base has increased from the first-order value of [H h] to [H h + P ]. It is as if the effective horizontal load (shear force) had been increased by [P  / h] due to sway. |  |
| The approach involves using the initial values of the axial loads and floor sways given by a first-order linear-elastic analysis of the entire structure for both vertical and horizontal loads. From these axial loads and sways one determines the extra “equivalent” horizontal force of [P f / hs] to be applied at the top of each column of each storey (i.e. at the level of the floor it supports) in the direction of the sway. In this expression, hs is the storey height of a given column and f is the relative sway between the top and bottom of the column. As equal but opposite horizontal forces are applied at the bottom of each column, there is no increase in the resultant horizontal force applied to the structure as a whole. The total additional horizontal force at any given floor level will be given by the sum for all the columns of the storey below minus the sum for all the columns of the storey above (if any). |  |
| - The result will be a closed system of forces. |  |
| - A new first-order analysis is carried out so as to incorporate the effects of the “equivalent” horizontal loads applied at each floor (including those at the foundation level). The procedure needs to be repeated until the values for the floor sways converge to an acceptable level of accuracy. If it does not converge within a few iterations, it can be concluded that the structure is unstable. When convergence is established, the resulting internal forces and moments in every member now include the P- effects. |  |
| - The method is summarised in Figure 7. The initial sway displacements are denoted by in Figure 7, where i denotes the storey level. |  |
| The total additional storey shear for any storey level is computed by: |  |
| - where: additional shear in storey i due the sway forces; sum of the column axial loads for storey level i; hi height of storey i which is between floors levels i - 1 and i; i, i - 1 total sway displacements of floor levels i and i-1 respectively, i.e. floor sways relative to the foundation level (level 0) where zero lateral movement is assumed. |  |
| - All storeys of a building are assumed to sway in the same direction. The storey shear at the top of columns is opposite in direction to that at the bottom of columns. The total “equivalent” sway force acting at any given floor is therefore computed as the difference between the “additional” storey shear from the columns of the storey below the floor and that for the columns of the storey above the floor, i.e.: |  |
| - The structure is analysed again using the first-order theory, either for the sway forces H'acting alone and then combining the results with those from the first analysis, or by including them in the lateral forces for when all loads are acting. When the i values at the end of a cycle are close (say within 5%) to those given by the previous cycle, the method can be considered to have converged sufficiently. Some computer programs in use perform second-order sway analysis in this way. |  |
| - Equivalent lateral force procedure |  |
| - Modified slope-deflection method |  |
| - The sum of the axial loads in all the columns of a storey must be equal to the total vertical load applied to that storey. Either a hand calculation or a first-order analysis can provide the value for the sum of the column axial loads in any storey. |  |
| - When using the slope-deflection method, a simple way of including P- effects is to use a modified sway equation expressing the lateral equilibrium of a storey. The modified sway equation becomes: |  |
| - where is the known sum of the axial loads in the columns j of the storey i and i is the sway of storey i. The other terms are as defined for the basic slope-deflection sway equation. |  |
| - This approach leads to a direct solution for sway including the P- effects without the need for any iteration in the calculations. The modified sway equation can also be used when using the moment-distribution method. |  |
| - Global elastic structural analysis |  |
| - First-order theory |  |
| - Assumptions, limitations, section and joint requirements |  |
| Linear-elastic analysis implies an indefinite linear response of sections and joints (see Figure 8). Equilibrium is expressed with reference to the undeformed configuration of the structure in a first-order analysis. | Part 1-1 5.4.2(1) & (2) |
| - Moment-rotation characteristics of member and joint |  |
| A priori, no requirements related to the ability of sections and joints to exhibit ductile behaviour (class of member cross-section, ductility class of joint) are imposed. However the class of member cross-section to be finally adopted depends on the type of cross-section verification (ultimate resistance criteria) used. | 5.4.1(2) |
| - Structural analysis |  |
| - First-order elastic global analysis with linear member and joint behaviour results in a linear load deflection curve (see Figure 9). |  |
| - Load displacement response: first-order elastic analysis |  |
| - Frame imperfections must be allowed for in the analysis. | Part 1-1 5.3.1(1) |
| - Designers are quite familiar with first-order elastic analysis which is the simplest of all possible types of analysis. Over the years, a variety of methods have been developed aimed at hand calculation such as the slope-deflection method, the moment-distribution method as well as analytical formulae (sometimes presented in non-dimensional graphical form) for rapid analysis. They can be generalised so as to include the joint behaviour [4]. The same applies to procedures based on matrix formulation, which have now almost entirely supplanted the hand methods, as computer use has become common practice in design offices. |  |
| - A significant advantage of the first-order elastic analysis is that it permits one to apply the principle of superposition of loading and load effects. |  |
| - Structural design |  |
| - This aspect is treated in more detail in Lecture 6 “Choice of structural analysis and consequences on design”. |  |
| - First-order elastic analysis provides a safe basis for design as long as the predicted response of the structure deviates only slightly from the actual response over a considerable range of load (i.e. structures having low axial loads). | Part 1-1 5.2 |
| - It is necessary to check the overall stability of the frame. The elastic critical buckling load for the frame may be evaluated using either a special analysis (see references [5], [6]) or by an approximate procedure (see Lecture 5 on “Structural classification and joint representation”). | Part 1-1 5.2.1 |
| - For most frames, a first-order elastic analysis generally provides a good tool for predicting the response of the structure and of its elements at the serviceability limit state (permissible deflections). At this level of loading, the non-linear effects will be relatively small. |  |
| - Once the design forces (axial forces, bending moments and shear forces) have been determined throughout the structure, the following design checks must be carried out: |  |
| - Check for ground-structure interaction effects and allow for them when required; - Check for second-order effects and allow for them when required; - Check the resistance (ultimate limit state) of the cross-sections of the members; - Check the resistance (ultimate limit state) and the classification requirements of the joints; - Check against instability phenomena (in-plane and/or out-of-plane); column buckling, lateral-torsional buckling , and partial or overall frame instability; - Check against local buckling and the resistance to concentrated loads; - Check fire resistance; - Check the serviceability limit state requirements, i.e. permissible deflections and, where required, vibrations; - Check for resistance to fatigue. |  |
| - Second-order theory |  |
| - Assumptions, limitations, section and joint requirements |  |
| - In this type of elastic analysis, the indefinitely linear-elastic response of sections and joints is still considered (Figure 8). The distribution of the internal forces is now computed on the basis of a second-order theory of the kinds outlined previously (although for frames meeting certain conditions, a method consisting in simplified nominal corrections to a first-order analysis may be used). Equilibrium equations are formulated for the deformed structure (P- effects) with account being made, when necessary, for member imperfections and for the decrease in member stiffness due to its axial compressive force (P- effects). |  |
| A priori, no requirements related to the ability of sections and joints to exhibit ductile behaviour (class of member cross-section, ductility class of joint) are imposed. However the class of member cross-section to be finally adopted depends on the type of cross-section verification (ultimate resistance criteria) used. | Part 1-1 5.4.1(2) |
| Contrary to a first-order elastic analysis, strictly speaking one may no longer superimpose the results from the analysis of individual load cases. One is advised to analyse each load combination separately. |  |
| - Structural analysis |  |
| - Figure 10 shows the load deflection response that results from a second-order elastic analysis in which all the loads are increased monotonically by the same load multiplier (proportional loading). The load deflection curve, which now includes geometric non-linearity, approaches asymptotically a horizontal corresponding to a peak value of cr (critical load multiplier). This value of cr corresponds to the elastic critical buckling load of the frame, relevant to the specified load case combination. |  |
| - If the P- effects are disregarded, then the computed peak load may be higher than the actual one. It is recalled that the more slender the compression members are the more significant the P- effects become. |  |
| - The elastic critical buckling load is an important reference, as it is the highest theoretical load that the frame can withstand in the absence of any material yielding. When yielding occurs as it is often the case, it will make the actual maximum load lower, and usually appreciably lower, than the elastic critical buckling load. |  |
| - Load-displacement response for second order elastic analysis |  |
| - Structural design |  |
| Contrary to first-order elastic analysis, second-order analysis provides internal forces and moments at the ends of the members that include the second-order effects, thus eliminating the need to examine this question further. |  |
| - The elastic critical buckling load of the frame may also be provided by a second-order elastic analysis but only if the load multiplier is increased sufficiently. However, if the frame is designed to remain elastic there is no need to check in-plane frame stability. Furthermore, since the P- effects are not usually significant there is then no need to check the in-plane stability of the members. When curved or very slender members are used, member imperfections and the P- effects should be included in the analysis. Nevertheless in this case, unless the members are modelled using enough elements to provide the moments and forces along its length, an in-plane stability check of the members is advised. |  |
| - With due attention being paid to the fact that some stability design checks are usually no longer required, the remaining design checks are the same as for a first-order elastic analysis. |  |
| - Global plastic structural analysis |  |
| - Introduction |  |
| It is recalled here that plastic methods of analysis are permitted only when certain minimum requirements on steel ductility, member cross-section ductility, joint ductility and lateral support at hinges locations are met. These are needed in order to guarantee that sections and joints, at least at the locations at which the plastic hinges may form, have sufficient rotation capacity to permit all the plastic hinges to develop throughout the structure. | Part 1-1 5.4.1(3) & 5.4.3 |
| - Elastic-perfectly plastic analysis (second-order theory) |  |
| - Assumptions, limitations, section and joint requirements |  |
| In the elastic-perfectly plastic analysis, it is assumed that any section and/or joint remains elastic up to the attainment of the plastic moment resistance, at which point it becomes ideally plastic. Plastic deformations are assumed to be concentrated at the plastic hinge locations which are assumed to have an infinite rotational capacity. That actual rotation capacities are sufficient to meet what is required must usually be checked later. |  |
| - Figure 11 shows the elastic-perfectly plastic behaviour of a section and a joint. The influence of the normal force and/or the shear force on the plastic moment resistance of the sections may either be accounted for directly or be checked later at the design verification stage. |  |
| - Note: The plastic hinge moment resistance to be adopted is the design value Mpl,Rd. The subscript symbols used here are explained in Module 4 on “Design of Members”. |  |
| The load displacement curve of the structure can then be determined. Computation of the plastic rotations at the plastic hinges may also be carried out so as to permit the check that the required rotation capacity is available. |  |
| - Behaviour of members and joints |  |
| - Structural analysis and design |  |
| The following is a typical description of the various steps of a second-order elastic-perfectly plastic analysis, which is usually performed with the load being applied by increments. For clarity, it is also assumed that plastic hinges are formed sequentially, although more than one can occur simultaneously. |  |
| One starts with the displacements from an elastic second-order analysis (see Figure 12, branch 1), and by monitoring the bending moments in the structure at each incremental increase of the loads. The load at which the first hinge occurs is identified as that load for which the plastic moment resistance is attained in a section or in a joint. The next analysis is made for further incremental loads for which the frame behaves differently with the introduction of a pinned joint at the location of the first plastic hinge (branch 2). It is recalled that a plastic hinge is assumed to be unable to resist any further moment increase and to have sufficient ductility to undergo the necessary rotation. The joint introduced at the plastic hinge acts as a pin only for the subsequent incremental increases in the loading, but transferring a moment equal to the plastic moment resistance. The modified structure is said to have “deteriorated”. The next plastic hinge is formed after further increase of the load level and the process is repeated. |  |
| - Load-displacement response for second-order elastic-perfectly plastic analysis |  |
| - This process is repeated with new hinges being formed (and corresponding pins being introduced) at each step, till the structure becomes unstable (mechanism or frame instability). The maximum load of the second-order elastic-plastic analysis corresponds to this load level which is shown as the reference load multiplier L2EPP in Figure 12. |  |
| - No additional design checks of the resistance of sections and joints are required if the influence of the normal force and/or the shear force is accounted for. As the rotations at the plastic hinges have been calculated, it is so possible to check that the required rotation capacity is available. |  |
| - When second-order theory is used, the global stability is covered by the structural analysis. Furthermore in that case, stability of the members will not have to be verified on condition that local member imperfections have been taken into account in the initial analysis when required (but see the remarks on design when using first-order elastic analysis). |  |
| It is clear that the number of remaining design tasks has been reduced compared to what is required when an elastic (first order or second order) analysis has been used. |  |
| - Elastic-plastic non-linear analysis (second-order theory) |  |
| - Assumptions, limitations, section and joint requirements |  |
| Using a second-order elastic-plastic analysis a better estimation of the structural response can be obtained (relative to that provided by a first-order or even a second-order elastic-perfectly plastic analysis for instance). |  |
| - Yielding of members and joints is a progressive process and so the transition from elastic behaviour to a plastic one is a gradual phenomenon. Once yielding starts, as the moment in the member cross section continues to increase, the plastic zone extends partially along the member as well as through the depth of the cross-section. This behaviour is considered by the plastic zone theory (see references [8], [10]). |  |
| - Moment-rotation characteristics of member and joint |  |
| - Figure 13 shows the moment-rotation characteristics of members and joints which are usually adopted in this type of analysis. The beneficial effects of material strain hardening or membrane action in joints have not been included in these models. |  |
| - The ductility requirements for the members and joints, and the procedure for analysis and for checks are the same as those outlined for second-order elastic-perfectly plastic analysis. |  |
| - The elastic-plastic method, because of its complexity, is not used for practical design purposes and is usually restricted to computer research applications. |  |
| - Rigid-plastic analysis (first-order theory) |  |
| - Assumptions, limitations, section and joint requirements |  |
| Contrary to the elastic-plastic analysis, the elastic deformations (of members, joints and foundations), being small compared to the plastic deformations, are ignored in the rigid-plastic analysis. As for the elastic-perfectly plastic analysis, the plastic deformations are concentrated in sections and joints where plastic hinges are likely to occur. These sections and joints are assumed to have an infinite rotational capacity. |  |
| - Figure 14 shows the idealised rigid-plastic response of the sections and the joints which are adopted for this type of analysis. As a result, the values of the design moment-resistance for sections and joints as well as the structural configuration and the loading are the only parameters that affect rigid-plastic analysis. |  |
| - Moment rotation characteristics of member and joint |  |
| - The ductility requirements for the members and the joints are the same as those outlined for the elastic-perfectly plastic analysis. Useful information on the minimum limits to be satisfied when using first-order rigid-plastic design for single-bay pinned-base frames are given in reference [9]. Rigid-plastic methods are not usually suited for second-order analysis. |  |
| - Structural analysis |  |
| - On the basis that the maximum loading that a given structure can sustain corresponds to that for which collapse occurs because a realistic plastic mechanism has been created, the analysis consists in identifying the critical mechanism. |  |
| - The maximum load (i.e. the collapse load) can be obtained by the direct application of the fundamental theorems of simple plastic design. These fundamental theorems are the lower-bound and the upper-bound theorems, also known as the static theorem and the kinematic theorem respectively. A third theorem, the uniqueness theorem, states that when the conditions of both the static and kinematic theorems are satisfied simultaneously by a given mechanism for the same loading case, the collapse load has been found [11]. |  |
| - An approach suitable for the manual application of the upper bound theorem is summarised here. According to this theorem, for a given structure and loading, any arbitrarily assumed plastic collapse mechanism occurs at a value of the load multiplier that is greater than or equal to the value of the collapse load multiplier. By examining the various possible mechanisms, one identifies the collapse mechanism for which the value of the load multiplier is least and which is both statically and plastically admissible. |  |
| - The collapse load for each mechanism can be obtained by applying the Principal of Virtual Work, i.e. by equating the external work to the internal work done in forming the mechanism. |  |
| - Load displacement response for rigid plastic analysis |  |
| Figure 15 shows the elementary mechanisms 1 and 2 as well as the combined mechanism 3 for a simple portal frame. |  |
| For the purposes of establishing the analysis/design equations for the simple frame in Figure 15, the following is assumed: |  |
| - The ratio of the design vertical load WEd acting at mid-span of the beam, to the design horizontal load HEd acting at the eaves, , is known from the load combination case evaluation; - Columns AB and DE, of height h, have the same cross-section design resistances; - The joints at A and at E have the same design resistances; - The joints at B and at D have the same design resistances; - The design moment at A and E, denoted Mpl,Rd,1, will be the smaller of the design resistances for the column section and for the joint; - The design moment at B and D, denoted Mpl,Rd,2, will be smaller of the resistances for the column section, for the beam section and for the joint; - The design moment at C, denoted Mpl,Rd,3, will be that for the cross-section of the beam which has a length of L; - The positive and negative design moments at any section or joint are the same. |  |
| The equations corresponding to each mechanism are then obtained from the application of the Principle of Virtual Work: |  |
| - Mechanism 1: - Since , we obtain - so that , which should have a value of at least unity. - Mechanism 2: Since , we obtain and , which should have a value of at least unity. - Mechanism 3: Since and , we obtain: , which should have a value of at least unity. |  |
| - Any load-displacement response is represented by a horizontal line, the ordinate of which is the associated collapse load multiplier value. In accordance with the upper bound theorem, the lowest curve shall be retained which is assumed to be that for mechanism 3 in this case. Therefore, the collapse load given by the rigid-plastic analysis for this structure and loading corresponds to that given by the value for the load multiplier LRP3 shown in Figure 15. |  |
| - For most cases of simple rectangular frames the manual application of the rigid-plastic method is simple and straightforward. For multi-storey and/or multi-bay frames, for which particular care has to be taken to identify hinges that form and later unload (so as not to appear in the final collapse mechanism), the use of a computer programme is usually required [5]. It is also necessary to give consideration to “partial collapse mechanisms” and to the possibility that two different mechanisms give the same value for the actual collapse load. In that case, the failure mechanism is in fact a combination of both, forming what is known as an “over-complete collapse” mechanism. For the example of Figure 15, no consideration has been given to the over-complete collapse case, which can only occur for particular load combination cases in a given structure (see reference [11]). |  |
| - Most typical frame structures can be analysed by considering complete collapse mechanisms (mechanisms 2 and 3 in Figure 15 are examples) and partial collapse mechanisms (mechanism 1 in Figure 15 is an example) only. For complete collapse mechanisms, the entire structure is always statically determinate at collapse. For a structure with r redundancies, a complete collapse mechanism involves r + 1 plastic hinges. A partial collapse mechanism has less, whilst an over-complete collapse mechanism has more plastic hinges. The number of independent complete collapse mechanisms is given by h - r, where h is the number of possible plastic hinge locations. |  |
| - Structural design |  |
| - The rigid-plastic analysis provides direct information in terms of the design frame resistance. Adequate design requires that the value of the load multiplier for the critical mechanism be at least unity. Allowance for in-plane stability and second-order effects may require that the value of the load multiplier be reduced (see Lecture 6 “Choice of structural analysis and consequences on design”). |  |
| - However, some additional design checks for sections and joints are required as the influence of the normal forces and/or the shear forces on the design moment resistances may not be negligible. As the rotations at the plastic hinges have been supposed infinite and no evaluation for them is provided, sections (and joints, when necessary) which can be assumed to be sufficiently ductile must be chosen. |  |
| - This type of analysis does not provide any information on the structural deflections that have occurred due to the loading. Therefore, normally it has to be complemented by an elastic analysis of the structure for the serviceability loading conditions. |  |
| - These distinctions apart, there is little difference between the other design tasks (for stability for instance) to be carried out compared to those required following a linear elastic analysis. |  |

- References

- Annex H [Informative], Modelling of building structures for analysis, ENV 1993-1-1:1992/A2, Amendment A2 to Eurocode 3 Part 1-1.

- EN 1998 Eurocode 8 Design provisions for earthquake resistance of structures.

- EN 1993-1-3 Eurocode 3 Part 1-3 General rules - Supplementary rules for cold formed thin gauge members and sheeting.

- Maquoi R. and Chabrolin, B., Frame design including joint behaviour, Report N° EUR 18563 EN, ECSC Contract No 7210-SA/212/320, European Commission, 1998.

- Livesley, R.K., Matrix methods of structural analysis, Pergamon Press, 1969.

- Gachon, H., Galea, Y., Model of non-linear analysis for plane frames (in french), Construction Métallique, N°2-1978.

- Galea, Y., Non-linear elasto-plastic analysis programme for plane frames (in french), Construction Métallique, N°4-1978.

- Chen, W.K., Goto, Y. and Liew, J.Y.R., Stability design of semi-rigid frames, John Wiley & Sons, Inc. 1996.

- ECCS -Technical Committee 8- Structural Stability, Technical Working Group 8.1/8.2 Skeletal Structures, Practical analysis of single-storey frames, ECCS Publication N° 61, 1991.

- Clarke, M..J., Plastic zone analysis of frames in Advanced analysis of steel frames: Theory, Software and Applications, Chen, W.F. and Toma, S., eds., Boca Rotan, FL, CRC Press, pp 259-274, 1994.

- Neal, B.G., Plastic methods of structural analysis, Chapman and Hall, 1956.

- The Steel Construction Institute, Steel Designers Manual, 5th Edition, Blackwell Scientific Publications, 1992.

- Eurocode 3: EN 1993–1–1: Eurocode 3: Design of steel structures. Part 1–1: General rules and rules for buildings.

- Eurocode 3: EN 1993–1–8: Eurocode 3: Design of steel structures. Part 1–8: Design of joints.