- Module 3

Member design

Lecture 9Members in bending and shear

- Contents:

1.	Introduction

2.	Ultimate limit states

2.1.	Restrained beams

2.1.1.	Section resistance to moment

2.1.2.	Section resistance to shear

2.1.3.	Section resistance to combined bending and shear

2.1.4.	Section resistance to combined bending, shear and axial force

2.2.	Unrestrained beams

2.2.1.	Section resistance

2.2.2.	Lateral-torsional buckling

2.2.3.	Buckling resistance of unrestrained beams

3.	Serviceability limit states

4.	References

| - Introduction |  |
| --- | --- |
| Beams are perhaps the most basic structural components. A variety of section shapes and beam types may therefore be used, depending on the magnitude of the loading applied and on the span, as shown in Table 1. |  |
| - Type of beam sections and special features |  |
| Beam type; Span range [m]; Notes. 0. Angles; 3 – 6; Used for roof purlins, sheeting rails, etc., where only light loads have to be carried. 1. Cold-formed sections; 4 – 8; Used for roof purlins, sheeting rails, etc., where only light loads have to be carried. 2. Rolled Sections UB, IPE, UPN, HE, RHS, CHS; 1 – 30; Most frequently used type of section; proportions selected to eliminate several possible types of failure. 3. Open web joists; 4 – 40; Prefabricated using angles or tubes as chords and round bars for web diagonals; used in place of rolled sections. 4. Castellated beams; 6 – 60; Used for long spans and/or light loads, depth of UB increased by 50%, web openings may be used for services, etc. 5. Compound sections e.g. IPE + UPN; 5 – 15; Used when a single rolled section would not provide sufficient capacity; often arranged to provide enhanced horizontal bending strength as well. 6. Plate girders; 10 – 100; Made by welding together 3 plates, sometimes automatically; web depth up to 3-4 m sometimes need stiffening. 7. Box girders; 15 – 200; Fabricated from plate, usually stiffened; used for OHT cranes and bridges due to good torsional and transverse stiffness properties. 8. Lattice girders; 20 – 100; Typically fabricated from hollow section, little self weight, large openings and hence transparency. |  |
| Of course, hollow sections may be substituted for open sections in common applications. However, they are especially used when stability and/or torsional rigidity are of paramount importance. |  |
| The most usual situation in which a hollow section is subjected to bending is as an horizontal beam supporting transverse gravity loads. Though, hollow sections experience bending in many other applications, such as: |  |
| - Portal frames where both beams and columns are subjected to significant bending; - Rigid-jointed trusses with loads applied not only at the truss nodes but also along the chord members; - Many transportation and handling systems. |  |
| A steel beam is simply designed with regard to both resistance and stiffness criteria. The design resistance moment has to be at least equal to the design maximum applied moment with due allowance made for coincident shear. Adequate stiffness shall be provided so that the transverse deflection and the possible vibrations do not affect adversely the serviceability requirements. |  |
| Beams bent about the major axis of their cross-section are usually designated as “restrained beams” or “unrestrained beams”, depending on their inability or their ability to move laterally and/or to distort about their longitudinal axis. |  |
| A slender unrestrained beam loaded in its stiff plane – i.e. bent about the major axis – is prone to fail by a form of instability that is specific to beams: lateral-torsional buckling. This phenomenon involves both lateral deflection (bending about the minor axis) and twisting. Figure 1 illustrates it with a slender cantilever beam loaded by a gravity load applied at the tip. |  |
| - Lateral torsional buckling of a slender cantilever beam |  |
| If the unbraced cantilever is perfectly straight and its cross-section is initially stress-free and perfectly elastic, the tip of the cantilever would deflect only in the vertical plane with no out-of-plane deflection, until the applied moment reaches a critical value at which the beam buckles by deflecting laterally and twisting. A design approach for beams prone to failure by lateral-torsional buckling must necessarily account for a large number of factors – including section shape, degree of lateral restraint, type of loading, residual stress pattern and initial imperfections – and is therefore relatively complex. |  |
| Beams which are prevented from such a form of instability are termed ‘restrained’. That may be achieved by providing the member with: |  |
| - Full lateral restraint by, for example, positive attachment of the floor system to the top flange of a simply supported beam; - Adequate lateral and torsional restraint to, preferably, the compression flange by, for example, duly connected profiled roof sheeting; - Closely spaced bracing elements so that lateral-torsional buckling of the beam cannot occur in between the unbraced lengths. |  |
| - The design resistance moment of a restrained beam is given as the one of the cross-section. In contrast, the design resistance moment of an unrestrained beam is affected by lateral-torsional buckling and is lower than the design resistance moment of the cross-section. |  |
| Sections bent about their minor axis cannot fail by lateral-torsional instability and behave as restrained beams. Beams with sections of large torsional stiffness behave similarly; hollow sections are just sections complying with this condition. Therefore, in general, lateral-torsional buckling resistance need not to be checked for circular hollow sections (CHS) and also for rectangular hollow sections (RHS) normally used in practice, i.e. those with a depth-to-width ratio smaller than about 2; that is due to the fact that their polar moment of inertia is very large in comparison with that of open sections. Also, there are few rectangular hollow sections which do not satisfy this ratio; they are termed ‘planks’ and should preferably be reserved for other uses than beams. |  |
| - Ultimate limit states |  |
| - Restrained beams |  |
| As explained above, the member resistance of restrained beams is given by the resistance of their cross-sections. |  |
| Let us consider a restrained simply supported beam made of a Class 1 or 2 RHS cold-formed hollow section, and loaded by a point load at mi-span as shown in Figure 2. |  |
| - Section resistance to moment |  |
| According to the engineering bending theory the strain distribution across the section is linear regardless of the stress state (Figure 3) and the value of the strain at the extreme fibres is simply proportional to the curvature. In contrast, the stress distribution pattern changes with increasing levels of loading – and thus of curvature – for a section with an idealised elastic-plastic-strain hardening behaviour. Initially, in the elastic range, the stress distribution is linear (Figure 3a). As the curvature increases, the extreme fibres reach the yield stress (Figure 3b) at the yield moment Mel, with Mel = Wel fy where Wel is the elastic section modulus (Point A in Figure 2). At larger curvatures and strains, yielding spreads inwards toward the neutral axis (Figure 3c). For the elastic-plastic-strain hardening material, the section yields almost completely and is fully plastic at high values of curvature (Figure 3d); theoretically, full plasticity can only occur at infinite curvature. The theoretical moment at which full yielding occurs is termed the ‘plastic moment’ Mpl, with Mpl = Wpl fy where Wpl is the plastic section modulus (Point B in Figure 2). Actually, strain hardening is initiated at relatively high curvatures and the stress can then exceed the yield stress (Figure 3e) and the moment can exceed the plastic moment. Generally, there is no significant yield plateau as strain hardening occurs immediately after yielding. Strength reserve due to strain hardening is fully disregarded in Figure 2; therefore, the diagram is limited to the plastic collapse load Fp. |  |
| - Behaviour of a restrained simply supported beam |  |
| (a) Elastic (b) First yield (c) Elastic plastic (e) Strain hardening (d) Fully plastic Strain distribution Stress distribution at several stages of loading - Strain and stress distributions in a Class 1 or 2 hollow section subjected to bending |  |
| Instead of referring to a load-displacement diagram, it is more usual to consider the moment-curvature relation of the most loaded section (see Figure 4). |  |
| For the ideal case, the moment-curvature curve includes a linear range and a transition from the elastic resistance moment Mel to the plastic resistance moment Mpl. Once the cross-section is fully yielded, increases in curvature can occur without a corresponding moment increase. The moment does not only reach Mpl but also the beam maintains Mpl as the curvature increases. The increasing curvature under a given continuously applied moment Mpl is termed a ‘plastic hinge’ and characterises the ductility of the steel beam. Due to strain hardening, the moment may rise above the plastic moment (see the thin solid line in Figure 4); this increase in bending moment is generally ignored (see the dotted line in Figure 4). Because of residual stresses in the RHS cold-formed section, yielding occurs before the elastic moment is reached. |  |
| Usually, no elastic deformation is assumed to occur before the plastic moment is reached the behaviour in terms of M- representation is then said ‘rigid-plastic’ (see the thick solid line in Figure 4). Should the elastic strains be accounted for, then the moment-curvature relation is said ‘elastic-perfectly plastic’ (see the dotted line in Figure 4). |  |
| Curvature  at mid-span Moment at mid-span Mpl Mel Elastic-plastic idealisation (-------) Actual moment-curvature response Rigid-plastic idealisation ( ) - Moment-curvature relationship for a cold-formed hollow section in bending |  |
| At any stage during the bending process, the section may fail by local instability, i.e. plate buckling in the compression elements. |  |
| In practice, at the ultimate limit state, the maximum design value of the applied bending moment MEd must remain smaller than or equal to the design resistance moment of the cross-section Mc,Rd. |  |
|  | Part 1-1 6.2.5(1) |
| The design resistance bending moment Mc,Rd depends on the section properties, material strength and section classification. Provided that the shear force in the cross-section is small enough so that its effect on the resistance moment may be neglected (see 2.1.3), the design moment resistance of the gross cross-section is obtained as follows: |  |
| - For Class 1 and 2 cross-sections: |  |
|  | Part 1-1 6.2.5(2) |
| - For Class 3 cross-sections: |  |
|  | Part 1-1 6.2.5(2) |
| - For Class 4 cross-sections: |  |
|  | Part 1-1 6.2.5(2) |
| with, relatively to the bending axis in consideration: Wpl is the plastic section modulus of the gross cross-section; Wel is the elastic section modulus of the gross cross-section; Weff is the elastic section modulus of the so-called effective cross-section; fy is the steel material yield strength. |  |
| The recommended value of the partial safety factor is M0 = 1.0; it is up to the National Annexes to fix it. |  |
| The effective cross-section is the one computed for bending. It accounts for local plate buckling in the compression elements, i.e. the compression flange and the compression zone of the webs. How to determine the effective width/depth of the compression elements is given in details in EN 1993-1-5. | Part 1-5 §4.3 |
| In any generality, Wel,min and Weff,min correspond to the fibre where the maximum elastic stress occurs. For hollow sections, which are always at least doubly symmetrical, the index ‘min’ is meaningless for Class 3 sections. |  |
| Fastener holes in the tension flange may be ignored provided that for the tension flange: |  |
|  | Part 1-1 6.2.5(4) |
| where Af is the cross-sectional area of the tension flange. |  |
| This relation is similar to the one introduced earlier (see the lecture on Tension Members) when ductility is requested; herein it applies to the sole tension flange and only in the regions where plastic hinges are liable to occur. As written earlier, it aims at requiring that the gross cross-section yields before fracture occurs in the net cross-section – i.e. the collapse mode remains ductile. |  |
| Fastener holes in the tension zone of webs need not be allowed for provided that the limit given by Eq. is satisfied for the complete tension zone comprising the tension zone of the web and the tension flange. | Part 1-1 6.2.5(5) |
| Except for oversize and slotted holes, fastener holes in the compression zone of the cross-section need not be allowed for provided they are filled with fasteners. | Part 1-1 6.2.5(6) |
| For continuous (statically indeterminate) beams made of at least a Class 2 hollow section, the attainment of the design resistance moment at the point of maximum bending moment obtained from an elastic analysis does normally not lead to collapse (see Figure 5). From that stage, the cross-section at that point behaves as a hinge and, provided it has a sufficient rotation capacity – a Class 1 section is then requested –, the moment distribution within the structure will change from the original elastic distribution as the successive hinges form. Redistribution of moments then enables the structure to withstand loads beyond that which produces the first hinge until sufficient hinges have formed to turn the structure into a mechanism. This design approach is termed ‘plastic design’. |  |
| - Load-deflection curve for a statically indeterminate two span beam |  |
| - Section resistance to shear |  |
| Generally, bending governs the design of most steel beams. However, shear resistance can be significant for short beams with heavy point loads. Figure 6 shows the elastic shear stress distribution in a rectangular solid section and in a RHS hollow section. In the latter case, almost all the shear force is carried by the webs; because of the small variation in shear stress through the web depth, it is sufficiently accurate for design to assume an average shear stress over the web depth. |  |
| - Elastic distribution of shear stresses in a hollow section |  |
| Resistance to shear is satisfied if the design shear force VEd at each cross-section is not larger than the design shear resistance Vc,Rd: | Part 1-1 6.2.6(1) |
| Steel in shear yields at a stress approximately equal to . Hence, for plastic design, provided shear buckling cannot occur, the design value of the shear force VEd at each cross-section is compared with the design plastic shear resistance Vpl,Rd of the shear area Av: |  |
|  | Part 1-1 6.2.6(2) |
| Shear areas Av for steel hollow sections are given in Table 2. |  |
| - Shear areas Av for typical hollow sections | Part 1-1 6.2.6(3) |
| Rectangular hollow sections with uniform thickness; Load parallel to depth; A h / (b + h) (*). Load parallel to width; A b / (b + h) (*). Circular hollow sections and tubes with uniform thickness; 0,6 A. (*) A is the total cross-sectional area. |  |
| Shear buckling in RHS sections is of concern when the depth-to-thickness ratio of the walls used as webs is such that: | Part 1-1 6.2.6(6) |
| where (fy in N/mm2) while the recommended value of the factor  in accordance with EN 1993-1-5 is: |  |
| -  = 1,2 for steel grades S235, S275, S355 and S460; -  = 1,0 for steel grades higher than S460. |  |
| It is up to the National Annex to specify the value(s) of . Conservatively,  may be conservatively taken equal to 1,0. |  |
| When Eq. is not applicable because shear buckling is governing, the buckling verification should be conducted in accordance with EN 1993-1-5; this remark is mostly useless as rectangular hollow sections commonly used in practice have proportions such that they are normally not prone to shear buckling. | Part 1-5 Chapter 5 |
| Alternatively, the shear verification may refer to the elastic shear resistance; then the following criterion applies: |  |
|  | Part 1-1 6.2.6(4) |
| Such a verification is conservative as it excludes partial plastic shear stress redistribution. That leads to refer to the elastic peak shear stress at the most critical point of the cross-section (at mid-depth of the RHS or CHS). |  |
| Fastener holes need not be allowed for in the shear verification except in verifying the design shear resistance at connection zones. | Part 1-1 6.2.6(7) |
| Where the shear force is applied in combination with a torque moment, the plastic shear resistance accounting for torsional effects should be reduced from Vpl,Rd to Vpl,T,Rd and the applied shear force must satisfy: |  |
| in which Vpl,T,Rd is derived as follows: |  |
| and t,Ed is the design shear stress (constant over the wall thickness) due to the torque moment. |  |

| - Section resistance to combined bending and shear |  |
| --- | --- |
| Bending is generally accompanied with shear. | Part 1-1 6.2.8(1) |
| Due allowance should be made for its effect on the moment resistance which is designated as MV,Rd. |  |
| When plastic design, resistance to combined bending and shear is satisfied if the design resistance moment MEd at each cross-section is not larger than the reduced design resistance moment MV,Rd. |  |
|  | Part 1-1 |
| When the design shear force is less than 50% of the design plastic shear resistance, its effects on the resistance moment may be disregarded, except where shear buckling reduces the section resistance. It is reminded that the latter situation is normally not met by hollow sections. | Part 1-1 6.2.8(2) |
| Otherwise, the reduced design resistance moment of the cross-section shall be determined on the basis of a reduced yield strength for the shear area: | Part 1-1 6.2.8(3) |
| This reduced strength depends as follows on the ratio of the design shear load to plastic shear capacity: |  |
| When torsion is present, the factor  is still given by means of Eq. under the reservation that Vpl,T,Rd is therein substituted for Vpl,Rd; it should be taken as 0 for VEd ≤ 0.5 Vpl,T,Rd. | Part 1-1 6.2.8(4) |
| - Section resistance to combined bending, shear and axial force |  |
| When coincident shear and axial force, allowance should be made on the effect of both on the resistance moment. | Part 1-1 6.2.10(1) |
| No reduction of the resistances for bending and axial force is required when the design value of the shear force VEd does not exceed 50% of the design plastic shear resistance Vpl,Rd except where shear buckling reduces the section resistance, see EN 1993-1-5. The latter reservation is not likely to apply hollow sections used in the tubular construction. | Part 1-1 6.2.10(2) |
| When VEd exceeds 50% of the design plastic shear resistance Vpl,Rd, the design resistance of the cross-section to combinations of moment and axial force should be calculated with a reduced yield strength as indicated in 2.1.3. | Part 1-1 6.2.10(3) |
| - Unrestrained beams |  |
| As said above, the design resistance moment of an unrestrained beam is affected by lateral-torsional buckling, therefore the need to give additional information on the latter phenomenon. |  |
| - Section resistance |  |
| The section resistance of unrestrained beams is obtained quite similarly to the one of restrained beams; therefore, sections 2.1.1 to 2.1.4apply. |  |
| - Lateral-torsional buckling |  |
| Figure 7 shows a perfectly elastic, initially straight beam loaded by equal and opposite end moments about the major principal axis of the cross-section. The beam is supposed unrestrained along its whole length, except at its both ends where the section is prevented from twisting and lateral deflection though free to rotate both in the bending plane and in the perpendicular plane. First, the beam deflects downwards, but at some stage, buckling occurs over the length of the member, in which the section moves laterally (out of the plane of bending) and twists. The buckled shape as well as the initial non deformed configuration are shown in Figure 7. |  |
| - Lateral torsional buckling of a simple beam under uniform moment |  |
| Assuming the material is indefinitely elastic, the moment necessary to cause buckling may be determined based on an energy analogy; lateral-torsional buckling occurs when the strain energy associated with twisting and out-of-plane deformations becomes smaller than the released potential energy of the applied external loads. It is termed ‘elastic critical moment’ Mcr,0. For a beam subjected to equal but opposite end moments, it is found to be: |  |
| with: |  |
| It torsion constant; |  |
| Iw warping constant; |  |
| Iz second moment of area about the minor axis; |  |
| L unrestrained length of beam. |  |
| The presence of the flexural stiffness EIz and torsional stiffness GIt and EI in the equation is a direct consequence of the lateral and torsional components of the buckling deformations. The relative importance of these items depend on the type of cross-section. |  |
| For hollow sections, the warping constant I may be neglected when compared to the torsional stiffness GIt which is usually very large for hollow sections, with the result of a normally high elastic critical moment. Then, the elastic critical moment writes in a more simple way: |  |
| As the aspect ratio h/b of the cross-section increases, the elastic critical moment decreases and lateral-torsional buckling may occur for RHS sections with large h / b ratios. |  |
| - Buckling resistance of unrestrained beams |  |
| Real beams are not perfectly straight nor free from residual stresses nor made of an infinitely elastic material. Figure 8 shows the effects of residual stresses, geometric imperfections and strain hardening on the lateral-torsional buckling strength of a Class 1 or 2 hollow section. |  |
| For large length (slender beams), the true behaviour approaches the elastic critical moment; the effects of imperfections are hidden by the proneness of the beam to experience pure elastic buckling. For short lengths (stocky beams), the detrimental effects of imperfections is more than counterbalanced by the beneficial effects of strain-hardening, but the beam capacity is taken conservatively as the plastic resistance of the section. In the range of intermediate slenderness, the interaction between yielding and buckling is the more pronounced with the result of a significant effect of both geometric and structural imperfections. A theoretical treatment of the complex problem is hopeless so that a combination of theory and test results is required to produce a reliable (safe) design approach. |  |
| Elastic critical moment Mcr,0 Section capacity Mb True behaviour Length L Moment - Effects of imperfections and strain hardening |  |
| Figure 9 compares a typical set of lateral torsional buckling test data with the theoretical elastic critical moments given by Eq. (16) and the section bending capacity. A normalised form of plot has been used which permits results from different test series (which have different cross-sections and material strengths) to be compared directly via a relative slenderness, . A design expression linking the plastic capacity of stocky beams with the elastic behaviour of slender beams is required. In EC 3, this is achieved by using a reduction factor for lateral torsional buckling LT. |  |
| - Comparison of test data with theoretical elastic critical moments |  |
| An unrestrained member or a restrained member, the restraints of which are not sufficiently effective, should be verified against lateral-torsional buckling as follows: |  |
|  | Part 1-1 6.3.2.1(1) |
| where MEd is the design value of the applied bending moment and Mb,Rd is the design buckling resistance moment of the member. |  |
| Some types of sections, such as square (SHS) or circular hollow sections (CHS), circular fabricated tubes and square welded sections are not susceptible to lateral-torsional buckling. | Part 1-1 6.3.2.1(2) |
| Most rectangular hollow sections (RHS) belong also to this category. For some proportions of RHS lateral-torsional buckling is however likely to occur; therefore some consideration is given in the following to that type of member instability. |  |
| The design buckling resistance moment Mb,Rd of a laterally unrestrained beam is thus taken as: |  |
|  | Part 1-1 6.3.2.1(3) |
| with: |  |
| - Wy = Wpl,y for Class 1 and 2 cross-sections; - Wy = Wel,y for Class 3 cross-sections; - Wy = Weff,y for Class 4 cross-sections. |  |
| It appears as a plastic cross-sectional resistance to bending multiplied by the reduction factor LT for lateral-torsional buckling; the partial safety factor is M1 because instability is concerned. As far as beams in buildings are concerned, the recommended value of M1 is 1,0; it is up to the National Annexes to specify it. |  |
| Though specific curves should govern lateral-torsional buckling, it has been observed that some of the column buckling curves are appropriate to represent realistically the phenomenon. Therefore, LT is expressed by: | Part 1-1 6.3.2.2(1) |
| where: |  |
| in which LT is an imperfection factor, the recommended values of which are given in Table 3 below; the less favourable curves for hollow sections (being part of “other sections”) compared to rolled sections are due to more severe residual stresses, see Table 4. Unexpectedly, EN 1993-1-1 does not distinguish between cold-forming and hot-finishing, what the CIDECT guide does in some extent. |  |
| - Recommended values for imperfection factors for lateral-torsional buckling | Part 1-1 6.3.2.2(2) |
| Buckling curve; a; b; c; d. LT; 0,21; 0,34; 0,49; 0,76. | Part 1-1 Table 6.3 |
| - Recommended lateral-torsional buckling curves for cross-sections | Part 1-1 6.3.2.2(3) |
| Cross-section; Limits; Buckling curve. Rolled sections; h / b ≤ 2; a. h / b > 2; b. Welded sections; h / b ≤ 2; c. h / b > 2; d. Other sections; /; d. | Part 1-1 Table 6.4 |
| The dimensionless slenderness is defined as: |  |
| - The elastic critical moment Mcr is computed based on gross cross-sectional properties and includes the effects of loading conditions, the real moment distribution over the length L, the end restraints and the possible elastic intermediate lateral restraints. | Part 1-1 6.3.2.2(2) |
| - On how to determine Mcr, see literature on elastic stability or make use of appropriate software. |  |
| - When the reduced beam slenderness is less than some limit value, lateral-torsional effects may be ignored and only cross-sectional checks apply. In accordance with EN 1993-1-1, the recommended value of this limit is 0.2. However it is agreed to increase it up to 0.4 as far as beams in buildings are concerned and provided the effects of end restraints are not accounted for in the determination of the elastic critical bending moment. It is up to the National Annexes to specify these limit values. |  |
| - As a rough guidance, the limiting L/(h-t) ratios for RHS, below which no lateral-torsional buckling check is necessary, are drawn from [8] and reported in Table 5. These values are established based with the assumption of a simply supported unrestrained beam of span L subjected to uniform moment distribution; therefore they are mostly conservative. |  |
| - Limiting L / (h – t) ratios preventing lateral-torsional buckling |  |
| b t h h > b; - L / (h – t) ≤. - S235; - S275; - S355; - S460. - 0.5; - 73,7; - 63,0; - 48,8; - 37,7. - 0.6; - 93,1; - 79,5; - 61,6; - 47,5. - 0.7; - 112,5; - 96,2; - 74,5; - 57,5. - 0.8; - 132,0; - 112,8; - 87,4; - 67,4. - 0.9; - 151,3; - 129,3; - 100,2; - 77,3. - 1.0; - 170,6; - 145,8; - 112,9; - 87,2. |  |
| - Serviceability limit states |  |
| In addition to the resistance verifications outlined above, the performance of beams has also to be checked at the serviceability limit states. Deflections and vibrations of beams must be limited to avoid adverse affects to the appearance or effective use of the structure, discomfort to the occupants, or damage to the finishes and contents of a building. Acceptable limits for deflections should be agreed between the client, designer and competent authorities. Guidance is given in Table 6 in accordance with EN 1990). |  |
| - Recommended limiting values for vertical deflections |  |
| Conditions; Limits. max; 1. Roofs generally; L / 200; L / 250. Roofs frequently carrying personnel other than for maintenance; L / 250; L / 300. Floors generally; L / 250; L / 300. Floors and roofs supporting plaster or other brittle finish or non-flexible partitions; L / 250; L / 350. Floors supporting columns (unless the deflection has been included in the global analysis for the ultimate limit state); L / 400; L / 500. Where max can impair the appearance of the building; L / 250. |  |
| For structures open to the public, it is important to ensure that oscillations and vibrations are not so great that they cause discomfort to users. Verification of the suitability of a design may be achieved by means of a dynamic analysis but in many cases limiting the deflection is sufficient. For example, floors in dwellings and offices should have a lowest natural frequency not less than 3 cycles/second. For floors in gymnasia or dance halls, the lowest natural frequency should not be less than 5 cycles/second. |  |
| Flat roofs (slope less than 5°) are especially vulnerable to pounding if the roof deflects such that pools of water may form. It is therefore necessary to check roof deflections carefully including allowances for construction inaccuracies, settlements of foundations, deflection of roof materials, etc. |  |

- References

- EN 1993-1-1: Eurocode 3: Design of steel structures – Part 1.1: General rules and rules for buildings. CEN, Brussels, May 2005.

- EN 1993-1-5: Eurocode 3: Design of steel structures – Part 1.5: Plated structural elements. CEN, Brussels, October 2006.

- EN 1990: Eurocode - Basis of structural design. CEN, Brussels, April 2002.

- CIDECT: Lecture 10 – Compression members.

- CIDECT: Lecture 7 – Local buckling and section classification.

- Timoshenko S.P. and Gere J.M., “Theory of Elastic Stability”, Second Edition, McGraw Hill, 1962.

- Trahair N.S. and Bradford M.A., “The Behaviour and Design of Steel Structures”, E&F Spoon, 1994.

- CIDECT, Construction with Hollow Steel Sections, “Structural stability of hollow sections”, Verlag TÜV Rheinland., 1992.

- Zhao X-L., Wilkinson T. and Hancock G.: “Cold-Formed Tubular Members and Connections”, Elsevier, 2005.